3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2207081

e-Meeting, 15th August – 29th August 2022

Source:	vivo

Title:          Discussion on sidelink positioning

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

At the RAN#94-e meeting, the SID on expanded and improved NR positioning RP-213588 [1] was approved. The objective regarding the sidelink positioning is as follows:

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

In this contribution, we will discuss the following aspects of sidelink positioning from RAN2’s perspective:

- Scenarios

- Architecture and UE roles

- Protocol stack

- General signaling procedure

- Latency and efficiency requirements

- Discussion

- 2.1 Scenarios

Three network coverage scenarios were considered in [2] when at least two UEs were involved in SL positioning for V2X and public safety use cases.

Figure 5.1-1: In-coverage scenario Figure 5.1-2: Partial coverage scenario Figure 5.1-3: Out-of-coverage scenario

Taking the case of two UEs as an example, an in-coverage scenario refers to the case where both UEs are inside the network. Partial coverage means that one UE remains inside the network coverage but the other UE is outside the network coverage. Out-of-coverage scenario refers to the case where both UEs are outside the network coverage. A UE may transit between in-coverage, partial coverage and out-of-coverage scenarios.

At the RAN1#109e meeting, the coverage scenarios for evaluation were concluded as follows, which indicated that all the coverage scenarios will be supported.

Agreement For the purpose of evaluations, in-coverage and out-of-coverage scenarios are prioritized during the SI. - Note: This prioritization is not intended to down-scope support of SL positioning for partial coverage scenarios.

In Rel-18, SA2 is discussing architecture enhancement to support Ranging based services and sidelink positioning. In the latest TR [3], SA2 assumes that all three coverage scenarios shall be supported:

## 4.2 Architecture requirements The enhancement to 5GC architecture shall be able to support Ranging-based services and sidelink positioning in in-coverage, partial coverage and out of network coverage scenarios as defined in TS 22.261 [5] and TR 38.845 [6]: - When the UE operates a use case having the corresponding positioning requirements, the requirements should be fulfilled when the UE is inside the network coverage as well as when it is outside the network coverage.

To align with RAN1 and SA2, we think all the coverage scenarios shall be supported in Rel-18.

Proposal 1: Confirm that for sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported.

Besides, RAN1 also discussed the operation scenarios of SL positioning and agreed that the following two operation scenarios shall be considered:

- Scenario 1: PC5-only-based positioning

- Scenario 2: Combination of Uu- and PC5-based positioning

To our understanding, there can be multiple combinations of coverage scenario and operation scenario:

Table 1: Supported operation scenarios for different coverage scenarios

| Coverage scenario | PC5-only-based positioning | Combination of Uu- and PC5-based positioning |
| --- | --- | --- |
| In-coverage | Yes | Yes |
| Partial coverage | Yes | Yes (If target UE in-coverage) No (If target UE out-of-coverage) |
| Out-of-coverage | Yes | No |

Note that the PC5-only-based positioning can be utilized for all three coverage scenarios. From a protocol impact perspective, RAN2 should focus on the operation scenario rather than the coverage scenario, and shall study the architecture and signaling procedures to enable these two operation scenarios.

Proposal 2: Study the architecture and signaling procedures to enable the following two operation scenarios:

- Operation Scenario 1: PC5-only-based positioning.

- Operation Scenario 2: Combination of Uu- and PC5-based positioning.

- 2.2 Architecture and UE roles

- 2.2.1 Overall Architecture

The legacy Uu-based positioning architecture is as below [2]. In this architecture, the LMF acts as a location server. The positioning measurement is based on Uu reference signals between UE and base station, i.e., DL-PRS and/or UL-SRS.

Figure 1: UE Positioning Overall Architecture applicable to NG-RAN

Different from Uu-based positioning, the sidelink positioning measurement is based on PC5 reference signals between UEs, i.e., SL-PRS. From our view, to enable the above two operation scenarios, there are two possible types of sidelink positioning architectures:

- Type 1: LMF as location server for sidelink positioning;

- Type 2: Location Server UE as location server without LMF involvement;

The corresponding architectures are shown in Figure 2 and Figure 3:

Figure 2: Type 1 architecture for sidelink positioning

Figure 3: Type 2 architecture for sidelink positioning

Note that the location server UE is a logical notion and may have the capability of Anchor UE. Meanwhile, the target UE can also act as a location server UE. For a specific sidelink positioning, the number of extra Anchor UEs may be zero, one, or multiple, which depends on the positioning type (e.g., relative positioning or absolute positioning) and QoS requirement.

The above two types of architecture are quite different. Type 1 architecture seems to be one type of evolution of traditional positioning architecture. However, it requires that the UEs need to communicate with the LMF, and is applicable for the combination of Uu- and PC5-based positioning. Type 2 architecture is one new architecture for sidelink positioning. It is applicable for PC5-only-based positioning. Moreover, Type 2 architecture can achieve a shorter positioning time delay compared with Type 1 architecture since Type 1 sidelink positioning procedures need to go across multiple network entities (base station/AMF/LMF). Although the architecture is in SA2 scope, they have not concluded on the architecture yet and will have a parallel discussion in August. RAN2 may start the signaling procedure discussion based on the above two types of architectures.

Proposal 3: Take the two types of architectures as the baseline for signaling procedure discussion of sidelink positioning.

- Type 1: LMF as location server

- Type 2: UE as location server

- 2.2.2 UE roles

In the SA2 TR [3], there are the following definitions of terms:

3.1 Terms For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]. Ranging: refers to the determination of the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface. Reference UE: A UE who determines a reference plane and reference direction in the Ranging based service and Sidelink positioning. NOTE 1: Reference UE is the same as Observer UE used in TR 22.855 [2]. Target UE: A UE whose distance, direction and/or position is measured comparing to the reference plane, reference direction and/or location of a Reference UE in the Ranging based service and Sidelink positioning. NOTE 2: Any UE participating in the Ranging/Sidelink Positioning can be both a Target UE and a Reference UE and can switch roles in the same Ranging/Sidelink Positioning session. Editor's note: Concerning Reference UE and Target UE, whether new name will be defined to differentiate with the term Reference UE and Target UE used in eLCS is FFS. Assistant UE: A UE who provides assistance for Ranging/Sidelink Positioning when the direct ranging/Sidelink positioning between a Reference UE and a Target UE cannot be supported. Located UE: A UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning. Editor's note: Change term Network assisted UE and Second UE to Located UE throughout the document. Location Server UE: A UE offering location server functionality in lieu of LMF, for Sidelink Positioning and Ranging over Sidelink. It interacts with a Target UE, Reference UEs, Assistant UE and Located UEs as necessary in order to calculate the location of the Target UE. Sidelink Positioning: Positioning UE using PC5. Positioning: A functionality, which detects a geographical location and optionally, velocity (of e.g. a mobile terminal). Relative position: An estimate of the UE position relative to other network elements or relative to other UEs. Editor's note: Definition on the terminology of Ranging and Sidelink positioning will be aligned with RAN WGs, and will be revisited when there's any conclusion in RAN WGs.

At the RAN1#109 meeting, RAN1 had the following agreement regarding terms:

Agreement - For absolute positioning evaluation, anchor UEs’ locations are known - In the evaluation of SL only positioning - Anchor UEs are used to locate target UEs - In the evaluation of Joint Uu/SL positioning - Both BS and anchor UEs are used to locate target UEs Agreement For the purpose of RAN1 discussion during this study item, at least the following terminology is used: - Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface). - Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information. - Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE. - Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes. - SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity. - Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion. Agreement For the purpose of RAN1 discussion during this study item, at least the following terminology is used: - Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. - FFS: clarification of the knowledge of the location of the anchor UE

RAN2 needs to align with SA2/RAN on some terms for sidelink positioning from the view of RAN2 when e.g., discussing the architecture and the positioning procedures. To our understanding, the following terms regarding UE roles can be introduced in RAN2 firstly (the others are FFS):

-	Target UE: UE to be positioned

-	Location Server UE: A UE offering location server functionality in lieu of LMF, for Sidelink Positioning and Ranging over Sidelink.

-	Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. For absolute positioning, the Anchor UEs’ locations are known.

Proposal 4: Align with SA2/RAN1 on the terms for sidelink positioning, and introduce the following terms of UE role as the baseline for further discussion:

- Target UE: UE to be positioned

- Location Server UE: A UE offering location server functionality in lieu of LMF, for sidelink Positioning and Ranging over PC5.

- Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. For absolute positioning, the Anchor UEs’ locations are known.

- 2.3	Protocol stack

There may be two different sidelink positioning architectures as discussed in the above section. Different architectures may correspond to different protocol stacks. For Type 1, the protocol for sidelink positioning is between the UEs and the LMF. To our understanding, the existing LPP protocol can be enhanced to support the sidelink positioning. Possible enhancement approaches include:

-	Alt 1: introduce new LPP messages to support sidelink positioning;

-	Alt 2: reusing the existing LPP messages with critical extension;

-	Alt 3: reusing the existing LPP messages with non-critical extension, i.e., introducing new IEs in the existing LPP messages.

Proposal 5: Reuse and extend the LPP for sidelink positioning procedures between UE and LMF.

For Type 2, the protocol for sidelink positioning is between the UEs. SA2 had some discussion about the protocol stack over PC5. In SA2 TR [3], it seems that the majority thought that a new protocol needs to be introduced and the protocol stack should be determined by RAN Working Groups.

TR 23.700-86: # 4 Architecture assumptions and requirements <ignore unrelated part> - SR5: The reference point between the Sidelink (SL) Positioning and Ranging function in UEs. It is carried over the PC5 reference point. Editor's note: Whether SR5 will be over PC5-RRC, PC5-D, PC5-U and/or PC5-S will be aligned with RAN WGs. ## 6.4 Solution #4: Ranging devices discovery and ranging procedure <ignore unrelated part> Editor's note: Whether a new Ranging/Sidelink positioning protocol is defined or an enhancement to LPP to support Ranging/Sidelink positioning is defined will be determined by RAN Working Groups.

To decouple sidelink positioning function from other functions, we think that a new protocol should be introduced over the PC5 interface, and the term in SA2 can be reused, i.e., Ranging/Sidelink Positioning Protocol (RSPP)

Proposal 6: Introduce a new protocol for sidelink positioning procedures between UEs, i.e., Ranging/Sidelink Positioning Protocol (RSPP).

Regarding the position of the new protocol in the protocol stack, two alternatives were captured in SA2 TR [3]:

-	Alt 1: (Solution #4) CP-based solution, with new protocol over V2X/ProSe layer, e.g., PC5-D, PC5-S;

-	Alt 2: (Solution #14) UP-based solution, with new protocol over PC5-U.

Figure 4: new protocol over V2X/ProSe Layer

Figure 5: new protocol over PC5-U

From the view of RAN2, in addition to Alt 1, the following CP-based alternatives should be considered:

-	Alt 3: CP-based solution, with new protocol over PC5-RRC.

-	Alt 4: CP-based solution, with new protocol over PDCP.

Figure 6: new protocol over PC5 RRC

Figure 7: new protocol over PDCP

For the UP-based solution, one issue is that the transmission priority of RSPP messages can not be guaranteed since the priority of SL DRB is lower than the one of SL SRB. Especially, when the UE has plenty of data to transfer and/or the sidelink resources are scarce, the time delay may be quite significant. However, the time delay is very important for positioning service since it is one of few positioning QoS parameters. Also, RSPP messages are likely defined by RAN2. It is a little strange that RSPP messages defined by RAN2 are transmitted as user data. By comparison, the transmission priority of RSPP messages can be guaranteed for the CP-based solutions.

Among the CP-based solutions, the lower layer the RSPP is located, the less of the signaling overhead and processing overhead.

Proposal 7: Study the protocol stack for the new protocol RSPP, the alternatives include:

- Alt 1: CP-based solution, with new protocol over V2X/ProSe layer, e.g., PC5-D, PC5-S;

- Alt 2: UP-based solution, with new protocol over PC5-U.

- Alt 3: CP-based solution, with new protocol over PC5-RRC.

- Alt 4: CP-based solution, with new protocol over PDCP.

- 2.4	General signaling procedures

- 2.4.1 Type 1 sidelink positioning signaling procedures

The following Figure 8 illustrates a typical signaling procedure for Type 1 sidelink positioning.

Figure 8: Type 1 sidelink positioning signaling procedures

1.	The LMF receives a Location Service Request from the AMF.

2.	The LMF obtains the sidelink positioning capability of the target UE from the target UE, e.g., via the LPP Request Capabilities message.

3.	The LMF determines to perform the sidelink positioning, e.g., when the UE is not able to perform Uu-based positioning (not support, or not able to measure on enough TRP to perform Uu-based positioning).

4.	The LMF obtains the information of Anchor UE(s), e.g., the target UE may initiate the discovery procedure over the PC5 interface to find the Anchor UE(s) and sends the information of Anchor UE(s) to the LMF.

5.	The LMF may perform the exchange of LPP messages for sidelink positioning between the LMF and Anchor UE(s), e.g., Assistance Data Transfer and Location Information Transfer.

6.	The LMF performs the exchange of LPP messages for sidelink positioning between the LMF and the target UE, e.g., Assistance Data Transfer and Location Information Transfer.

7.	The LMF obtains the location information of the target UE based on the information from Step 5 or 6.

8.	The LMF sends Location Service Response to the AMF.

Proposal 8: For Type 1 sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Potential standard impact:

-	New or enhanced existing signaling procedure to exchange capability for sidelink positioning.

-	New or enhanced existing discovery procedure to obtain the information of Anchor UEs. (in SA2 scope, if reuse the ProSe discovery procedure)

-	New or enhanced existing signaling procedure to exchange the assistant information and measurement result/position information for sidelink positioning.

Proposal 9: For Type 1 sidelink positioning, the potential standard impact includes enhancing the LPP messages about Capability transfer, Assistance Data transfer, and Location Information Transfer.

- 2.4.2 Type 2 sidelink positioning signaling procedures

The following Figure 9 illustrates a typical signaling procedure for Type 2 sidelink positioning.

Figure 9: Type 2 sidelink positioning signaling procedures

1.	The Location Server UE receives a Sidelink Location Request message, or the Location Server UE itself determines the need for some location service.

2.	The Location Server UE may initiate the discovery and/or sidelink unicast link establishment with target UE and Anchor UEs.

3.	The Location Server UE initiates the sidelink positioning capability exchange procedure with the target UE and Anchor UEs.

4.	The Location Server UE performs the assistance data transfer procedure with target UE and Anchor UEs.

5.	The target UE transmits the SL PRS and the Anchor UEs measure the SL PRS, or the Anchor UEs transmit the SL PRS and the target UE measures the SL PRS.

6.	The Location Server UE performs the location information transfer procedure with the target UE or Anchor UEs.

7.	The LMF obtains the location information of the target UE based on the information from Step 6.

8.	The LMF provides the location estimate via Sidelink Location Response.

Proposal 10: For Type 2 sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Potential standard impact:

-	New protocol for sidelink positioning may need to be introduced, including new signaling procedures for sidelink positioning capability exchange, assistance data transfer and location information transfer.

-	New signaling procedure for Sidelink Location Request/Response. (in SA2 scope, if the request/response are LCS messages)

-	New or enhanced existing discovery procedure. (in SA2 scope, if reuse the ProSe discovery procedure)

Proposal 11: For Type 2 sidelink positioning, the potential standard impact includes new RSPP messages for the procedures between UEs, including Capability transfer, Assistance Data transfer, and Location Information Transfer.

Note that there are some potential standard impacts on SA2 side, RAN2 shall LS to SA2 if needed.

Proposal 12: When RAN2 concludes on the standard impact, LS to SA2 if needed, e.g., on protocol stack and discovery enhancement.

- 2.5	Latency and efficiency requirements

To reduce the latency and improve efficiency, some features were introduced for Uu-based positioning in Rel-17, e.g.:

-	Preconfigured assistance data: Multiple DL-PRS configurations can be provided to the UE before or during an ongoing LPP positioning session, to be then utilized for potential positioning measurements at a future time.

-	Preconfigured measurement gap: A set of measurement gap patterns can be pre-configured to UE and the UE can request activation and deactivation of the pre-configured MG via MAC CE.

-	On-Demand PRS: The LMF may control and decide whether PRS is transmitted or not and change the characteristics of an ongoing PRS transmission. The on-demand PRS transmission procedure can be initiated either by the UE or LMF.

For the sidelink positioning at specific scenarios, e.g., V2X, the latency and efficiency requirement will be critical. Therefore, we need to investigate how to reduce signaling overhead and latency for sidelink positioning, the potential aspects include：

-	Aspect 1: reducing the interaction and latency of configuration signaling,

-	Aspect 2: reducing unnecessary PC5 and/or Uu connection establishment,

-	Aspect 3: reducing the probability of positioning failure due to inappropriate configuration.

Proposal 13: Study essential solutions to support the latency and efficiency requirements for sidelink positioning, the potential optimization aspects include:

- reducing the interaction and latency of configuration signaling,

- reducing unnecessary PC5 and/or Uu connection establishment,

- reducing the probability of positioning failure due to inappropriate configuration.

- Conclusion

Proposal 1: Confirm that for sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported.

Proposal 2: Study the architecture and signaling procedures to enable the following two operation scenarios:

- Operation Scenario 1: PC5-only-based positioning.

- Operation Scenario 2: Combination of Uu- and PC5-based positioning.

Proposal 3: Take the two types of architectures as the baseline for signaling procedure discussion of sidelink positioning.

- Type 1: LMF as location server

- Type 2: UE as location server

Proposal 4: Align with SA2/RAN1 on the terms for sidelink positioning, and introduce the following terms of UE role as the baseline for further discussion:

- Target UE: UE to be positioned

- Location Server UE: A UE offering location server functionality in lieu of LMF, for sidelink Positioning and Ranging over PC5.

- Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. For absolute positioning, the Anchor UEs’ locations are known.

Proposal 5: Reuse and extend the LPP for sidelink positioning procedures between UE and LMF.

Proposal 6: Introduce a new protocol for sidelink positioning procedures between UEs, i.e., Ranging/Sidelink Positioning Protocol (RSPP).

Proposal 7: Study the protocol stack for the new protocol RSPP, the alternatives include:

- Alt 1: CP-based solution, with new protocol over V2X/ProSe layer, e.g., PC5-D, PC5-S;

- Alt 2: UP-based solution, with new protocol over PC5-U.

- Alt 3: CP-based solution, with new protocol over PC5-RRC.

- Alt 4: CP-based solution, with new protocol over PDCP.

Proposal 8: For Type 1 sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Proposal 9: For Type 1 sidelink positioning, the potential standard impact includes enhancing the LPP messages about Capability transfer, Assistance Data transfer, and Location Information Transfer.

Proposal 10: For Type 2 sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Proposal 11: For Type 2 sidelink positioning, the potential standard impact includes new RSPP messages for the procedures between UEs, including Capability transfer, Assistance Data transfer, and Location Information Transfer.

Proposal 12: When RAN2 concludes on the standard impact, LS to SA2 if needed, e.g., on protocol stack and discovery enhancement.

Proposal 13: Study essential solutions to support the latency and efficiency requirements for sidelink positioning, the potential optimization aspects include:

- reducing the interaction and latency of configuration signaling,

- reducing unnecessary PC5 and/or Uu connection establishment,

- reducing the probability of positioning failure due to inappropriate configuration.

- Reference

- RP-213588		Revised SID on Study on expanded and improved NR positioning

- 3GPP TR		TR 38.845 Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases

- 3GPP TR		TR 23.700-86 Study on Architecture Enhancement to support Ranging based services and sidelink positioning