3GPP TSG RAN WG2 #119e                            		R2-2207107

August 17th– 26th, 2022

Source:	CATT

Title:	Discussion on RAT dependent integrity

Agenda item:	8.2.3

Document for:	Discussion and Decision

Introduction

Positioning integrity is a measure of the trust in the accuracy of the position-related data and the ability to provide timely warnings based on assistance data provided by the network. Integrity enables applications to make the correct decisions based on the reported position, e.g., when monitoring a robotic arm to decide whether its arm movement is within allowed limits to ensure safety distances to humans and other objects. In Rel-17, 3GPP has developed the methodologies, procedures, and signaling support for GNSS integrity. For Rel-18, 3GPP is going to extend it to RAT-dependent positioning techniques, as defined in the following objective in the SI of “Study on expanded and improved NR positioning”[1]:

- Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In this contribution, we would like to discuss the solutions for the RAT-dependent positioning integrity from RAN2 perspective.

Discussion

Definitions of RAT-dependent Integrity

Integrity is the measure of the trust that can be placed in the correctness of the positioning information supplied by a positioning system. Integrity includes the ability of the system to provide timely warnings to users when the system should not be used. From the perspective of functionality, there is no clear difference of the integrity for different positioning method. Further, RAN1 has already discussed the definitions of RAT-dependent integrity, and made the following agreement. Therefore, it is reasonable to reuse the general concepts of the RAT-independent integrity for RAT-dependent integrity.

Agreement For the purpose of discussion of error sources, reuse the definitions for RAT-dependent integrity and update the references to GNSS in Section 8.1.1a in TS38.305 to also include RAT-dependent methods. - Note: The intention of the proposal is not to make text proposals for TS 38.305 - FFS: whether to modify and/or how to modify, for the purpose of discussion in RAN1, terms in 8.1.1a in TS 38.305 (e.g., definitions for “Error”, “Bound”, “Time-to-Alert (TTA)”, “DNU”, “Residual Risk”, “irMinimum, irMaximum”) for RAT dependent positioning methods

Proposal 1: For RAT-dependent integrity, RAN2 to agree to reuse the general definitions of RAT-independent GNSS positioning integrity.

Integrity of RAT-dependent positioning

The integrity is a kind of evaluation for the location estimates, i.e., whether the calculated location estimates can be trustful. Based on this, it is nature for the entity that performs the location estimation to calculate and provide the positioning integrity.

Proposal 2: RAN2 to agree the entity that performs the location estimation in RAT-Dependent methods shall calculate the protection level correspondingly.

In RAN1#109e [2], the following agreement was reached:

Agreement - UE-based/assisted DL positioning methods, UL and DL&UL positioning methods are considered in the study

According to the agreements, both the UE-based and UE-assisted DL/UL/DL&UL positioning method can be included in the scope of the RAT-dependent positioning integrity. Therefore, both UE-based positioning integrity (for UE-based positioning method) and UE-assisted positioning integrity (for UE-assisted positioning method) should be studied correspondingly.

Table 1: Supported UE positioning methods and the corresponding positioning integrity mode

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | UE-based positioning integrity | LMF-based integrity |
| --- | --- | --- | --- | --- | --- |
| DL-TDOA | Yes | Yes | No | Yes | Yes |
| DL-AoD | Yes | Yes | No | Yes | Yes |
| Multi-RTT | No | Yes | Yes | No | Yes |
| NR E-CID | No | Yes | Yes | No | Yes |
| UL-TDOA | No | No | Yes | No | Yes |
| UL-AoA | No | No | Yes | No | Yes |

Proposal 3: RAN2 to agree that both the integrity mode of UE-based positioning integrity and LMF-based positioning integrity should be studied in Rel-18.

Procedures of RAT-dependent positioning integrity

In RAN1#109e [2], the following agreement was reached:

Agreement - Study sources of error for timing-based positioning and angle-based positioning methods, focusing on the following aspects - Origin of the error source - e.g., At UE and/or network side - e.g., From assistance information, and/or measurements - Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Criteria to become an error source (e.g., whether it is quantifiable, how much influence an error source has on determination on integrity) - It is encouraged to provide evaluation assumptions (e.g., requirements in TS 38.101, TS 38.104, TS 38.133, evaluation assumptions in TR 38.857) if evaluation is used to determine a distribution, mean and standard deviation or range of values of an error source

Based on above agreements, the error sources for RAT-dependent positioning integrity are originated from UE and NW side. To be simplify, we use NG-RAN node originated error sources to represent the error sources originated from NG-RAN node, and UE originated error sources to represent the error sources originated from UE side in the following discussion.

And we will make some analysis on the procedure and signalling to support the RAT-dependent positioning integrity based on different origin of the error sources.

- UE-based DL positioning integrity procedure

For UE-based DL positioning integrity, the positioning integrity calculation is performed by UE, and then provides the calculated protection level to LMF. To assist the UE to make positioning integrity calculation, positioning error sources as well as the required target IR (TIR) should be provided to UE. Since the integrity calculation is made by UE for DL-positioning, the UE originated error sources are already known by UE based on implementation. Therefore, LMF need to provide the NG-RAN node originated error sources and the TIR to UE.

Proposal 4: For UE-based DL positioning integrity, LMF should provide the NG-RAN node originated error sources and the required TIR to UE.

Further, the NG-RAN node originated error sources may be common for different UE within a region, thus except the dedicated signalling method, broadcasting can be considered in R18 on providing the error sources to UE.

Proposal 5: On provision of NG-RAN node originated error sources to UE, RAN2 to agree that both dedicated LPP signaling and broadcasting can be supported.

An example of the general procedure for UE-based DL positioning integrity procedure is given as the following.

Figure 1: UE-based DL positioning integrity procedure

- UE-assisted DL positioning integrity procedure

For UE-assisted DL positioning integrity, the positioning integrity calculation is performed by LMF. To assist the LMF to make positioning integrity calculation, the UE originated error sources as well as the NG-RAN node originated error sources should be provided to LMF.

Proposal 6: For UE-Assisted DL positioning integrity, RAN2 to agree that UE provide the UE originated error sources to LMF, i.e., the measurement error sources.

Proposal 7: For UE-Assisted DL positioning integrity, RAN2 to agree that NG-RAN node provide the NG-RAN node originated error sources originated from NG-RAN to LMF, i.e., the assistance data error sources.

An example of the general procedure for UE-assisted DL positioning integrity procedure is given as the following.

Figure 2: UE-assisted DL positioning integrity procedure

- UL positioning integrity procedure

For UL positioning integrity, the positioning integrity calculation is performed by LMF. To assist the LMF to make positioning integrity calculation, the UE originated error sources as well as the NG-RAN node originated error sources should be provided to LMF. However, since there is not any UL LPP assistance signalling, thus the UE should provide the UE originated error sources to its serving gNB, then the serving gNB provide the obtained UE originated error sources to LMF.

Proposal 8: For UL positioning integrity, RAN2 to agree that UE provide the UE originated error sources to its serving NG-RAN node, i.e., the measurement error sources, and then the serving NG-RAN node provide such UE originated error sources to LMF.

Proposal 9: For UL positioning integrity, RAN2 to agree that NG-RAN node provide the NG-RAN node originated error sources originated from NG-RAN to LMF, i.e., the assistance data error sources.

An example of the general procedure for UL positioning integrity procedure is given as the following.

Figure 3: UL positioning integrity procedure

- DL&UL positioning integrity procedure

For DL&UL positioning integrity, the positioning integrity calculation is performed by LMF. To assist the LMF to make positioning integrity calculation, the UE originated error sources as well as the NG-RAN node originated error sources should be provided to LMF. Further, the UE originated error sources can be directly provided to LMF via the current LPP provide location information message directly.

Proposal 10: For DL&UL positioning integrity, RAN2 to agree that UE provide the UE originated error sources to the LMF.

Proposal 11: For DL&UL positioning integrity, RAN2 to agree that NG-RAN node provide the NG-RAN node originated error sources originated from NG-RAN to LMF.

An example of the general procedure for DL&UL positioning integrity procedure is given as the following.

Figure 4: DL&UL positioning integrity procedure

Conclusions

In this contribution, we discussed the solutions for the integrity of RAT dependent positioning techniques. Based on the discussion, we have the following proposals:

Proposal 1: For RAT-dependent integrity, RAN2 to agree to reuse the general definitions of RAT-independent GNSS positioning integrity.

Proposal 2: RAN2 to agree the entity that performs the location estimation in RAT-Dependent methods shall calculate the protection level correspondingly.

Proposal 3: RAN2 to agree that both the integrity mode of UE-based positioning integrity and LMF-based positioning integrity should be studied in Rel-18.

Proposal 4: For UE-based DL positioning integrity, LMF should provide the NG-RAN node originated error sources and the required TIR to UE.

Proposal 5: On provision of NG-RAN node originated error sources to UE, RAN2 to agree that both dedicated LPP signaling and broadcasting can be supported.

Proposal 6: For UE-Assisted DL positioning integrity, RAN2 to agree that UE provide the UE originated error sources to LMF, i.e., the measurement error sources.

Proposal 7: For UE-Assisted DL positioning integrity, RAN2 to agree that NG-RAN node provide the NG-RAN node originated error sources originated from NG-RAN to LMF, i.e., the assistance data error sources.

Proposal 8: For UL positioning integrity, RAN2 to agree that UE provide the UE originated error sources to its serving NG-RAN node, i.e., the measurement error sources, and then the serving NG-RAN node provide such UE originated error sources to LMF.

Proposal 9: For UL positioning integrity, RAN2 to agree that NG-RAN node provide the NG-RAN node originated error sources originated from NG-RAN to LMF, i.e., the assistance data error sources.

Proposal 10: For DL&UL positioning integrity, RAN2 to agree that UE provide the UE originated error sources to the LMF.

Proposal 11: For DL&UL positioning integrity, RAN2 to agree that NG-RAN node provide the NG-RAN node originated error sources originated from NG-RAN to LMF.

References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning.”

- MCC Support, “Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0”.