3GPP TSG-RAN WG2 Meeting #119-e                                                    R2-2207126

Online, 17 August–26 August 2022

Source:	Qualcomm Incorporated

Title:	Discovery and Relay (re-)selection for UE-to-UE relay

Agenda Item:	8.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

According to WID [1] and RAN2 agenda for UE-to-UE relay, this contribution discusses RAN2 part for UE-to-UE relay discovery and (re-)selection.

- Specify mechanisms to support single-hop Layer-2 and Layer-3 UE-to-UE relay (i.e., source UE -> relay UE -> destination UE) for unicast [RAN2, RAN3, RAN4].

  - Common part for Layer-2 and Layer-3 relay to be prioritized until RAN#98

    - Relay discovery and (re)selection [RAN2, RAN4]

    - Signalling support for Relay and remote UE authorization if SA2 concludes it is needed [RAN3]

# Discussion

2.1 Scenario and assumption

UE-to-UE relay was studied in Rel-17, and the study outcome is captured in the TR 38.836 [2]. Clause 5.1 of  TR 38.836 captures the assumptions for UE-to-UE relay, and they can be summarized as following.

| Coverage | - All UEs are in-coverage or out-of coeverage or partial-coverage are supported - UEs can be in coverage of the different cells |
| --- | --- |
| RRC state | - All RRC states for all UEs are supported |
| RAT type | Only NR access type is considered on both of sidelink interface and Uu interface (already confirmed in WID) |
| Cast type | Unicast for UE-to-UE communication |
| Service continuity | Service continuity during mobility is not supported |

For coverage, RRC states, RAT type, it is proposed to confirm the assumptions captured in TR 38.836 [2].

For cast type on UE-to-UE communication, only unicast is considered Rel-18; but for cast type carried in discovery message, SA2 is discussing possible cast types, then RAN2 should wait for SA2 progress and probably do some alignment work.

For service continuity, since UE-to-UE mobility (e.g. path switch) is captured in SA2 discussion, then based on SA2 request, RAN2 may need to do alignment work.

Proposal 1: RAN2 confirm that the Scenario, Assumption and Requirement in section 5.1 of TR 38.836 apply for UE-to-UE relay support, with below clarifications:

- For cast type on UE-to-UE communication, only unicast is considered

- For cast type used for discovery message transmission,  RAN2 should wait for SA2 progress and can do alignment work per SA2 request.

- For service continuity during mobility, RAN2 should wait for SA2 progress and can do alignment work per SA2 request.

2.2 UE-to-UE Relay discovery

Discovery mechanism was discussed and introduced in Rel-17, and corresponding AS aspects e.g. protocol, bearer, configuration, resource pool, resource allocation mode are standerdised. For U2U discovery message, most of aspects defined in Rel-17 can be reused, mainly including the following.

- Discovery message is carried by AS PDCP/RLC/MAC/PHY layers.

- SL-SRB4 is introduced to carry U2U discovery message.

- Configuration can be provided by SIB/dedicated message or pre-configured.

- Reuse the resource pools defined for Rel-17 discovery.

- Resource allocation mode 1 or mode 2 based on in-coverage or out-of-coverage.

- Discovery message can be transmitted in any RRC states

Proposal 2: U2U relay Discovery reuses the following mechanisms defined in Rel-17 for U2N relay.

-	Reuse the U2N relay discovery protocol stack for U2U relay discovery

- Reuse SL-SRB4 for U2U relay discovery.

- U2U relay discovery configuration can be provided by SIB/dedicated message or pre-configured.

- Reuse the resource pools defined for Rel-17 U2N relay discovery.

- Support Resource allocation mode 1 or mode 2 based on in-coverage or out-of-coverage.

- Discovery message can be transmitted in any RRC state

Since all the UEs can be out-of-coverage, the Discovery message should has no restriction by Uu coverage. That means UEs in any coverage state can transmit Discovery message and gNB will not configure the Uu threshold used by U2U Relay UE to determines whether to transmit Discovery message.

Proposal 3: gNB will not configure the Uu threshold used by U2U Relay UE to determine whether to transmit Discovery message.

SA2 is discussing U2U Relay discovery higher layer related issues, e.g. discovery procedure and mechanism, cast-type etc. For those SA2-led issues, RAN2 should wait for SA2 progress.

Proposal 4: RAN2 should wait for SA2 progress on U2U Relay discovery higher layer mechanism and cast type.

2.3 UE-to-UE Relay (re-)selection

Relay selection

U2N Relay selection was discussed and standardised in Rel-17 and relay link quality is the AS condition for Relay selection. The Remote UE performs radio measurements at PC5 interface for Relay selection, and a threshold can be pre-configured or configured by gNB to evaluate whether the relay link quality is satisfied for relay transmission.

Proposal 5: PC5 link quality (SD-RSRP) is the AS layer condition for U2U Remote UE to select a suitable U2U Relay UE.

Proposal 6: A threshold is configured by gNB or pre-configured to the U2U remote UE for U2U Relay selection evaluation. FFS whether to configure separate threshold with the one configured to U2N Relay selection.

SA2 is discussing U2U Relay selection together with Discovery procedure, from AS layer point of view, AS should perform U2U Relay selection when receiving indication from upper layer.

Proposal 7: U2U Remote UE performs relay selection when receiving indication from upper layer.

Relay reselection

Similar with defined for U2N Relay reselection, the radio link quality towards the serving U2U Relay UE can be used for U2U relay reselection evaluation trigger condition. UE can use SL-RSRP (if there is data transmission towards the serving Relay UE) or SL-RSRP/SD-RSRP in case that there is no data transmission. In addition, the other conditions defined to trigger U2N Relay reselection can be borrowed for U2U Relay reselection trigger conditions.

Proposal 8: U2U Remote UE may trigger U2U Relay reselection under the following conditions:

- PC5 link quality towards the current  U2U Relay UE is bellowed the (pre)configured threshold. FFS whether separate threshold with the one configured to U2N Relay reselection.

- U2U Remote UE is notified by the Relay UE the failure or release of PC5 link  on the second hop.

- The current PC5 link is released due to e.g. RLF, release by upper layer, etc.

# Conclusion

This contribution discusses the common part for UE-to-UE L2 based relay and L3 based relay, and provides the following proposals.

Proposal 1: RAN2 confirm that the Scenario, Assumption and Requirement in section 5.1 of TR 38.836 apply for UE-to-UE relay support, with below clarifications:

- For cast type on UE-to-UE communication, only unicast is considered

- For cast type used for discovery message transmission,  RAN2 should wait for SA2 progress and can do alignment work per SA2 request.

- For service continuity during mobility, RAN2 should wait for SA2 progress and can do alignment work per SA2 request.

Proposal 2: U2U relay Discovery reuses the following mechanisms defined in Rel-17 for U2N relay.

-	Reuse the U2N relay discovery protocol stack for U2U relay discovery

- Reuse SL-SRB4 for U2U relay discovery.

- U2U relay discovery configuration can be provided by SIB/dedicated message or pre-configured.

- Reuse the resource pools defined for Rel-17 U2N relay discovery.

- Support Resource allocation mode 1 or mode 2 based on in-coverage or out-of-coverage.

- Discovery message can be transmitted in any RRC state

Proposal 3: gNB will not configure the Uu threshold used by U2U Relay UE to determine whether to transmit Discovery message.

Proposal 4: RAN2 should wait for SA2 progress on U2U Relay discovery higher layer mechanism and cast type.

Proposal 5: PC5 link quality (SD-RSRP) is the AS layer condition for U2U Remote UE to select a suitable U2U Relay UE.

Proposal 6: A threshold is configured by gNB or pre-configured to the U2U remote UE for U2U Relay selection evaluation. FFS whether to configure separate threshold with the one configured to U2N Relay selection.

Proposal 7: U2U Remote UE performs relay selection when receiving indication from upper layer.

Proposal 8: U2U Remote UE may trigger U2U Relay reselection under the following conditions:

- PC5 link quality towards the current  U2U Relay UE is bellowed the (pre)configured threshold. FFS whether separate threshold with the one configured to U2N Relay reselection.

- U2U Remote UE is notified by the Relay UE the failure or release of PC5 link  on the second hop.

- The current PC5 link is released due to e.g. RLF, release by upper layer, etc.

# Reference

[1]    RP-221262: New WID on NR sidelink relay enhancements

[2]    3GPP TR 38.836 V17.0.0 Study on NR sidelink relay