3GPP TSG-RAN WG2 Meeting #119	 R2- 2207180

Electronic 17 – 26 Aug, 2022

Agenda Item:	8.9.4

Source:	Xiaomi

Title:	Discussion on multi-path

Document for:	Discussion

Introduction

In multi-path, remote UE can connect to the same gNB via both direct and indirect path. There are two alternatives to achieve indirect path according to [1]. The first alternative is to reuse U2N relay. This structure is called multi-path U2N relay. The other is to use the ideal connection. This structure is called UE aggregation. In this contribution, we discuss the scenario and potential benefit of multi-path relaying.

Discussion

Evaluation on multi-path U2N relay

In R17, U2N relay UE is allowed to transmit/monitor U2N relay discovery when it’s RSRP is below threshHighRelay. U2N relay UE is supposed to provide coverage extension to remote UE at cell edge. Therefore, U2N relay UE is supposed to be at cell edge. Therefore, the channel quality of relay UE may be similar, if remote UE is at cell edge, or worse than remote UE, if remote UE is at cell centre. Furthermore, since indirect path includes two links, it costs more radio resource to transmit the traffic compared with direct path. If the packet is split over the two paths, the efficiency on the indirect path may be much lower than direct path. The benefit of throughput improvement may be marginal.

Observation 1: The benefit of throughput improvement may be marginal, due to low resource efficiency on indirect path in multi-path U2N relay.

The reliability of remote UE’s direct path may be low since it’s at the cell edge. U2N relay UE is allowed to transmit/monitor U2N relay discovery when it’s RSRP is above threshHighRelay. Therefore, indirect path may provide more reliable transmission compared to direct path. If duplicated packet is transmitted via two paths, multi-path may provide benefit of reliability improvement. Furthermore, if one path is failed, remote UE may be able to recover via the left path.

Observation 2: There may be benefit of reliability improvement by packet duplication or failure recovery in multi-path U2N relay.

Proposal 1: RAN2 to specify the packet duplication and failure recovery in multi-path U2N relay.

Evaluation on UE aggregation

Although ideal connection is out of 3GPP, it’s beneficial to clarify the characteristic of ideal connection to perform evaluation and potential subsequent design. In U2N relay, remote UE and relay UE are connected by sidelink unicast, so the two UEs are supposed to be in proximity. We understand the ideal connection may be achieved by other local wireless NW or wire. Therefore, the UEs performing aggregation should also be in proximity.

Proposal 2: RAN2 to confirm the UEs aggregated by ideal connection are in proximity.

It’s important to clarify the transmission profile of the ideal connection, including data rate and transmission latency. We understand ideal connection should not be the bottleneck of the indirect path, which means ideal connection can provide comparable data rate and transmission latency as direct path/Uu interface.

Proposal 3: RAN2 to confirm the ideal connection can provide comparable data rate and latency as direct path/Uu interface.

Another important issue to be clarify is whether the UEs’ Uu capability are similar or the remote UE may be significantly less capable than relay UE, e.g. remote UE is a redcap UE. It may impact the evaluation on the performance of direct path and indirect path of the remote UE. We understand the aggregated UEs’ capability should be similar, which means the direct and indirect path of the remote UE is comparable from performance perspective.

Proposal 4: RAN2 to confirm the aggregated UEs’ Uu capability is similar, which means Uu connection of remote UE and relay UE can provide comparable data rate and latency.

With the above proposals as assumption, we think the use case and evaluation of UE aggregation is similar with multi-path relay. The solutions for U2N relay can be reused for UE aggregation by excluding a part of the solutions which is unnecessary for the UE aggregation. Therefore, we propose to prioritize the basic functionality of U2N multipath relay. The function on L1 and L3 can be reused for UE aggregation.

Proposal 5: RAN2 to prioritize the basic functionality of U2N multipath relay. The function on L1 and L3 can be reused for UE aggregation.

Scenarios

Fig 1. Multi-path structure

As depicted in fig 1, remote UE can connect to NW via both direct and indirect path in multi-path. Direct path includes the Uu connection between remote UE and NW, which is denoted by L1. Indirect path includes two links, i.e L2 and L3. L2 denotes the connection between remote UE and relay UE. L3 denotes the Uu connection between relay UE and NW. In U2N relay, L2 is achieved by sidelink unicast connection. In UE aggregation, L2 is achieved by ideal connection. Since ideal connection is out of 3GPP, UE aggregation may only have impact on the L1 and L3.

Frequency diversity could improve the reliability. Remote UE and relay UE may connect to different cell deployed by the same gNB. L1 and L3 may be inter-frequency. Furthermore, NW is allowed to provide inter-frequency sidelink transmission resource. Therefore, L2 and L1/L3 may also be inter-frequency.

Proposal 6: RAN2 to study the scenario that L1 and L3 are inter-frequency deployed.

Proposal 7: RAN2 to study the scenario that L2 and L1/L3 are inter-frequency deployed.

Conclusion

Based on the discussion in section 2, we have following proposals:

Observation 1: The benefit of throughput improvement may be marginal, due to low resource efficiency on indirect path in multi-path U2N relay.

Observation 2: There may be benefit of reliability improvement by packet duplication or failure recovery in multi-path U2N relay.

Proposal 1: RAN2 to specify the packet duplication and failure recovery in multi-path U2N relay.

Proposal 2: RAN2 to confirm the UEs aggregated by ideal connection are in proximity.

Proposal 3: RAN2 to confirm the ideal connection can provide comparable data rate and latency as direct path/Uu interface.

Proposal 4: RAN2 to confirm the aggregated UEs’ Uu capability is similar, which means Uu connection of remote UE and relay UE can provide comparable data rate and latency.

Proposal 5: RAN2 to prioritize the basic functionality of U2N multipath relay. The function on L1 and L3 can be reused for UE aggregation.

Proposal 6: RAN2 to study the scenario that L1 and L3 are inter-frequency deployed.

Proposal 7: RAN2 to study the scenario that L2 and L1/L3 are inter-frequency deployed.

Reference

[1] RP-213585 New WID on NR sidelink relay enhancements