3GPP TSG-RAN WG2 #119-e	R2-2207254

Electronical meeting, 17 August – 29 August 2022

Agenda Item:	6.20.1

Source:	Ericsson

Title:	Discussion on RAN4 LS R4-2211171

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the RAN4 LS [1] and expresses our views accordingly.

# 2	Discussion

In RAN4 discussion, it was identified to be beneficial for a UE to know whether CCA is being used in neighbour cells where it is performing measurements or RRM procedures, since CCA is not mandatory in FR2-2 in some regions. If the UE does not know the CCA configuration of a cell it is performing measurements, it may not know which requirements will apply in RRC_IDLE state and in RRC_CONNECTED state.

UE may not be aware of whether CCA is being used in a neighbour cell.

The issue is relevant, however, addition of new signalling for CCA configuration of neighbour cells would result inevitable changes to the specs. However, R17 WI is already completed. Any change would result into backward compatibility issue. Therefore, it should be avoided.

New signalling change for CCA configuration of neighbour cells shall be avoided.

Therefore, no additional work in RAN2 to introduce new signalling for CCA configuration of neighbour cells.

- No additional work in RAN2 to introduce new signalling for CCA configuration of neighbour cells.

We think it is not a critical issue if UE doesn’t know the CCA configuration for neighbour cells.

For any neighbour cell in unlicensed band, UE can just assume that CCA will be used in that neighbour cell regardless of whether CCA is being used or not used in that cell. Based on this assumption, UE will apply relaxed measurement requirements for that cell even though CCA is not used in that cell.

- For any neighbour cell in unlicensed band where measurements or RRM procedures are being performed, UE always applies relaxed requirements regardless of whether CCA is being used or not used in that cell.

Based on above agreements, RAN2 can send a LS reply to RAN4. RAN2 shall inform RAN4 that

- RAN2 confirms that UE doesn’t know CCA configuration for neighbour cells given the existing specs

- no additional work in RAN2 to introduce new signalling for CCA configuration of neighbour cells

- RAN2 sends a LS reply to RAN4 informing that

  - RAN2 confirms that UE doesn’t know CCA configuration for neighbour cells given the existing specs

  - no additional work in RAN2 to introduce new signalling for CCA configuration of neighbour cell

# Conclusion

In the previous sections we made the following observations:

Observation 1	UE may not be aware of whether CCA is being used in a neighbour cell.

Observation 2	New signalling change for CCA configuration of neighbour cells shall be avoided.

Based on the discussion in the previous sections we propose the following:

Proposal 1	No additional work in RAN2 to introduce new signalling for CCA configuration of neighbour cells.

Proposal 2	For any neighbour cell in unlicensed band where measurements or RRM procedures are being performed, UE always applies relaxed requirements regardless of whether CCA is being used or not used in that cell.

Proposal 3	RAN2 sends a LS reply to RAN4 informing that

a.	RAN2 confirms that UE doesn’t know CCA configuration for neighbour cells given the existing specs

b.	no additional work in RAN2 to introduce new signalling for CCA configuration of neighbour cell

# References

R4-2211171, LS on CCA configurations of neighbour cells, Nokia