3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207257

Elbonia, 17 – 26 August 2022

Agenda item:	5.1.3.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification to expiry of IDLE mode measurements

WID/SID:	LTE_NR_DC_CA_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Rel-15 first introduced the IDLE mode measurements for LTE CA, followed by Rel-16 introducing them for LTE/NR DC and NR CA purposes. When UE is requested to provide IDLE measurements, it is given a timer T331 that controls how long UE should measure. Once the timer expires, UE stops measuring.

In the context of RAN5 discussions on Rel-16 test cases, there have been some discussions on how to test the expiry of T331. In that discussion, it was noticed that there is an ambiguity in whether UE can report very “old” measurements even when T331 is running. In this contribution, we discuss a clarification to RAN2 specifications on this matter.

# 2	IDLE mode measurements

UE starts doing IDLE mode measurements when it receives measIdleConfig in RRCRelease, as per TS38.331, clause 5.3.8.3, and starts the timer T331 at the same time:

1> if the RRCRelease includes the measIdleConfig: 2> if T331 is running: 3> stop timer T331; 3> perform the actions as specified in 5.7.8.3; 2> if the measIdleConfig is set to setup: 3> store the received measIdleDuration in VarMeasIdleConfig; 3> start timer T331 with the value set to measIdleDuration; 3> if the measIdleConfig contains measIdleCarrierListNR: 4> store the received measIdleCarrierListNR in VarMeasIdleConfig; 3> if the measIdleConfig contains measIdleCarrierListEUTRA: 4> store the received measIdleCarrierListEUTRA in VarMeasIdleConfig; 3> if the measIdleConfig contains validityAreaList: 4> store the received validityAreaList in VarMeasIdleConfig;

The IDLE mode measurement configuration (including the T331 configuration) is then stored in UE variable VarMeasIdleConfig., UE then performs measurements for as long as the timer T331 is running, storing the acquired measurement results in UE variable VarMeasIdleReport  as shown in the below excerpt from TS38.331, clause 5.7.8.2a:

While in RRC_IDLE or RRC_INACTIVE, and T331 is running and T319a is not running, the UE shall: 1> perform the measurements in accordance with the following: 2> if the VarMeasIdleConfig includes the measIdleCarrierListEUTRA and the SIB1 contains idleModeMeasurementsEUTRA: 3> for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig: 4> if UE supports NE-DC between the serving carrier and the carrier frequency indicated by carrierFreqEUTRA within the corresponding entry: 5> perform measurements in the carrier frequency and bandwidth indicated by carrierFreqEUTRA and allowedMeasBandwidth within the corresponding entry; 5> if the reportQuantitiesEUTRA is set to rsrq: 6> consider RSRQ as the sorting quantity; 5> else: 6> consider RSRP as the sorting quantity; 5> if the measCellListEUTRA is included: 6> consider cells identified by each entry within the measCellListEUTRA to be applicable for idle/inactive mode measurement reporting; 5> else: 6> consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting; 5> for all cells applicable for idle/inactive measurement reporting, derive measurement results for the measurement quantities indicated by reportQuantitiesEUTRA; 5> store the derived measurement results as indicated by reportQuantitiesEUTRA within the measReportIdleEUTRA in VarMeasIdleReport in decreasing order of the sorting quantity, i.e. the best cell is included first, as follows: 6> if qualityThresholdEUTRA is configured: 7> include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThresholdEUTRA; 6> else: 7> include the measurement results from all cells applicable for idle/inactive measurement reporting;

Once the T331 is stopped, UE performs actions according to clause 5.7.8.3, which causes UE to release the VarMeasIdleConfig:

#### 5.7.8.3 T331 expiry or stop The UE shall: 1> if T331 expires or is stopped: 2> release the VarMeasIdleConfig. NOTE: It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

However, as the NOTE indicates, UE is allowed to continue doing the measurements according to NR SIB4/SIB11 or E-UTRA SIB5/SIB24 configuration even after this. Further, since UE only releases the VarMeasIdleConfig, any measurements UE may have stored in VarMeasIdleReport are retained.

Observation 1: UE does not release stored IDLE mode measurements in VarMeasIdleReport upon T331 expiration.

However, once the UE then send RRCSetupRequestComplete, it will only check whether it has measurements available based on VarMeasIdleReport, i.e. even if T331 has expired (potentially a long time ago),

2> if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or 2> if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport: 3> include the idleMeasAvailable;

Observation 2: UE indicates availability of IDLE mode measurements based on VarMeasIdleReport even if T331 has expired.

Then if network queries these results, UE shall indicate the stored measurements and only then discard the VarMeasIdleReport:

1> if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell: 2> set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available; 2> set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available; 2> discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

This is the same behaviour as in LTE Rel-15 but creates a problem: Since the VarMeasIdleReport is not released until the results are queried by the network, the UE will retain them even if the network provides another VarMeasIdleConfig in a subsequent RRCRelease. Thus, it is possible that UE may retain (very) old measurement results in the IDLE mode measurement results!

Observation 3: Since VarMeasIdleReport is only cleared when UEInformationResponse is sent successfully, UE may retain it across multiple RRC connections.

Obviously this was not the intention: The reason why the results were not cleared in LTE Rel-15 as the same time as T331 was stopped was that there could be only a short time between T331 expiry and connection establishment, so it was left up to UE implementation how to handle such cases: Requiring UE to clear the results immediately might have resulted in non-optimal behaviour. But since the results are now only cleared when queried, this presents a problem of the “too old” results. From RAN5 perspective, this also creates a problem in that it is difficult to test what happens after T331 expiry: Since UE stops measurements but doesn’t clear them, testing that UE no longer performs the measurements after T331 expiry is difficult since UE may report “old” measurements. Additionally, allowing UE to “continue” the IDLE mode measurements may even mask the behaviour, since it’s not clear when UE has done the measurements it reports, so network has no way of differentiating those.

Observation 4: Not clearing the VarMeasIdleReport after T331 expiry makes RAN5 test cases more cumbersome since it’s not possible to test what happens at T331 expiry.

Since this is causing problems for testing, we would propose to correct this. There are two main solution alternatives to fix the issue:

- Clear VarMeasIdleReport at T331 expiry (simple but may cause UEs to drop some measurements unnecessarily, and may cause issues with legacy UEs)

- Clear VarMeasIdleReport at RRCRelease (to avoid old measurements persisting across multiple RRC connections)

Proposal 1: RAN2 to discuss which option to adopt to resolve the problem of VarMeasIdleReport being retained until queried by network.

Finally, we would note that if the correction is agreed for at least future UEs, a UE capability could also allow RAN5 to differentiate the test cases. However, this is better discussed once it is known which direction to go with the correction. A correction without a capability could also be considered if no UEs supporting this functionality already exist in the field. Similarly, once the solution has been chosen, it may be also prudent to send LS to RAN5 to inform them of the RAN2 decision.

Proposal 2: Send LS to RAN5 informing them of the RAN2 decisions (including whether a UE capability is needed).

# 3	Conclusion

This document has made the following observations:

Observation 1: UE does not release stored IDLE mode measurements in VarMeasIdleReport upon T331 expiration.

Observation 2: UE indicates availability of IDLE mode measurements based on VarMeasIdleReport even if T331 has expired.

Observation 3: Since VarMeasIdleReport is only cleared when UEInformationResponse is sent successfully, UE may retain it across multiple RRC connections.

Observation 4: Not clearing the VarMeasIdleReport after T331 expiry makes RAN5 test cases more cumbersome.

And proposed the following:

Proposal 1: RAN2 to discuss which option to adopt to resolve the problem of VarMeasIdleReport being retained until queried by network.

Proposal 2: Send LS to RAN5 informing them of the RAN2 decisions (including whether a UE capability is needed).