3GPP TSG RAN WG2 Meeting #119-e                                                       R2-2207280

# Electronic meeting, August 17-26, 2022

Agenda item:		8.9.4

Source:			Intel Corporation

Title:		Discussion on Multi-path Relaying

Document for:	 	Discussion

# Introduction

In the agreed WID for NR Sidelink Relay Enhancements for NR Release 18, one of the objectives is to study the benefit and potential solutions for multi-path support as follows [1]. This contribution discusses overall requirements on how such multipath support may be supported and enabled.

- Study the benefit and potential solutions for multi-path support to enhance reliability and throughput (e.g., by switching among or utilizing the multiple paths simultaneously) in the following scenarios [RAN2, RAN3]: - A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2). Note 3A: Study on the benefit and potential solutions are to be completed in RAN#98 which will decide whether/how to start the normative work. Note 3B: UE-to-Network relay in scenario 1 reuses the Rel-17 solution as the baseline. Note 3C: Support of Layer-3 UE-to-Network relay in multi-path scenario is assumed to have no RAN impact and the work and solutions are subject to SA2 to progress.

# Discussion

Multi-path with relay was added as a new objective to be studied for Rel-18 to support reliability and throughput enhancement considering U2N relay as well as a non-standardized UE-UE interconnection. In scenarios where the Remote UE is in-coverage but with poor channel conditions, invoking the relay path, in addition to the direct path, was considered to be beneficial. Though we don’t have RAN1 TU/support as part of this WI which could provide simulation results for demonstrating the benefits of multi-path with relaying, however, based on past specification efforts for LTE DC and NR DC, we can conclude that having multiple links could be beneficial especially if it can be dynamically configurable.

## Architecture and Protocol Stack

For multi-path support in Rel-18, it is assumed that a Remote UE is connected to the gNB using two paths i.e. a direct Uu path, and an indirect path via i) L2 U2N relay UE or via ii) another UE with an assumably ideal connection to the remote UE. In reference to the indirect path, the WID mentions that solutions for both cases where indirect path via L2 U2N relay UE as well as the case where an ideal U2U connection exists are similar such that the solution for (i) may be reused for (ii) with some exclusions as necessary. To this end, it can be observed that a single architecture as shown in Figure 1, can be used for both these scenarios.

Figure 1: Multipath Relaying Architecture

For multipath communication, a single architecture can be used to support indirect connection both via L2 U2N based or an ideal UE-UE connection.

Next, we discuss the protocol stack for the cases of L2 U2N relay and ideal UE-UE connection.

### Multipath with U2N Relay

For L2 U2N Relay in NR Release 17, the SRAP sublayer is above the RLC sublayer, for both user and control planes, for both Uu and PC5 interfaces. The Uu SDAP, Uu PDCP and Uu RRC are terminated between L2 U2N Remote UE and gNB. On the other hand, SRAP, RLC, MAC and PHY are terminated in each hop (the PC5 hop and the Uu hop) [2]. To support multiple paths from the Remote UE to the gNB, the Remote UE needs to maintain a complete Uu stack, with the Uu RLC, Uu MAC and Uu PHY also terminating between the Remote UE and the gNB. The protocol stacks for the user plane and control plane of L2 U2N Remote UE architecture are presented in Figure 2 and Figure 3, respectively. The protocol stacks for the user plane and control plane of L2 U2N Relay architecture remain the same as in Figure 16.12.2.1-1 and Figure 16.12.2.1-2 in TS 38.300 [2].

Figure 2: User Plane Protocol Stack for Dual path transmission

Figure 3: Control Plane Protocol Stack for Dual path transmission

To support multiple paths for the Remote UE towards the gNB, the Remote UE needs to maintain a complete Uu stack, with the Uu RLC, Uu MAC and Uu PHY also terminating between the Remote UE and the gNB.

The protocol stacks for the user plane and control plane of L2 U2N Relay architecture remain the same as in Figure 16.12.2.1-1 and Figure 16.12.2.1-2 in TS 38.300.

### Multipath with ideal UE-UE

As part of multipath support, the case where Remote UE has connectivity via another UE (where the UE-UE inter-connection is assumed to be ideal) instead of PC5 sidelink, is also presented. This could be particularly beneficial to boost the UL data throughput to cater for the limited UL transmission power handicap of any given Remote UE. Here the ideal connection could be due to close proximity of the two UEs and/or for example a lossless wired connection between them. From RAN2 perspective, since it is assumed to be a non-3GPPP standardized connection between the two UEs, it is up to the Forwarding UE (FUE) to identify and map the incoming data from one or multiple Remote UEs through some mapping mechanism similar to SRAP for Rel-17 L2 U2N relaying case. However, this interaction between the UEs is not required to be specified in 3GPP RAN2 since it is a non-3GPP link. At the same time, from the FUE to the gNB, the SRAP layer may be required for the Uu link, similar to Rel-17 L2 U2N relaying case, for bearer mapping and RLC channel designation etc. The user plane protocol stack for this scenario is shown in Figure 4 below. The control plane protocol stack is FFS.

- For multipath with non-standard UE-UE link, where the Remote UE is connected to the Forwarding UE (FUE) via an ideal connection, a 3GPP SRAP layer may be required over the Uu link between the FUE and the gNB, similar to Rel-17 L2 U2N relaying case. However, the interaction between the Remote UE and the FUE is not specified in 3GPP RAN2 and no SRAP layer is defined for the ideal U2U interconnection.

Figure 4: User plane protocol stack for dual path transmission with ideal UE-to-UE connection

In the WID, however, multipath with ideal UE-UE is proposed with the underlying assumption that the solutions from multipath with relay and Release 17 are to be reused by mostly excluding aspects that are either not applicable or unnecessary for this scenario. With this understanding, we think that the main focus of the initial study phase (at least the next two meetings) should be on multipath with relay and RAN2 needs to finalize the technical details of the relay use case before further discussing the multipath with ideal UE-UE connection.

- For multipath support, at least for the first two meetings of the study phase, RAN2 will only focus on the case where the indirect connection is via the U2N Relay UE. Work (if any) for the case of ideal UE-UE connection can be pursued after the technical details of multipath with relay is finalized.

In the following sections, we will therefore focus on details of multipath transmission only considering an L2 U2N relay for the indirect path.

## Multipath Transmission using L2 U2N Relay

### Requirements for multipath with relay support

In NR Release 17, for UE to Network Relay, the Remote UE could be in-coverage or out-of-coverage. However, for the case of multipath support, both the Remote UE and Relay UE must be in coverage such that the Remote UE has a valid Uu connection to the same gNB. Also, since the scenario is for multiple paths connecting the Remote UE to its serving gNB and as mentioned in the WID “A UE is connected to the same gNB using one direct path and one indirect path”, it is therefore also required that the Relay UE selected for the indirect path is also connected to the Remote UE’s serving gNB, even if the Remote UE and Relay UE belong to different cells.

- To support multipath transmission/reception with Relay, both the Remote UE and the Relay UE shall be in coverage of the same gNB (as agreed in the WID).

Additionally, in NR Release 17, Remote UE(s) must be in RRC_CONNECTED to perform transmission/reception of relayed unicast data and also needs to establish its own PDU sessions/DRBs with the network before user plane data transmission. These conditions would also hold true for the case of multipath transmission, where the Remote UE may use either Uu link or Relay link or both links for data transmission and shall remain in RRC_CONNECTED state while doing so and would have its own PDU session/DRB with the network.

The Remote UE shall be in RRC_CONNECTED and have its own PDU session with the network to perform multipath transmission/reception.

### Multipath support for reliability and throughput enhancement

In NR Release 17, the Remote UE can only maintain a single connection (either direct or indirect) at any given time, and there is no capability to support or enable multipath transmission for a Remote UE. In contrast, support for multipath with Relay corresponds to utilizing both direct and indirect paths by the Remote UE to transmit/receive signalling and data, and in summary, it may mean support of:

- Data duplication for reliability enhancement

- Data aggregation/switching for throughput enhancement

Multipath support for reliability and throughput enhancement corresponds to following use-cases where: a) Remote UE simultaneously performs transmit/receive operation on two active paths with data duplication b) Remote UE simultaneously performs transmit/receive operation on two active paths with data aggregation/switching

- RAN2 to study all the use-cases for multipath support with relay for reliability and throughput enhancement (i.e. duplication, aggregation/switching).

### Enabling and configuring multipath support

The gNB may enable the support of multipath to a Remote UE either explicitly or implicitly by sending measurement configuration or configuring necessary link quality thresholds. The Remote UE performs measurements and provides the report to the gNB via the existing link that can be direct or indirect relay path. Thereafter, the gNB adds the new path by providing relevant bearer configuration to Remote UE and Relay UE accordingly. This procedure uses Release-17 path switching as baseline. It is possible that the Remote UE had an active connection to the gNB directly and finds another indirect path which satisfies certain criteria e.g. PC5 link threshold with hysteresis criterion, and this indirect path is configured as an additional path to enable multipath transmission [case 1]. Another use case is where the Remote UE is active on the indirect path and finds that a Uu link is available which satisfies e.g Uu link quality threshold with hysteresis condition, and consequently this direct Uu path is configured as an additional link to support multipath [case 2].

- L2 U2N Relay-based path switching procedures are used as baseline for enabling multipath. i.e. gNB makes the decision of adding the second path.

- gNB provides link quality thresholds for enabling multipath at the Remote UE either via broadcast signalling or dedicated signalling. i.e. PC5 link quality threshold to enable candidate relay UE measurement report if already using direct path and Uu link quality threshold to enable the direct path measurement report if already using indirect path.

The signalling flow for both the cases, [case 1] when adding indirect path, and [case 2] when adding direct path are shown in Figure 5 and Figure 6, respectively.

Figure 5 showcases the procedure for when indirect path is added while the Remote UE has an existing direct path. Rel-17 path switching procedure is followed wherein the Remote UE provides a list of candidate relay UEs satisfying AS criteria and the gNB chooses the Relay UE. The gNB continues to maintain the Remote UE’s Uu link and the Remote UE continues to perform Uu procedures e.g. RLM after being connected on both the paths. Figure 6 shows the addition of a direct path when the Remote UE is already active with an indirect path. This will happen as per legacy procedure and once the Remote UE performs RACH with the gNB, it is aware that the Remote UE is now active with multiple paths and the Remote UE/Relay UE maintain the PC5 link.

- When indirect path is added alongside direct path, the direct path is not affected as in the gNB maintains the Remote UE’s direct path i.e. it does not release Remote UE’s Uu link.

- When direct path is added alongside indirect path, the indirect path is not affected as in the Remote UE/Relay UE maintains the indirect path i.e. Remote UE or Relay UE’s AS layer does not release the corresponding PC5-RRC connection (and PC5 unicast link) after receiving RRC reconfiguration from gNB.

Figure 5: Signalling flow when adding indirect path

Figure 6: Signalling flow when adding direct path

### Configurable data duplication, aggregation/switching operation

Multipath support with relay is similar to the dual connectivity scenario defined in LTE and NR but differs in that there is only one gNB. The procedures of data splitting and duplication are mostly directly applicable to multipath as well. In DC, both SRB and DRB are allowed to be split and transmitted across two paths for better reliability. Similarly, for multipath, we can support both SRB and DRB to be sent across both links and it is easier in this case as the destination is the same gNB. Duplication detection can be used to discard any duplicate packets, however, care must be taken to ensure that there is no additional overhead and wastage of resources. We can rely on the gNB to configure the Remote UE as needed based on QoS requirement of the service, channel conditions, load etc.

- Multipath with Relay supports splitting of both SRB and DRB.

The Remote UE performs initial connection establishment (using SRB0) via either direct or indirect path and then establishes the second path as per the signalling flow discussed earlier. The gNB then configures the two paths on a per-bearer basis (for both signalling and data radio bearers). Split SRB may be configured by the gNB for SRB1 and SRB2 so that the UE can use either path for duplication or aggregation/switching.

- When multipath with Relay is enabled for a Remote UE, split SRB for SRB1 and SRB2 can be configured by the gNB to allow CP duplication or aggregation/switching.

Similar to DC, we can rely on gNB to configure the data radio bearers accordingly when both paths are available at the Remote UE and to set the primary path/secondary path within PDCP configuration as applicable.

- When multipath is enabled for a Remote UE, the gNB provides necessary configuration on a per-bearer basis.

Packet duplication in multipath should be configurable as it results in wastage of resources if not deemed necessary. Data duplication can be configured to be supported on both direct path and indirect path for a Remote UE with multipath enabled where the receiving PDCP entity (gNB in UL, Remote UE in DL) can perform duplicate detection and discard.

- When multipath with relay is enabled for a Remote UE, the split bearer can also be configured to support duplication i.e. the Remote UE and the gNB can transmit/receive duplicate packets over both direct path and indirect path.

For UL transmission where duplication is not configured for a given bearer, we think that the gNB can configure whether the bearer uses direct path or indirect path accordingly (semi-static manner). We can further discuss whether it can be done dynamically based on other criteria.

- For UL data transmission, when multipath with Relay is enabled for a Remote UE, for a given bearer when duplication is not enabled and both paths are available, as a baseline, it can be up to gNB configuration to select which path is used.

At the same time, which path is used in the downlink when duplication is not configured for a given bearer, is left to gNB implementation.

- For DL data transmission, when multipath with Relay is enabled for a Remote UE and both direct and indirect paths are available and duplication is not enabled, it is up to network implementation to choose which of the two paths is used.

### Flow control

During Release-17, we had some high-level discussions on whether flow control is to be supported for L2 U2N relay. Company view was split down the middle, and we agreed to deprioritize flow control for Release 17. Companies primarily felt that it was an optimization feature and for a single hop and single path architecture, flow control can be handled through proper configuration by the gNB. However, the proponents suggested that flow control at Relay UE in UL and DL is essential to alleviate congestion on its Uu link and provide necessary QoS.  Therefore in Release 18, we can study to see if there are benefits to supporting flow control at the Relay UE when there are multiple paths.

- Discuss whether to support flow control at the Relay UE in UL and/or DL when there are multiple paths.

### Resource Allocation Modes

In NR Release 17, the U2N Remote UE can only be configured to use resource allocation mode 2 for data to be relayed. In the case of multipath transmission, however, the Remote UE may use mode 1 when transmitting over Uu link only, while it may use mode 2 only when transmitting simultaneously over both Uu and Relay link, or when transmitting over Relay link only. Mechanism needs to be defined for such mode switching.

- Discuss whether a Remote UE configured with multipath uses only resource allocation mode 2 or allowed to use both mode 1 and mode 2.

# Conclusion

In this contribution we discuss multipath communication in sidelink relaying environment and make the following observations and proposals:

Observation 1.	For multipath communication, a single architecture can be used to support indirect connection both via L2 U2N based or an ideal UE-UE connection.

Observation 2.	To support multiple paths for the Remote UE towards the gNB, the Remote UE needs to maintain a complete Uu stack, with the Uu RLC, Uu MAC and Uu PHY also terminating between the Remote UE and the gNB.

Observation 3.	The protocol stacks for the user plane and control plane of L2 U2N Relay architecture remain the same as in Figure 16.12.2.1-1 and Figure 16.12.2.1-2 in TS 38.300.

Observation 4.	The Remote UE shall be in RRC_CONNECTED and have its own PDU session with the network to perform multipath transmission/reception.

Observation 5.	Multipath support for reliability and throughput enhancement corresponds to following use-cases where: a) Remote UE simultaneously performs transmit/receive operation on two active paths with data duplication b) Remote UE simultaneously performs transmit/receive operation on two active paths with data aggregation/switching

Proposal 1.	For multipath with non-standard UE-UE link, where the Remote UE is connected to the Forwarding UE (FUE) via an ideal connection, a 3GPP SRAP layer may be required over the Uu link between the FUE and the gNB, similar to Rel-17 L2 U2N relaying case. However, the interaction between the Remote UE and the FUE is not specified in 3GPP RAN2 and no SRAP layer is defined for the ideal U2U interconnection.

Proposal 2.	For multipath support, at least for the first two meetings of the study phase, RAN2 will only focus on the case where the indirect connection is via the U2N Relay UE. Work (if any) for the case of ideal UE-UE connection can be pursued after the technical details of multipath with relay is finalized.

Proposal 3.	To support multipath transmission/reception with Relay, both the Remote UE and the Relay UE shall be in coverage of the same gNB (as agreed in the WID).

Proposal 4.	RAN2 to study all the use-cases for multipath support with relay for reliability and throughput enhancement (i.e. duplication, aggregation/switching).

Proposal 5.	L2 U2N Relay-based path switching procedures are used as baseline for enabling multipath. i.e. gNB makes the decision of adding the second path.

Proposal 6.	gNB provides link quality thresholds for enabling multipath at the Remote UE either via broadcast signalling or dedicated signalling. i.e. PC5 link quality threshold to enable candidate relay UE measurement report if already using direct path and Uu link quality threshold to enable the direct path measurement report if already using indirect path.

Proposal 7.	When indirect path is added alongside direct path, the direct path is not affected as in the gNB maintains the Remote UE’s direct path i.e. it does not release Remote UE’s Uu link.

Proposal 8.	When direct path is added alongside indirect path, the indirect path is not affected as in the Remote UE/Relay UE maintains the indirect path i.e. Remote UE or Relay UE’s AS layer does not release the corresponding PC5-RRC connection (and PC5 unicast link) after receiving RRC reconfiguration from gNB.

Proposal 9.	Multipath with Relay supports splitting of both SRB and DRB.

Proposal 10.	When multipath with Relay is enabled for a Remote UE, split SRB for SRB1 and SRB2 can be configured by the gNB to allow CP duplication or aggregation/switching.

Proposal 11.	When multipath is enabled for a Remote UE, the gNB provides necessary configuration on a per-bearer basis.

Proposal 12.	When multipath with relay is enabled for a Remote UE, the split bearer can also be configured to support duplication i.e. the Remote UE and the gNB can transmit/receive duplicate packets over both direct path and indirect path.

Proposal 13.	For UL data transmission, when multipath with Relay is enabled for a Remote UE, for a given bearer when duplication is not enabled and both paths are available, as a baseline, it can be up to gNB configuration to select which path is used.

Proposal 14.	For DL data transmission, when multipath with Relay is enabled for a Remote UE and both direct and indirect paths are available and duplication is not enabled, it is up to network implementation to choose which of the two paths is used.

Proposal 15.	Discuss whether to support flow control at the Relay UE in UL and/or DL when there are multiple paths.

Proposal 16.	Discuss whether a Remote UE configured with multipath uses only resource allocation mode 2 or allowed to use both mode 1 and mode 2.

# References

[1] RP-221262, WID on NR sidelink relay enhancements, RAN Meeting #95e

[2] TS 38.300, NR and NG-RAN Overall description; Stage-2