3GPP TSG-RAN WG2 #119e	R2-2207292

Electronic meeting, 17 – 26 August, 2022

Agenda item:		8.3.2	gNB and UE supporting techniques

Title:			Finer granularity configuration for NES

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the SID [1], the objectives for RAN2 are the following:

Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

• How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

Also, a framework for load condition is mentioned:

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

In this Tdoc, we will discuss how to achieve finer granularity energy saving and identify some issues.

# 2. Discussion

In legacy, the network can already decide to switch off an entire cell and the load will be offloaded to neighbouring cells, however this may not always be: (i) doable, e.g. for coverage cells if no other cell is available, the network still has to ensure service to UEs, or (ii) justifiable, e.g. if it would mean neighbouring cells would use more power to enhance coverage than it would save for serving cell (plus overhead of HO).

In the following table, we propose a definition for the types of load and for each load type we discuss the relevance of legacy switch-off and analyse potential network energy saving behaviour:

| Load type | Relevance of cell switch-off and example of appropriate network energy saving strategy and drawbacks |
| --- | --- |
| No active UE (0% load) | The cell coulds witch off, but some UEs may be camping on it and rely on this cell for future DL/UL data. |
| Few active UEs (5-20% load) | The cell could switch off and offload its UEs, but some UEs may require high QoS only achievable in this cell, other techniques that do not require complete cell switch off may be preferrable. |
| Moderate load (20-40%) | Switching off the cell, if possible, would lead to HO overhead and may not be energy efficient for UEs and NW in general. Other methods such as smart scheduling may be more efficient. |
| Normal to high load (40-70%) | Switching off the cell should not considered, but cell is not being utilised to its maximum potential and NES should be achievable. |
| High to full load (70-100%) | Network energy saving is not appropriate here. |

In all cases described in the table, avoiding to completely switch off the cell for a long period of time and using finer granularity methods may prove beneficial.

Observation 1: Switching off a cell and offloading its UEs is rarely optimal

As proposed by multiple companies already [2,3], the possibility to have more dynamic sleeping periods would be beneficial.

Proposal 1: RAN2 should discuss different time granularity and dynamic sleep patterns

The main drawback of legacy cell switch-off is having to offload its UEs to neighbouring cells. Upon switching on again, previous UEs may discover the cell again and proceed to another handover back to the cell. This procedure, if it happens more frequently, would be very wasteful. In the case of shorter sleep pattern of e.g. a few subframes where the cell is not transmitting or receiving, unlike for legacy cell switch-off, the switch-on time is known and UEs may not need to be offloaded.

In practice, this would mean that UEs would keep their RRC state when the cell wakes up, i.e. Inactive UEs stay inactive and Connected UEs stay connected and previously acquired SI would remain valid, as if nothing happened.

Proposal 2: RAN2 to assume that UEs can keep their RRC state in the cell between short sleep periods

Furthermore, we believe that these dynamic patterns of finer granularity should also apply to other aspects of the cell’s energy consumption.

The SID includes “frequency, spatial, and power domains”, for instance to switch off BWPs, beams or reducing range. Saving energy could also be achieved by reducing some functionalities, such as further reduced signalling or paging.

Proposal 3: RAN2 should consider other types of granularities including but not limited to e.g. spatial, spectral, functionality

Once a configuration is chosen by the cell, it is important to standardise how the UEs will be made aware. Since the scale of changes should affect all UEs (including Idle or future UEs), broadcasting some or all of the configuration may necessitate to update SI.

More specifically, the minimum SI includes essential information for cell measurements (location of PSS/SSS) as well as initial access (e.g. PRACH). However, to achieve NES, some form of the aforementioned short sleep patterns may be implemented when these resources would be available in legacy systems, as pointed out by [2].

For instance, a joining UE may access the PRACH as per legacy but if the cell is in sleep mode, it will not be decoding such attempted access. This would lead the legacy UE to assume its PRACH attempt failed and wrongfully try again and increase its uplink power.

Observation 2: If some essential SI or UL resources are off during sleep periods, this can affect legacy UEs

Proposal 4: RAN2 to assume that essential functionalities are kept regardless of the NES configuration if legacy UEs are served

The SID states that “legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.”

We see three options to achieve a trade-off between serving legacy UEs and achieving NES:

- Every NES technique is backwards compatible and essential functionalities remain always-on

- Cell resources are split between NES-capable UEs and legacy UEs, e.g. the initial BWP could stay backwards compatible and other more aggressive NES techniques can be applied in other BWPs

- The cell can use legacy switch-off to offload its legacy UEs (and stay barred) but keep NES-capable UEs

The last option is the one that can achieve the most gains in terms of NES since even with low to no load, keeping the cell on consumes a lot of energy. It would correspond to the "greenfield deployment” scenario mentioned in the SID but, thanks to the legacy cell switch-off mechanism, it would not cause Non Backward Compatibility issues for legacy UEs that were connected to the cell before implementing NES techniques.

Proposal 5: RAN2 to discuss backward compatibility, we may need to render the cell inaccessible to legacy UEs

# 3. Conclusion

In this contribution, we propose:

Proposal 1: RAN2 should discuss different time granularity and dynamic sleep patterns

Proposal 2: RAN2 to assume that UEs can keep their RRC state in the cell between short sleep periods

Proposal 3: RAN2 should consider other types of granularities including but not limited to e.g. spatial, spectral, functionality

Proposal 4: RAN2 to assume that essential functionalities are kept regardless of the NES configuration if legacy UEs are served

Proposal 5: RAN2 to discuss backwards compatibility, we may need to render the cell inaccessible to legacy UEs

# References

[1] RP-220297, Revised SI: Study on network energy savings for NR

[2] RP-212384, Discussion on network energy saving for 5G Advanced, ZTE, Sanechips

[3] RP-212422, Motivation for Network Energy Saving in Rel-18, Ericsson