3GPP TSG-RAN WG2 #119e	R2-2207296

Electronic meeting, 17 – 26 August, 2022

Agenda item:		8.7.3	Network verified UE location

Title:			Assumptions on Network verified location

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In [1],

In NTN, radio cells may be larger and/or may be moving on the Earth surface. This makes difficult to determine reliably the UE location and even the country in which it is located. This may prevent NTN to support of regulatory services (see Key Issue #10: Regulatory services with super-national satellite ground station in TR 23.737).

As part of Release 18, it is proposed to identify the use cases and requirements (e.g. accuracy, privacy, reliability, latency), which mandate the network operator to cross check the UE location reported by the UE, in order to fulfil the regulatory requirements for potential use cases/services (i.e. emergency call, lawful intercept, public warning, charging/billing).

More specifically, the related TR 38.882 [2] states:

Furthermore, a malicious UE might "fake" its selected PLMN in order to attempt connecting to a different core network. Upon such an attempt the AMF will disconnect the UE and inform the RAN node via an appropriate NGAP cause value, so the RAN can take appropriate action on subsequent attempts by the same UE.

The UE may send GNSS measurements to the RAN over RRC, but this has at least the following drawbacks:

-	In principle, just as a malicious UE could fake its selected PLMN, it could also fake its GNSS measurements;

-	Sending GNSS measurements over RRC before AS security is set up raises security and privacy issues.

Because of the above, relying only on signalling GNSS measurements over RRC is not considered a viable solution to this issue.

However, the level of “maliciousness” is later refined as:

Some further observations:

a)	At least some of the information the UE supplies to the network will have to be considered as trusted, to avoid extreme conclusions (at least RRC measurements cannot be faked);

b)	Core networks connecting to the same shared RAN will always require some degree of common coordination / configuration: this is typically the case for network sharing (especially MOCN). For NTN, this may include e.g. specific timer settings/behaviour for UE connection attempts;

Furthermore, it must be noted that “the scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority”.

In this contribution, we discuss realistic assumptions on location verification depending on level of maliciousness.

# 2. Discussion

2.1 The limits of location estimation

Localisation methods are essentially a triangulation using 3 (or more to reduce estimation error) distance estimations between an object and 3 (or more) known positions. It must also be noted that the 3 known positions should not be linearly related, i.e. form a triangle/plane in space and not be aligned. With only one satellite, it may be possible to acquire multiple measurements over time but its trajectory makes for poor accuracy compared to 3 or more satellites that are far apart.

The most accurate way to obtain these distances is by measuring a delay difference between the emission and reception of a reference signal. This difference can be measured either by the UE from downlink reference signals sent by the base stations or by the base station(s) by measuring an uplink reference signal sent by the UE.

Due to the high difference of transmit power, a downlink approach is usually preferrable. However, in the case of a malicious UE, it could tamper with the measurements and inform the network of erroneous reference signal measurements. Knowing the satellites’ location and downlink timing alignment, it is possible to falsify the acquired measurements to give a false location to the network.

Observation 1: A malicious UE could falsify downlink reference signal reports to provide a fake location to the network

One solution would be to acquire uplink reference signal, but with only one satellite available, the UE could still fake the timing of sending the reference signal to trick the network. Only if the same signal is received simultaneously by multiple satellites can the network truly be sure of a UE’s position.

Observation 2: A malicious UE could falsify the start of its uplink reference signal and without multiple satellites receiving it simultaneously, the network cannot detect it

Assuming that “at least RRC measurements cannot be faked” should mean that such observed scenarios should not happen. But RAN2 should at least point out that a solution where all RRC measurements are trusted may not be fully compliant with some objectives.

Proposal 1: RAN2 discuss and clarify the expected level of maliciousness of UEs and their capacity to tamper UE location

2.2 Verification vs active location

As shown in the first section, an active network-based UE localisation requires a lot of time and energy resources and is likely to be inaccurate, compared to a regular UE reported GNSS location. The cost and delay will be non-negligible if the verification is required for every location report, especially in LEO, considering the limited service time of one satellite. Thus, it does not have vocation to be used automatically, but only in particular circumstances.

Observation 3: Location verification will be expensive and should not be used often

Proposal 2: Network verified UE location should not be used automatically for every UE reported location

Another topic is whether the goal of this verification is to have a network-based solution to (i) acquire accurate positioning information, (ii) to verify when the location does not seem correct, or (iii) when extra accuracy may be needed, e.g. close to a border. Although in the third case an active location verification will likely not be more precise than the UE reported location.

Achieving targeted accuracy may not be feasible, however it could at least rule out an “obviously” fake location report, using both past UE information and active low accuracy location estimation.

With a targeted accuracy of 5-10km, the network may anyway not be sure about a UE’s country of location when close to a border. As long as the low accuracy position verification method is not contradicting the UE’s reported location, the network does not have any other choice but to trust the UE, otherwise every border area with a width of up to 20km cannot be covered by NTN networks.

Proposal 3: If a method used by the network is not contradictory with the location provided by the UE, then the latter should be trusted by the network

Furthermore, if RAN2 would agree to trust a UE, this would only be for a given amount of time.

Thus, if the UE can pass the verification after report of its location once, the location provided by the UE can be trusted for following service for a period of time.

However, if the trigger for requesting network verified UE location is still valid after some time, then the network should be able to request another verification,

Proposal 4: After passing a verification following a network verified location, the location provided by the UE should only be trusted for a period of time.

# 3. Conclusion

In this contribution, we propose:

Proposal 1: RAN2 discuss and clarify the expected level of maliciousness of UEs and their capacity to tamper UE location

Proposal 2: Network verified UE location should not be used automatically for every UE reported location

Proposal 3: If a method used by the network is not contradictory with the location provided by the UE, then the latter should be trusted by the network

Proposal 4: After passing a verification following a network verified location, the location provided by the UE should only be trusted for a period of time.

# References

[1] RP-221820 R18 SID

[2] 3GPP TR 38.882 V1.0.0 (2022-06)