3GPP TSG-RAN WG2 Meeting #119e	 Tdoc R2-22xxxxx

Online, 15th Aug – 26th Aug 2022

Agenda Item:	8.7.4

Souce:	MediaTek Inc.

Title:	Improving Cell Reselection in NR-NTN

Document for:		Discussion and decision

# 1 Introduction

Rel-18 enhancements for NR-NTN was approved at RAN Plenary #95e [1]. The corresponding work item has identified mobility aspects for idle UE and connected UEs. The work item objective (WID) has mentioned enhancements in cell-reselection to reduce the UE’s power consumption as a major objective. Previous agreements in Rel-17 NR-NTN Work Item has agreed time-based and location-based cell reselection for quasi-earth-fixed cells. In this contribution we discuss cell reselection in earth-moving cells and how satellite ephemeris can assist cell re-selection in NR-NTN.

# 2 Discussion

Figure 1: LEO-satellite movement with cells sweeping over earth's surface

In terrestrial networks, cells are fixed, but UEs might be mobile according to different trajectories.  Thus, for idle mode mobility management, UE selects (or re-selects) the best cell. On the other hand, the situation is quite different in NTN, especially for LEO satellites with earth-moving cells [2]. As shown in Figure 1, most of the LEO satellites travel at some speed relative to the earth. Thus, for LEO satellites, the cells are moving over time, albeit in a predictable manner.

Observation 1: In NTN, LEO satellites are moving over time, relative to the earth, albeit in a predictable way.

2.1 Time and Location based Cell Re-selection in Earth Moving Cells

During Rel-17 NR-NTN agreement, time-based cell re-selection was agreed for quasi-earth-fixed cells, where the time when the serving cell is going to stop serving a geographical area is provided to the UE. The same solution can be re-used for time-based cell reselection in earth-moving cells

Proposal 1: RAN2 will reuse the time-based cell reselection, agreed for quasi-earth-fixed cells in Rel-17 NR-NTN, as the baseline for time-based cell re-selection in earth-moving cells.

On the other hand, location-based cell re-selection was discussed in Rel-17 NR-NTN for a prolong time. While the proponents argued for performance improvements, concerns were raised by some UE vendors, as frequent location checking in idle mode can severely impact UE’s idle mode power savings. As a compromise, finally it was agreed as optional feature with an optional UE capability to support it. As earth-moving cells are expected to involve more mobility in comparison to quasi-earth-fixed cells, it is likely to result in more frequent cell re-selection. Hence, RAN2 needs to discuss location-based cell re-selection in earth-moving cells, considering UE’s idle-mode power consumption aspects.

Proposal 2: Location-based cell reselection in earth-moving cells should consider UE’s idle mode power saving aspects.

2.2 Using Satellite Ephemeris for Assisting Cell Re-selection

During initial access, the assistance information corresponding to the serving beam can indicate the following information of the beams around the potential trajectory due to the satellite movement:

- Physical cell IDs of the beam candidates as the next beam (first tier)

- Physical cell IDs of the beam candidates as the beam after next beam (second tier)

- SSB information of the beam candidates as the next beam (first tier)

- The starting frequency domain position

- Periodicity and symbol offset

- SSB information of the beam candidates as the beam after next beam (second tier)

- The starting frequency domain position

- Periodicity and symbol offset

- The difference of the frequency pre-compensation value between the serving beam and the beam candidates as the next beam (first tier)

- The difference of the frequency pre-compensation value between the serving beam and the beam candidates as the beam after next beam (second tier)

Figure 2: Beam trajectory and next cell information

As an example, let’s assume the UE is in beam 6 (see figure above). The assistance information indicates the information of beam 2 and beam 7, which are the candidates of next beam on 1st tier. It can also indicate the information of beam 3 and 11 which are the candidates of the beam after next beam on 2nd tier. This assistance information can be delivered through SIB.

Proposal-3: Physical cell Identity and the SSB configuration of the next candidate cell(s) for re-selection can be included in ephemeris and indicated in SIB for cell re-selection in NTN. UE can use this next cell information for faster cell re-selection in RRC idle state.

# 3 Conclusions

In this contribution we point how satellite ephemeris can assist cell re-selection in NTN.

Observation 1: In NTN, LEO satellites are moving over time, relative to the earth, albeit in a predictable way.

Proposal 1: RAN2 will reuse the time-based cell reselection, agreed for quasi-earth-fixed cells in Rel-17 NR-NTN, as the baseline for time-based cell re-selection in earth-moving cells.

Proposal 2: Location-based cell reselection in earth-moving cells should consider UE’s idle mode power saving aspects.

Proposal-3: Physical cell Identity and the SSB configuration of the next cell(s) can be included in ephemeris and indicated in SIB for cell re-selection in NTN. UE can use this next cell information for faster cell re-selection in RRC idle state.

# 4 References

- RP-220953, Thales, NR NTN (Non-Terrestrial Networks) enhancements, WID, RAN#95e, June 2022.

- TR 38.821 “Solutions for NR to support non-terrestrial networks”