# 3GPP TSG RAN WG2 Meeting #119-e      	                           R2-2207423
E-Conference, 17th – 26th August 2022

Agenda item:	8.3.2

Source:	Apple

Title:	Initial discussion on RAN2 work of Network energy saving

WID/SID:	FS_Netw_Energy_NR– Release 18

Document for:	Discussion and Decision

# 1 Introduction

SI of Network energy saving (RP-213554) was agreed in RAN#94e [1]. The related SID objectives involving RAN2 are summarized below.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

In this contribution, we share our initial views on general RAN2 work on network energy saving.

# 2 Discussion

## 2.1 Time domain technique

In RAN1#109-e [2], the following agreements were made on the potential aspects related to time domain techniques to be studied in RAN1.

Agreement

Further study techniques and enhancements for increasing time domain energy saving opportunities by the gNB, including (but not limited to) the following aspects:

- potential methods of reducing/adapting transmission/reception of common channels/signals, e.g. SSB, SIB1, other SI, paging, PRACH, and its impact to initial access procedure, cell (re)selection, handover, synchronization and measurements performed by the idle/inactive/connected UE;

  - potential methods of reducing transmission/reception of common channels/signals can include no- or reduced-transmission/reception, increased periodicity, enablement of on-demand transmission/reception of common channels/signals, or offloading of common channels/signals to other carriers or use of light or relaxed versions of common channels /signals

- potential methods of reducing/adapting transmission/reception of periodic and semi-persistent signals and channels configuration such as CSI-RS, group-common/UE-specific PDCCH, SPS PDSCH, PUCCH carrying SR, PUCCH/PUSCH carrying CSI reports, PUCCH carrying HARQ-ACK for SPS, CG-PUSCH, SRS, positioning RS (PRS), etc.

- semi-static and/or dynamic cell on/off in one or more granularity, e.g. /subframe/slot/symbol; some examples are:

  - Cell/network node activation request by the UE, for example using signal/channel from UE for gNB’s wake-up request

  - enhancements to L1/L2 based mobility to efficiently enable a network node (e.g. TRP, repeater) on/off operation within a cell (within network energy saving SI scope)

  - signaling enhancements for indication of semi-static and/or dynamic cell/subframe/slot/symbol on/off duration

- support of periodic and/or on-demand reference signal(s) from the gNB to aid discovery of a cell;

- dynamic adaptation of UE C-DRX configurations in a UE-group or cell-specific manner

- Mechanism to utilize potential energy saving states or sleep modes and the transition between states from leveraging cell on/off opportunities

  - including studies of waking up gNB due to user traffic, or user density, or gNB receiving wake up signal

  - including technique to allow discovery and measurement of cells in sleep or dormant states

- UE assistant information facilitating BS time domain adaptation

Note: For all techniques above, study of time domain techniques is applicable for single component carrier and multi-component carrier cases. Use of UE grouping and its interaction with proposed techniques can be considered.

Our view on its potential RAN2 work is illustrated in Table.1.

| Solution | Potential RAN1 impacts | Potential RAN2 impacts | Our preference on RAN2 work |
| --- | --- | --- | --- |
| Adaptation of common RS for IDLE / INACTIVE UE | - How to reduce transmission of SSB / SIB1/SI/paging/PRACH - New DL/UL signalling to notify RS adaptation - Impacts on initial access | Cell reselection, measurement in IDLE/INACTIVE state | Wait for RAN1 progress - The specific solution is not clear for now - Its impacts on initial access may be significant, which should be evaluated by RAN1 first. - If RAN1 can achieve consensus, RAN1 can trigger RAN2 study correspondingly. |
| Adaptation of DL common / dedicated RS for CONNECTED UE | - How to reduce transmission of CSI-RS/TRS/PDCCH - L1 measurement (e.g. CSI feedback, L1-RSRP) | L3 measurement (e.g. SSB/CSI-RS L3 measurement), Handover (FFS), RRC impact | RAN2 can first study adaptation of RS for L3 measurement, and wait for RAN1 progress on other aspects. |
| Adaptation of UL transmission for CONNECTED UE | - How to reduce gNB blind detection for UL periodic transmissions (e.g. CG-PUSCH, SR, SRS) | MAC impacts (e.g. CG-PUSCH) | Wait for RAN1 progress - The specific solution is not clear for now - If RAN1 can achieve consensus, RAN1 can trigger RAN2 study correspondingly. |
| Semi-static and/or dynamic cell on/off | - New gNB wake-up signalling - New discovery RS | Enhancement of L1 / L2 mobility | - Wait for RAN1 progress on whether new signalling/RS will be introduced - Wait for RAN2 progress on Rel-18 mobility enhancement (baseline of L1 / L2 mobility) |
| Cell specific DRX / UE group DRX | - New signalling for cell specific / UE group DRX | Mechanism of cell specific DRX | RAN2 can start the discussion as DRX study is generally led by RAN2 |
| Sleep mode adaptation / transition | - gNB behaviour in different sleep mode - Signalling for sleep mode transition | Not clear for now | Wait for RAN1 progress because gNB sleep modes may highly depend on the Network energy saving performance evaluation. |

Table.1 Potential RAN1 / RAN2 impacts and preference for RAN2 work on time domain solutions

As a summary, we think RAN2 can first study adaptation of RS for L3 measurements and cell specific DRX / UE group DRX. RAN2 impacts on other candidate solutions are not clear for now, and can wait for their progress in RAN1.

Proposal 1: For time domain techniques, RAN2 first study adaptation of RS for L3 measurements and cell specific DRX / UE group DRX. RAN2 impacts on other candidate solutions are not clear for now, and wait for their progress in RAN1.

## 2.2 Frequency domain technique

In RAN1#109-e [2], the following agreements were made on the potential aspects related to frequency domain techniques to be studied in RAN1.

Agreement

Further study techniques and enhancements for frequency resource usage adaptation by the gNB, including (but not limited to) the following aspects:

- For operations with single-carrier or within a single CC

- Enhancements to dynamic bandwidth adaptation

    - including adjustments to RBs and/or BWP used by (Rel-18) UEs for transmission and reception, reducing BWP switch delay, UE-group BWP switching, and joint adaptation of transmission bandwidth and power spectral density

- supporting UE group-common BWP or cell-specific BWP or dedicated BWP for network energy savings, and related BWP switching mechanism

- Enhancements for the case of frequent BWP switching such as resource configurations for SPS PDSCH and Type-2 CG PUSCH

- For operation with multi-carrier

- enablement of reducing/adapting common channels/signals for some CC in multi-carrier operations

    - including enablement of SSB-less secondary cell operation for some CC in case of inter-band CA. For SSB-less cell operation enablement, study the conditions and restrictions required for the operation and the related procedures for idle/inactive/connected UEs including SCell activation procedure with potential RAN4 involvement

    - including enablement of SIB-less operation for some CC in case of intra-band and inter-band CA.

    - Reducing/adapting gNB’s transmission/reception of other common channels/signals (than SSB) and TRS for some CC in multi-carrier operations

- enhancements on Scell activation and deactivation, enhancements on Scell dormancy and dynamic Pcell switching

    - including triggering conditions and methods for signaling activation/deactivation

    - including UE group common dynamic Pcell switching

Our view on its potential RAN2 work is illustrated in Table.2.

| Solution | Potential RAN1 impacts | Potential RAN2 impacts | Our preference on RAN2 work |
| --- | --- | --- | --- |
| BWP enhancement | - Adaptation of RB(s) in BWP - Optimization of BWP switching including reducing switch latency and UE group-common / cell specific signalling | MAC and RRM impacts (if new BWP switching signalling can be agreed) | Wait for RAN1 progress on optimization of BWP switching, similar to specification of BWP in NR Rel-15. |
| Adaptation of common RS in multi-carrier | As only SSB is mandatory in SCells of inter-band CA in current spec. gNB can turn off other RS based on implementation. Thus, only SSB-less SCell in inter-band CA needs further study. | Not clear for now | RAN1 / RAN4 need to first evaluate feasibility of SSB-less SCell in inter-band CA before RAN2 get involved in this study. |
| Enhancement of SCell activation / deactivation and dynamic PCell Switching | - New signalling for dynamic PCell switching or SCell activation / deactivation | MAC impacts (if dynamic PCell switching is agreed) | Wait for RAN1 progress |

Table.2 Potential RAN1 / RAN2 impacts and preference for RAN2 work on frequency domain solutions

As a summary, we think although there may be some RAN2 impacts on frequency domain technique, RAN2 can wait progress on BWP enhancement, dynamic PCell switch and SSB-less SCell in inter-band CA.

Proposal 2: For frequency domain techniques, RAN2 wait RAN1 progress (e.g., BWP switching enhancement, dynamic PCell switch and SSB-less SCell in inter-band CA)

## 2.3 Spatial domain technique

In RAN1#109-e [2], the following agreements were made on the potential aspects related to spatial domain techniques to be studied in RAN1.

Agreement

Further study techniques and enhancements for the adaptation of number of spatial elements of the gNB, including (but not limited to) the following aspects:

- Note: spatial elements may include antenna element(s), TxRU(s) (with sub-array/full-connection), antenna panel(s), TRxP(s) (co-located or geographically separated from each other), logical antenna port(s) (corresponding to specific signals and channels)

- impact to UE operations from dynamic adaptation of spatial elements, e.g. measurements, CSI feedback, power control, PUSCH/PDSCH repetition, SRS transmission, TCI configuration, beam management, beam failure recovery, radio link monitoring, cell (re)selection, handover, initial access, etc.,

- feedback/assistance information from the UE required for support dynamic spatial element adaptation

  - for example, CSI measurement and reports, SR, etc

- signaling methods, including reduced signaling, for enabling dynamic spatial element adaptation

  - for example, group-common L1 signaling, broadcast signaling, MAC CE, etc.

- dynamic TRxP adaptation;

  - study of triggering on/off conditions for TRxP(s)

    - note this may not have specification impact and could potentially be up to network implementation.

  - study of SSB, PL-RS, TRS, and CSI-RS re-configuration and its impact to initial access procedure, synchronization and measurements performed by the idle/inactive/connected UE

- dynamic logical port adaptation and efficient port reconfigurations

  - study details of signaling the port (e.g. NZP CSI-RS ports) (if required to be known by the UE)

  - study dynamic adaptation (including activation/deactivation) of CSI measurement or report configuration for port adaptation

- Joint adaptation of spatial-domain, frequency-domain and/or power-domain configurations to avoid coverage loss

- grouping of UEs to reduce transmission and reception footprint at the gNB; including but not limited to the following

  - grouping of users in spatial domain

Our understanding is that the main proposals are MIMO enhancement via adaptation on the number of Tx/Rx antenna elements/ports, the number of panels and TRxP. Similar to other MIMO discussion in RAN2, we think RAN2 may design some new MAC-CE and specify UE capability after RAN1 send LS to RAN2. For now, RAN2 don't need to involve in the discussion.

Proposal 3: For spatial domain techniques, RAN2 may design some new MAC-CE and specify UE capability after RAN1 trigger the discussion, similar to other MIMO discussion in RAN2.

## 2.4 Power domain technique

Agreement

Further study the necessity of RAN1 change for techniques and enhancements for adaptation of transmission power/processing and/or reception processing of signals/channels by the gNB, including (but not limited to) the following aspects:

- dynamic adjustment of transmission power

  - including which signals/channels the adaptation of transmission power should be applicable for. For example, dynamic DL power control for specific channel / reference signal, such as CSI-RS, adjustment of maximum PSD assigned to PRBs of PDSCH, etc.

  - studying potential UE feedback/assistance information for adjustment of transmission power

  - studying PA efficiency improvements to maintain transmission quality (e.g., EVM) when operating at higher efficiency, potentially with RAN4 involvement

  - studying geographical area/user density to adjust the transmission power

- adaptation of gNB transceiver algorithms and processes to improve power efficiency:

  - including techniques aided by UE, e.g., utilizing legacy or enhanced feedback mechanism;

  - for example, adaptation of digital pre-distortion (DPD), use of digital post distortion (for improving power efficiency) by the UE, adaptation to transceiver filtering operation

  - impact to UE implementation and power consumption should be considered

- tone reservation techniques (to improve PAPR and power efficiency);

  - It is noted that tone reservation techniques for UE will be studied in Rel-18 further NR coverage enhancement WI, as indicated in RP-213579

For above list of power domain techniques, we think only RAN1 impacts are expected and it will highly depend on the gNB power consumption model. No RAN2 impacts are foreseen.

Proposal 4: For power domain techniques, RAN2 impacts are not foreseen.

## 2.5 UE assistance information

In RAN1#109-e [2], the following agreements were made on UE assistance information.

Agreement

Further study techniques and enhancements on assistance information from the UE to aid the gNB to perform energy saving techniques

- Some examples of assistance information are, but not limited to:

  - preferred SSB configurations,

  - indication of semi-static UL channel transmissions,

  - indication of UE’s buffer status for UL channel transmissions,

  - UE traffic information such as service priority, delay tolerance, data rate, data volume, traffic type, time criticality, and packet size(s),

  - coverage, mobility status, location.

  - conditions for triggering the assistance information from the UE

As can be seen, RAN1 agreed that the UE can provide some assistance information to aid the gNB to perform energy saving. However, it may cause impact to UE's extra power consumption and implementation complexity. Similar to Rel-16 WI of UE power saving, some UE power saving solutions are at the cost of extra gNB power consumption or complexity. Thus, for each network energy saving solution, its impacts on UE power consumption and complexity should be carefully evaluated.

Proposal 5: For each network energy saving solution, its impacts on UE power consumption and complexity should be carefully evaluated.

# 3 Conclusion

In this contribution, we share our initial views on general RAN2 work of network energy saving. Our proposals are:

Proposal 1: For time domain techniques, RAN2 first study adaptation of RS for L3 measurements and cell specific DRX / UE group DRX. RAN2 impacts on other candidate solutions are not clear for now, and wait for their progress in RAN1.

Proposal 2: For frequency domain techniques, RAN2 wait RAN1 progress (e.g., BWP switching enhancement, dynamic PCell switch and SSB-less SCell in inter-band CA)

Proposal 3: For spatial domain techniques, RAN2 may design some new MAC-CE and specify UE capability after RAN1 trigger the discussion, similar to other MIMO discussion in RAN2.

Proposal 4: For power domain techniques, RAN2 impacts are not foreseen.

Proposal 5: For each network energy saving solution, its impacts on UE power consumption and complexity should be carefully evaluated.

# 4 References

[1] RP-213554, New SI: Study on network energy savings for NR.

[2] RAN1#109e, Chair Notes.