3GPP TSG-RAN WG2 Meeting #119-e	R2-2207429

e-Meeting, 17 Aug – 29 Aug 2022

Agenda item:	8.5.2

Source:	Apple

Title:	Considerations on XR-awareness, QoS-metrics, and XR-specific traffic handling

WID/SID:	FS_NR_XR_enh

Document for:	Discussion / Decision

# Introduction

In continuation of the 3GPP work on XR in RAN1 and SA4 in Rel-17, RAN has approved a RAN2-led study item on XR enhancements for NR in Rel-18 [1].

The SI objectives include three broad areas on 1) XR-awareness, 2) XR-specific power saving and 3) XR-specific capacity improvements, as shown in the table below.

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

This contribution provides an initial view on the first study item objective on XR-awareness in RAN, including XR traffic characteristics, XR traffic handling and the impact on QoS metrics and parameters. In particular, we highlight a list of topics that we consider important to be included in the RAN2 study.

# Discussion

## Use cases and drivers

XR (Extended Reality) is a broad term covering under one umbrella AR (Augmented Reality), MR (Mixed Reality) and VR (Virtual Reality). Along with cloud computing, XR applications typically require high throughput and relatively low latency. XR services exhibit deterministic behavior with a defined QoS, often consisting of multiple parallel streams with a stringent QoS on some or all the streams. XR traffic patterns are also characterized by quasi periodic data transmission with varying TB size. Refer to [15] for an overview of scenarios and use cases, as well as [7] for an initial view from a RAN perspective.

Figure 1: Multiple data flows in XR traffic

Part of the end-to-end packet latency is contributed by the radio network and the transport network. The proportion of latency contributed by other parts in the end-to-end service can be reduced with edge computing. The 5G system has been designed to deliver eMBB, URLLC and mMTC services. For NR, support of eMBB and URLLC was introduced in Rel-15 and enhancements have been made in Rel-16 and Rel-17.

If the XR use cases are not distinct enough from existing eMBB/URLLC use cases, XR-specific enhancements are hardly justified, as there are many already for generic eMBB/URLLC use cases. From discussion in [5][6], SA4 studies [8][9], and cited studies from [3][4], we do see some unique characteristics in the traffic profile of XR applications. Then XR-specific traffic characteristics [5], especially multiple periodic data streams with variable packet sizes as depicted in Figure 1, should be the driving motif for NR enhancements for XR.

Proposal 1: NR enhancements for XR should be motivated by XR traffic characteristics, especially the multiple data flow aspects.

## QoS characteristics of XR traffic

Based on the model that is chosen, RAN and SA will need to identify whether/how QoS information can be conveyed between different layers in an entity (UE, RAN, CN) and throughout the network itself.

XR applications can operate on Application Data Units (ADUs) and larger data bursts (represented by a bitstream) where a data burst may consist of a set of ADUs. A single ADU in the set can get packetized into multiple lower layer data packets sized smaller than the ADU. The application layer data is typically delivered in bursts that occur relatively periodical. According to SA4, an ADU is the smallest independently usable data unit [10][11].

For XR, SA2 coined the term PDU Set [2] which we consider equivalent to an ADU in the context of XR.

A data burst can consist of several PDU Sets where each PDU Set is transmitted in a number of smaller data packets, such as RTP packets mapped to IP packets / PDCP SDUs. For example, in the case of an audio codec, a PDU Set is defined as an audio frame. In the case of H.264 (AVC) or H.265 (HEVC), a PDU Set is defined as a NAL unit (NALU). Figure 2 provides an illustration.

Figure 2: PDU sets and data bursts

XR services can operate on a “PDU Set” which comprises multiple IP packets, and each QoS PDU can be mapped to a QoS flow. A QoS flow is identified using QoS Flow ID and each PDU Set within the QoS Flow may be identified using a PDU Set SN. Each QoS flow can be used to deliver one or more PDU Set. The UE and the network may further identify information relating to each PDU Set, such as PDU Set importance and PDU Set dependency. The CN may also provide the information relating to PDU Sets to RAN.

New QoS parameters for PDU Set based QoS handling may be defined in 5GS, such as:

- PDU Set Delay Budget (PSDB)

- PDU Set Error Rate (PSER)

- Whether to drop a PDU Set in case PSDB is exceeded

- Whether all PDUs are needed for the usage of PDU Set by application layer

- PDU Set Priority

Based on the new QoS parameters associating to XR, a certain subset (or a minimum number) of packets in a PDU Set must be successfully delivered within PSDB time interval, otherwise the PDU Set may not be useful for application layer. In some cases, all packets in a PDU Set are needed, so application layer cannot tolerate any packet loss.

Moreover, a PDU Set may be characterized by a notion of “importance” indicating how important the PDU Set (e.g., a video frame) is for the application. For example, an important PDU Set could be an I-Frame, while a less important PDU Set could be a P-Frame. Figure 3 shows an example.

Figure 3: Illustration of important and non-important PDU sets in one XR traffic stream

According to SA2 [2], the RAN can obtain the information of importance for each PDU Set (e.g., from the CN or from the application). Apparently, important PDU Sets should be treated with better reliability, while non-important PDU Sets can be processed in a more relaxed manner. New QoS information may be useful for the RAN to improve user experience and/or resource efficiency.

QoS handling in the current 5G system

- The QoS flow is the finest granularity for QoS forwarding treatment in the 5G System

- The 5G QoS characteristics are associated with 5QI values and each packet in a QoS flow is treated according to the same QoS requirements

- The QoS forwarding treatment is applied per packet/PDU

Expected QoS handling for XR

- For XR/media services, a group of packets are used to carry payloads of a PDU Set (e.g., a frame, video slice/tile)

- A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g., a frame or slice for XRM Services)

- Usually, all PDUs in a PDU Set are needed by APP layer to use the corresponding unit of information. In some cases, the APP layer can still recover parts of the information unit, when some PDUs are missing.

- The QoS forwarding treatment may apply per PDU Set

Key issues and additional aspects related to XR traffic characteristics are listed in [2] and [16].

Proposal 2: NR enhancements for XR should take new QoS parameters agreed by SA2 into account, to improve both user experience and resource efficiency.

## Study Areas for XR awareness at RAN and Application

### QoS parameters anticipated for XR

In the light of XR awareness and new requirements on QoS, RAN2 should study the set of parameters proposed by SA and identify whether any additional parameters are required. To this end, RAN2 has currently received an LS from SA2, which mostly focusses on power saving aspects. We refer to our power saving paper [17] in this regard.

According to TR 38.838, the XR periodic traffic may be associated with random jitter, where the actual packet arrival time may be offset from the nominal timing (based on traffic periodicity). On the other hand, the packet size also varies over time.

Figure 4: Single stream DL traffic model [7]

Thus, the packet arrival rate is “quasi-periodic” due to the random jitter. The jitter is characterized by a random variable following a probability distribution.

With respect to QoS characteristics defined in the 5G system today (e.g., averaging window, MDBV, periodicity), over the radio interface, traffic for a given 5QI can utilize multiple periodicity values. The averaging window is not necessarily a multiple of the actual periodicity used and MDBV is not necessarily a multiple of the burst size within that periodicity. Therefore, indicating number of bits (burst size) per periodicity can be useful to allow for more fine-granular scheduling. Also, XR services may not always be delay-critical. If non-delay critical 5QIs are associated with a periodicity, then there is no default burst size since MDBV is only available for delay critical GBR.

From this consideration, we think that the parameters associated with a PDU Set may include and are not limited to the following, RAN2 should study these further.

Main parameters

- Periodicity, which may be driven by application layer (frame rate) or lower layer (CG/SPS)

- Burst parameters (burst size, arrival time or timing such as start/stop)

- Jitter and probability distribution

- PDU Set importance and frame/slice type

- PDU Set QoS metrics (delay budget, latency, reliability, priority, survival time, error rate, max packet size, direction, transmission rate/distribution, etc.)

Additional parameters

- PDU Set identifier and granularity mapping information (e.g., does the PDU Set link at the granularity level of a DRB/LCH, QoS flow, CG/SPS, etc.)

- Inter/intra PDU Set dependency (i.e., within a traffic flow or within a PDU Set)

- Traffic correlation or synchronization preference (i.e., between different XR traffic flows)

- Direction (UL or DL)

- Interface (not all parameters may be needed on every layer)

Proposal 3: RAN2 to study and identify a set of QoS parameters to facilitate reliable and efficient transmission of PDU Sets at lower layers and in the 5G system, including awareness of packet /PDU set characteristic across different L2 sublayers and how to optimize the system for the bursty traffic characteristics exposed by XR.

### XR awareness at RAN

An XR application server may assist the RAN with information about XR-specific characteristics of a traffic flow where the provision of such information happens through the CN. Accuracy of this information is gated by factors such as the refresh interval that can be applied for each UE and traffic flow, the available signalling capacity, and the delay within the CN. Furthermore, certain characteristics, such as arrival of critical information, jitter, buffer residency time, may suddenly change at either side of the connection. In some cases, the signalling of feedback/adjustment information via the application layer from the UE to the server and then back from the server to the RAN may simply take too long.

RAN2 should study the circumstances where assistance information is beneficial from the CN and from the UE, for example, based on service requirements and QoS, power consumption, capacity, including the type of information to be provided. Possible restrictions as to when a UE can provide extra information or even become the sole source of XR traffic information to the RAN may be applied by the network. An evaluation of trigger events, timeliness of information, update frequency as well as the actual signalling methods and procedures itself appears beneficial.

Assistance information from the CN to the RAN currently exists for 5G advanced interactive services and URLLC where TSCAI is optional. When TSCAI is unavailable the network may in some cases rely on pre-configured QoS flows using QoS mapping tables, which is less dynamic. Accurate and up-to-date information on expected burst timings helps reduce collisions when the network can utilize this information to schedule grants accordingly. It seems reasonable to assume that a gNB may not aways receive sufficient information from within the network. Compared to URLLC/IIoT with TSN where the gate schedule is relatively constant, packet arrival patterns are more dynamic in XR. TSN can control packet arrival cycles from a single network element such as the CNC but for XR, a pattern update can originate from multiple entities in the system. This means the provision of XR assistance information becomes more complex.

Observation 1: Given the large variety of XR service types and deployment options, we assume CN assistance information to the RAN may not be suitable or available for all types of scenarios.

In cases where the CN is unable to provide timely and up-to-date XR traffic characteristics to the RAN or when inherent characteristics require a fast adjustment of the current allocation, the UE can provide assistance information to the gNB. A UE may also indicate a preference for a parameter update when required, for example, in use-cases heavily relying on CG/SPS. This helps ensure radio resources are allocated appropriately. On a high level, the network may configure the UE to enable/disable scheduling assistance information, e.g., at QoS flow establishment or through an update at a later time.

Proposal 4: Study scenarios where assistance information from the UE is useful, for example, based on service requirements and QoS, power consumption and capacity, including suitable methods and type of information to be provided.

### XR awareness at Application

Similar to an occasion at the UE or XR server side where the structure of application layer transactions and payload may be known partly in advance to the actual payload transmission and where the application may suggest certain adjustments to lower layers, the same principle may apply in the other direction as well. Applications can receive RAN information through APIs, either from E2E or from lower layers. At the same time an application may provide input on expected traffic pattern and characteristics in return. The information can be used to predict burst size, periodicity etc., or request for an appropriate parameter adjustment ahead of time. An adjustment could apply for a limited period or apply persistently as an update of the connection setting.

Proposal 5: Study and identify RAN information suitable to provide guidance to XR applications, including related procedures.

Awareness of such information at application level may also be used to optimize the quality of a connection from end to end. An application layer may provide feedback to optimize the connection setting through other methods, such as the ones currently studied by SA2 [2] for codec/rate adaptation.

Adjustments may be based on application layer parameters, for example, to optimize an XR session between two UEs or between a UE and an XR application server. Scenarios with XR traffic between UEs served by different network operators or even between different countries require special consideration as well. SA4 specs define access network bitrate recommendations (ANBR), which is supported by the Recommended Bit Rate MAC CE in RAN2 specs. We have noted that ANBR is limited to bitrate recommendations for DRBs and does not allow to report periodicity, burst arrival, and packet distribution based on XR traffic flows. Accurate knowledge of expected burst timing benefits the scheduling at each peer end (both at application level as well as at lower layers) and helps reduce jitter and latency. In this context, a concept to signal scheduling adjustments may be needed not only for MTSI but also for XR architectures based on DASH or WebRTC. RAN2 would need to work with SA4 and SA2 on suitable concept updates and including the set of potential scheduling parameters required.

Proposal 6: Study enhancements for application/higher layer influence and E2E parameter adjustments in interaction with SA4 and SA2.

### Synchronization of traffic flows

XR traffic consists of multiple parallel traffic flows with a characteristic to synchronize data content between them. For example, video and audio traffic may need to be synchronized while at the same time, other data such as pose, sensor, or scene data can be related to a picture frame. All this data is very dynamic and latency/delay sensitive.

Also, more advanced services such as immersive XR and media services involve streams of data of different modalities. Some of these streams can be transmitted at the same time and in a coordinated manner (in terms of QoS coordination, traffic synchronization, power saving, etc) to a single or even multiple application servers for further processing. In return, multiple outcomes may be generated as feedback. Depending on the XR service, a single UE or multiple UEs may be involved. For scenarios with multiple UEs, multiple outcomes may have to be played out at different UEs in a synchronized manner. For scenarios where only a single UE is involved, contents of multiple traffic flows must be synchronized in the sense that there is a dependency between them. A synchronisation requirement may be expressed as a threshold that describes the maximum tolerable temporal separation of the onset of two stimuli, one of which is presented to one sense and the other to another sense, such that the accompanying sensory objects are perceived as being synchronous [12].

A simple example is the synchronization between audio and video (lip sync) in traditional multimedia. For XR and immersive services, multiple other modalities may be present, such as information perceived by sensors about the environment (e.g., brightness, temperature, humidity), haptic data (e.g., pressure, texture, vibration, temperature) or kinaesthetic senses (e.g., gravity, pull forces, sense of position awareness) [14].

Therefore, requirements for and possible options to support traffic synchronization between flows are desirable to be studied. As a side note, traffic flows scheduled in transport blocks transmitted too far apart of each other can create a high demand on battery consumption for the UE [17].

How can RAN2 facilitate the synchronization between related traffic flows? While a synchronized play-out can be achieved by timestamping and buffering of packets at the application layer, lower layers are not aware of the relation between different traffic flows. Buffering large amount of data in jitter buffers and/or buffers for synchronized play-out increases memory and cost. If lower layers can be aware of synchronization requirements or help ensure packets can be transmitted closely together when needed, this will reduce the demand to buffer packets and thus help alleviate memory requirements while allowing for reasonably fast processing at the same time.

In addition, policy coordination between multiple QoS flows may be required, part of which is related to multi-modal flows being investigated by SA1 and SA2. RAN2 should consider respective solutions that are being developed by SA2, e.g., for multi-modality traffic flow coordination, application layer media synchronization, and key issue #1 and #2 in [2].

Proposal 7: Study how RAN2 protocols can be aware of synchronization requirements to help ensure application layer packets of related traffic flows are delivered in close temporal proximity.

### MAC scheduling related aspects

As shown in the previous section, the 5G QoS model as it exists today is packet based. SA2 is evaluating enhancements to the QoS handling based on PDU Sets. Changes to the QoS model have the potential to impact the packet processing and the data flow in RAN and the UE. Moreover, RAN2 will need to develop an understanding of the mechanisms to support a mapping of XR traffic flows to DRBs and in higher layers. Critical questions are, for example, whether QoS flows always remain the finest granularity. Close collaboration with SA2 is needed here.

Assuming multiple QoS flows can be mapped to the same DRB where some of them belong to PDU Sets of different importance and/or different QoS requirements, RAN2 needs to evaluate the impact to lower layers and whether this means any changes to the scheduling at MAC. For XR traffic, an application not only has multiple streams of concurrent data where QoS requirements may differ, some of the streams also have similar QoS requirements as well. As a result, in this option, multiple QoS flows may be mapped to the same DRB more frequently.

However, each of the XR traffic flows has its own traffic arrival pattern, QoS characteristics and presumably its own set of associated configured grants. Based on the current 5G NR design the MAC layer cannot identify different QoS flows within a LCH and there is no clear mapping between CGs and QoS flows / QFIs. This means MAC cannot select the most appropriate data when constructing a MAC PDU as it cannot identify SDUs based on QoS flows or QFIs within the LCH.

Observation 2: In the current 5G NR design the MAC layer cannot identify different QoS flows within a LCH and there is no clear mapping between CGs and QoS flows / QFIs.

In other words, PDU Sets of different importance may require different QoS treatments (e.g., error rate, priority, PDB, etc.). When a traffic flow contains for example I-frames and P-frames from an XR application, two approaches exist for differentiation of PDU Sets. Option a) maps PDU Sets of different importance to different QoS flows, and option b) keeps them in the same QoS flow. This has implications on the scheduling at the MAC layer. In both cases there is a problem how to identify the traffic relating to a PDU Set with a given importance level.

In option b), even though a QoS flow represents the smallest granularity level in the 5G system, a method to identify PDU Sets within a QoS flow will be needed. This can be challenging if a differentiated treatment of PDUs associated with a PDU Set is intended to be facilitated by the MAC or PHY layers. Likewise in option a) the 5G QoS model allows more than one QoS flow on the same DRB where QoS flows might need to be identified at lower layers.

Hence, we think that RAN2 should study this problem and establish appropriate ways for mapping and identification of PDU Sets. In connection with this question, it also needs to be established to what extent lower layers have a notion of PDU Sets and/or the QFI, and what this would means to intra-UE prioritization and scheduling of radio resources. A goal might be to not to introduce complex changes to the scheduling at MAC.

Proposal 8: RAN2 to evaluate the treatment, logical channel prioritization and mapping of packets and PDU Sets to radio resources at the MAC layer.

### Identification of PDU Sets and packets

For 5G NR to support differentiated treatment of PDU Sets and consequently for identification of packets within a PDU Set, we anticipate that some changes to packet headers will be required, both for end-to-end signalling as well as at intermediate layers involved.

According to the ongoing XR study in SA2 [2], the UE and the network may identify the PDU Set based on:

- PDU Set sequence number (SN)

- Start/End PDU of the PDU Set

- PDU SN within a PDU Set

- Number of PDUs within a PDU Set

Based on the model and the parameter set that is chosen, RAN2 will need to identify whether/how any additional identification can be conveyed between different layers in the UE and the gNB. While awareness about which packets belong to which PDU Set is to be added, packets may or may not be carried within the same QoS flow. In some cases, lower layers will need to ensure these packets can be treated as a group.

In this context, RAN2 should study not only what header information is required for identification of PDU Sets and to support the burstiness of traffic, it should also consider scenarios where packets may be received out-of-sequence.

Proposal 9: RAN2 to study the type of header information required to describe and identify a PDU Set and the packets therein to facilitate XR traffic characteristics and scenarios such as out-of-sequence delivery of packets.

For completeness and with respect to XR awareness, even though this issue is more related to capacity, we would also like to mention that RAN and application awareness of the dependency between XR frames can be beneficial. For example, an XR application with video may use different types of frames (I/B/P) where the efficiency of the decoding may depend on the receive sequence and interrelation between frames depending on the type of frames.

Proposal 10: Study mechanisms for XR awareness of Inter PDU Set dependencies, including procedures and parameters for signalling.

# Conclusions

This contribution provides a first view on study areas around XR awareness as part of the RAN2 study for NR enhancements for XR. We have following observations and proposals:

Proposal 1: NR enhancements for XR should be motivated by XR traffic characteristics, especially the multiple data flow aspects.

Proposal 2: NR enhancements for XR should take new QoS parameters agreed by SA2 into account, to improve both user experience and resource efficiency.

Proposal 3: RAN2 to study and identify a set of QoS parameters to facilitate reliable and efficient transmission of PDU Sets at lower layers and in the 5G system, including awareness of packet /PDU set characteristic across different L2 sublayers and how to optimize the system for the bursty traffic characteristics exposed by XR.

Observation 1: Given the large variety of XR service types and deployment options, we assume CN assistance information to the RAN may not be suitable or available for all types of scenarios.

Proposal 4: Study scenarios where assistance information from the UE is useful, for example, based on service requirements and QoS, power consumption and capacity, including suitable methods and type of information to be provided.

Proposal 5: Study and identify RAN information suitable to provide guidance to XR applications, including related procedures.

Proposal 6: Study enhancements for application/higher layer influence and E2E parameter adjustments in interaction with SA4 and SA2.

Proposal 7: Study how RAN2 protocols can be aware of synchronization requirements to help ensure application layer packets of related traffic flows are delivered in close temporal proximity.

Observation 2: In the current 5G NR design the MAC layer cannot identify different QoS flows within a LCH and there is no clear mapping between CGs and QoS flows / QFIs.

Proposal 8: RAN2 to evaluate the treatment, logical channel prioritization and mapping of packets and PDU Sets to radio resources at the MAC layer.

Proposal 9: RAN2 to study the type of header information required to describe and identify a PDU Set and the packets therein to facilitate XR traffic characteristics and scenarios such as out-of-sequence delivery of packets.

Proposal 10: Study mechanisms for XR awareness of Inter PDU Set dependencies, including procedures and parameters for signalling.

# References

- RP-220285, Revised SID, Study on XR Enhancements for NR, Nokia, RAN#95e

- TR 23.700-60, Study on XR (Extended Reality) and media services, Rel-18

- H. Koumaras, C. Skianis, G. Gardikis, A. Kourtis, Analysis of H.264 Video Encoded Traffic, International Network Conference 2005, Samos, Greece

- S. Tanwir and H. Perros, A Survey of VBR Video Traffic Models, in IEEE Communications Surveys & Tutorials, vol. 15, no. 4, pp. 1778-1802, Fourth Quarter 2013

- R1-2008454, Views on XR application, traffic models and evaluation methodology, Apple, RAN1 #103-e, October 2020

- R1-2101365, Views on XR traffic models, Apple, RAN1 #104-e, January/February 2021

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR, Rel-17

- R1-2101765, LS to on XR-Traffic Models, SA4 #111-e, January, 2021

- TR 26.926, Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems, Rel-17

- TS 26.244, Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP), Rel-17

- TS 26.234, Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs, Rel-17

- TR 22.847, Study on supporting tactile and multi-modality communication services, Rel-18

- TR 22.842, Study on Network Controlled Interactive Services, Rel-17

- TS 22.261, Service requirements for the 5G system, Rel-18

- TR 26.928, Extended Reality (XR) in 5G, Rel-17

- S2-2201807, Key issue on traffic characteristics of XR and media service, Lenovo, Motorola Mobility, Huawei, HiSilicon, vivo, CATT, SA2#149e

- R2-2207430, Power Saving for Periodical XR Traffics, Apple, RAN2#119e, August 2022