3GPP TSG-RAN WG2 Meeting #119                                                 	R2-2207476

Online, 17th - 29th Aug 2022

Source:	CATT

Title:	Data for MRO related Enhancements

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

The R18 SONMDT WID has been revised on RAN#96 meeting [1]. The first bullet is about data collection for SON features:

- Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback, - Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2] - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

Since both features needs air interface extension and the reported content can be directly used in the NG-RAN node or be directly transmitted between NG-RAN nodes, it is appropriate to discuss them in RAN2 firstly. In this contribution, we analyse the 2 features, and intend to make clear the enhancements and the impact on RAN2.

Discussion

MRO for MR-DC SCG failure

The MRO for MR-DC SCG failure in NR-DC scenario has been supported in R17, and the enhanced parameters are all added in the SCGFailureInformation message in TS38.331. In RAN#96, it is agreed to include the MRO for MR-DC SCG failure scenario in R18 as R17 leftover.

For NR-DC scenario, the enhanced parameters in SCGFailureInformation message in TS38.331 include:

| parameter | IE/value & condition |
| --- | --- |
| failedPSCellId and previousPSCellId | Frequency and PCI |
| timeSCGFailure | 0-1023 |
| perRAInfoList | Only can be included if failureType is set to synchReconfigFailureSCG, or is set to randomAccessProblem while T304 is running |
| failureType | Reuse current R16/17 parameter |

The scenarios of MR-DC include NG-EN-DC, EN-DC, NE-DC. Since the parameters used for SCG failure optimization should be reported to the MN, the NG-EN-DC, EN-DC scenarios may have impact on TS36.331, and the NE-DC scenario may have impact on TS38.331. To reduce the impact on LTE specification, or to only enhance the NG-RAN node for MRO, it is necessary to consider which MR-DC scenarios should be included in the R18 SCG failure MRO scope.

Proposal 1: RAN2 to identify the MR-DC scenarios for which to perform the SCG failure related information report in R18.

For the identified scenarios, another thing needs to be decided is that whether the parameters reported in NR-DC scenario can be the baseline for other MR-DC scenarios.

Proposal 2: Identify whether the 5 information requested by RAN3 LS ‎ R3-211332 are all needed for MR-DC scenarios other than NR-DC.

In R17 discussion, it is concerned that whether the SCGFailureInformation message is appropriate to transmit the parameters for SCG MRO purpose. Since the usage of SCG failure report related messages is to request the network to resume the data transmission by a second link as soon as possible. Some companies thought that SON related UE stored measurements are not needed immediately after a failure and thus the mandatory messages’ (SCGFailureInformation, SCGFailureInformationNR, SCGFailureInformationEUTRA) size should not be increased for the SON purposes [2]. In the end of R17 discussion, it is compromise to use the SCGFailureInformation message for NR-DC scenario. Therefore, it is necessary to consider whether the SCG failure report related messages can still be used for transmitting the parameters for MRO purpose for other MR-DC scenarios, or other messages should be used.

Proposal 3: Identify whether the SCG failure report related messages can be used for transmitting the parameters for MRO purpose in MR-DC scenarios other than NR-DC scenario.

MRO enhancement for inter-system handover voice fallback

For MRO enhancement for inter-system handover voice fallback, it is described [3] that “the voiceFallbackIndication IE is introduced to be included in the MobilityFromNRCommand message for voice fallback purpose. UE shall select a suitable E-UTRA cell in priority after Inter-RAT HO failure happens. Failure reporting of Inter-RAT handover from NR to E-UTRA with the voice fallback purpose should be helpful.”  Obviously, the indicator in the RLF/HOF related report could assist the network to identify whether the failure is occurred because of inappropriate parameters configuration or because of voice fallback. The irrelevant failure, such as voice fallback, should be excluded from the MRO optimization. Then only an indicator is needed to be introduced in the HOF report.

Proposal 4:  Introduce a voice fallback indicator in HOF report to assist the network MRO.

Conclusion

According to the analysis in section 2, we propose:

For MR-DC SCG failure

Proposal 1: RAN2 to identify the MR-DC scenarios for which to perform the SCG failure related information report in R18.

Proposal 2: Identify whether the 5 information requested by RAN3 LS ‎ R3-211332 are all needed for MR-DC scenarios other than NR-DC.

Proposal 3: Identify whether the SCG failure report related messages can be used for transmitting the parameters for MRO purpose in MR-DC scenarios other than NR-DC scenario.

For inter-system handover voice fallback

Proposal 4:  Introduce a voice fallback indicator in HOF report to assist the network MRO.

Reference

- RP-221825 Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) CMCC

- R2-2108964	Report of [AT115e][871][SON/MDT] Modeling aspects related to information required by SN/SCG (CATT)

- RWS-210263 Considerations on SON related enhancements in Rel-18 Lenovo