3GPP TSG-RAN WG2 Meeting #119                                                 	R2-2207477

Online, 17th - 29th Aug 2022

Source:	CATT

Title:	General Considerations on SON MDT enhancements

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

The R18 SONMDT WID has been revised on RAN#96 meeting [1]. The second bullet is about mixed features of SON and MDT enhancements:

- Support of SON/MDT enhancements for [RAN3, RAN2]: - MR-DC CPAC - Successful PScell change report - Successful Handover Report (e.g. inter-RAT) - NPN - RACH report - fast MCG recovery - NR-U (MRO and UL MLB)

In this contribution, we analyse the last 4 features, and intend to make clear the enhancements and the impact on RAN2.

Discussion

NPN related optimization

The RAN part NPN feature is introduced in Release 16. The scope is to support the basic NPN functionality in NG-RAN which includes:

- CAG/SNPN relevant parameter broadcast;

- CAG/SNPN mobility in IDLE/INACTIVE and in CONNECTED states;

- CAG/SNPN access control.

In Release 17, only the SNPN feature has been enhanced in RAN from the 3 parts as below:

- Support SNPN along with subscription / credentials owned by an entity separate from the SNPN;

- Support UE onboarding and provisioning;

- Support of IMS voice and emergency services.

Since this is the first time to attempt NPN optimization in SONMDT, it is suggest only considering the R16 scope of NPN.

Proposal 1: Only consider the optimization of R16 basic NPN functionality in R18 SONMDT.

The NPN optimization in SONMDT can be studied from the perspectives of:

- MRO

- MDT

- L2 measurement

For MRO, the RLF/HOF report needs to be enhanced to include the NPN related information. One typical scenario is that only one handover target cell A satisfies handover triggering condition, but the UE cannot access to the cell A because UE does not support to access NPN cell. When analyze the RLF Report retrieved from the UE, the good handover target cell A is not selected as handover target cell. From the legacy reported parameters in the HOF report, we can only deduce the handover triggering condition should be optimized and cannot consider the failure is caused by UE NPN capability limit because the UE context has already been removed when the network performing MRO analysis. In order to correctly analyze RLF and handover failure involving with the NPN cell, the RLF/HOF Report shall be enhanced to include the NPN information, and at least the NID/CAG-ID should be included.

For MDT, it is considered that both the immediate MDT and logged MDT can be applied in the NPN related cell, whatever it is a mixed cell or a NPN only cell.

For L2 measurement, the NPN capable UE may have high priority than other normal UEs for access control or for flow control. Therefore some L2 measurement result recorded per UE may be impacted due to the different UE NPN capabilities. For example, the throughput or the data volume of the NPN capable UE could be better than the non-NPN capable UE, if the resource of the cell cannot satisfy all UE’s requirement. To let the operator know the reason of different L2 measurement result for different UEs, the NPN related information e.g. NPN capability or the NPN/PLMN the UE access to should be sent to the TCE together with the per UE L2 measurement result.

Proposal 2: The NPN optimization in SONMDT can be studied from MRO, MDT and L2 measurement perspectives.

RACH report related

SgNB RACH report for (NG)EN-DC scenario

SgNB RACH report for NR-DC scenario has been finished in R17.

In RAN3 LS [2] in R17, RAN3 asks RAN2 to consider UE RACH report for SgNBs, therefore the scenarios should include both (NG)EN-DC and NR-DC. But the (NG)EN-DC scenario is not supported due to limit of time. It is still agreed by RAN3 to introduce the X2AP change for EN-DC case for the SgNB RA report issue [3]:

Introduce the Access and Mobility Indication procedure to X2AP to transfer RACH Reports from eNB to en-gNB in EN-DC case.

To fulfill the RAN3 SgNB RACH report requirement, and to align the RAN2 and RAN3 procedures, it is suggest supporting the SgNB RACH report of (NG)EN-DC scenario in R18.

Proposal 3: Study and support the SgNB RACH report of (NG)EN-DC scenario in R18.

RACH report about RACH partitioning information

Background

In Rel-17, In order to enable early identification of the RACH feature or RACH feature combination on the network side, RACH partitioning was introduced in Rel-18.

- For RedCap, the UE can indicate the reduced capabilities to the network;

- For SDT, the UE can indicate the SDT transmission to the network, so that the network can perform suitable scheduling for the subsequent transmission;

- For coverage enhancement, the UE can indicate the network that the channel condition is poor. So that, MSG3 repetition can be performed;

- For Slicing, RACH isolation can be achieved and the UE can indicate that the traffic for specific NSAG (Network Slice AS Group) arrives;

- Some features can also be combined, for example, coverage enhancement and slicing. So that the network can identify the feature combinations.

Generally, the basic MAC procedure for RACH partitioning can be summarized as [4]:

- Step 1: RRC indicates to MAC whether SDT, Redcap, Slice group X is applicable for any RACH;

- Step 2: MAC selects the carrier, i.e. SUL or NUL;

- Step 3: MAC selects the BWP;

- Step 4: MAC determines CE applicability, i.e. MAC check whether the RSRP is below the configured RSRP threshold;

- Step 5: MAC selects the RACH partition.

RACH partitioning in MDT

To enable better utilization of the RACH resources, it is necessary for the UE report the triggered RACH feature to the network. For example, the UE can report the RACH feature or feature combination to the network. Hence, the network can reassign the RACH resources based on the triggered RACH feature/feature combination.

Proposal 4: The UE indicates the feature /feature combination to the network to facilitate the RACH resource utilization.

It may happen that multiple RACH features (feature combinations) are available or there is no RACH resource corresponding to the available RACH feature/feature combination for the UE. In order to handle these issues, RACH feature prioritization, i.e. featurePriorities is defined. The UE can select the feature with higher feature priority or feature combination which includes the feature configured with higher feature priority. So it may happen that the triggered RACH feature/feature combination is not the same as the available RACH feature/feature combination for the UE. For example, when the UE triggers NSAG1 and CE, but there is no RACH resource for NSAG 1+ CE. Hence, the UE can compare the feature priority of NSAG1 and CE. Assuming the CE has higher priority, the UE selects the RACH resource of CE. But this does not corresponding to the actual available RACH feature/feature combination, i.e. NSAG1+CE. So RAN2 needs to discuss whether and how to address this issue.

Observation 1: The RACH feature/feature combination which is selected by the UE may not be same as the RACH feature/feature combination that is available for the UE.

Proposal 5: RAN2 to study whether and how to address the issue in observation 1.

Fast MCG recovery

The fast MCG link recovery is introduced in R16. The specification mentioned that:

- If radio link failure occurred in MCG, first the MCG failure should be recorded in VarRLF-Report, and then UE should initiate the MCG failure information procedure if condition fulfilled;

- If MCG failure is sent to the network and if any of the response message of RRCRelease/ MobilityFromNRCommand/RRCReconfiguration with reconfigurationwithSync for the PCell is received before T316 expired, the VarRLF-Report will be cleared.

The UE action of clearing the VarRLF-Report upon successful fast MCG recovery is agreed in ASN.1 review and is merged in [5] which have been discussed under MDT topic. Therefore, we consider other enhancement such as failure of fast MCG recovery may continue be discussed in R18.

Observation 2: UE action of clearing the VarRLF-Report upon successful fast MCG recovery is supported in R16.

For other enhancements, [6] already has some history discussion. In our view, 3 scenarios as follows need to be discussed for failure of fast MCG recovery:

- MCG failure only. This scenario may be caused by no T316 configuration or T316 expire, thus no fast MCG recovery can be successfully performed by the UE, the RLF report for MCG failure, i.e. legacy RLF report content could be considered;

- MCG failure, and then SCG failure. This scenario includes the cases of SCG failure occurs during T316 running/ SCG failure upon MCG is suspended. Since the MCG link cannot be recovered via SCG link, the already recorded MCG failure related information needs to be report to the network, maybe binding with the failed SCG information;

- MCG failure, and SCG has already failed. This scenario includes the cases of MCG failure occurs while SCG link has not been recovered/ MCG failure upon SCG is suspended. Since the MCG link cannot be recovered and the recovery message cannot be transmitted because of SCG already failure, the already recorded SCG failure related information needs to be report to the network, maybe binding with the failed MCG information.

Since all the 3 scenarios may have relationship with the MCG recovery failure, before discussing the UE behavior of how to record or send the fast MCG recovery related information to the network, it is suggest discussing whether the scenarios listed above could all be included in the scope first.

Proposal 6: Discuss the scope of fast MCG recovery related scenarios, which include:

- MCG failure only;

- MCG failure occurred, and then SCG failure;

- MCG failure occurred, and SCG has already failed.

NR-U

In Rel-16, NR-U was introduced. In order to notify the network that LBT failure happens on the SpCell, the UE can switch to one BWP in which no consistent LBT failure has been triggered and perform RACH as described in [4]. Therefore, consistent LBT failure can be defined as one RA purpose.

Proposal 7: Add a new RA purpose of LBT on SpCell.

RAN3/RAN2 work split

For the first RAN2 meeting of Rel-18, it is proposed to discuss the RAN3/RAN2 work split and try to agree on a principle to guide the subsequent work. For the last 4 features listed in this contribution, we have the consideration on RAN3/RAN2 split as below:

| Feature name | RAN2 | RAN3 |
| --- | --- | --- |
| NPN | MDT and L2 measurement part can be started from RAN2 | MRO part can be started from RAN3 |
| RACH report | RAN2 discusses SgNB RACH report for (NG)EN-DC scenario first, since RAN3 has already introduced the X2AP change for EN-DC case | RACH partitioning can be started from RAN3 since most enhancements are used for SON |
| Fast MCG recovery | Most recovery work are performed in RAN2, and the scenarios can be discussed first | Can be discussed following RAN2 decision |
| NR-U | Can be discussed following RAN3 decision | MRO and MLB optimizations for NR-U could be continued based on R17 discussion |

Therefore, we propose to start some of the features from RAN2, and others can be started from RAN3 first.

Proposal 8: RAN2 first to discuss the features as below:

- MDT and L2 measurement part of NPN enhancement;

- SgNB RACH report for (NG)EN-DC;

- Fast MCG recovery enhancement.

Conclusion

According to the analysis in section 2, we propose:

For NPN related optimization

Proposal 1: Only consider the optimization of R16 basic NPN functionality in R18 SONMDT.

Proposal 2: The NPN optimization in SONMDT can be studied from MRO, MDT and L2 measurement perspectives.

For RACH report related

Proposal 3: Study and support the SgNB RACH report of (NG)EN-DC scenario in R18.

Proposal 4: The UE indicates the feature /feature combination to the network to facilitate the RACH resource utilization.

Observation 1: The RACH feature/feature combination which is selected by the UE may not be same as the RACH feature/feature combination that is available for the UE.

Proposal 5: RAN2 to study whether and how to address the issue in observation 1.

For Fast MCG recovery

Observation 2: UE action of clearing the VarRLF-Report upon successful fast MCG recovery is supported in R16.

Proposal 6: Discuss the scope of MCG recovery enhancement related scenarios, which include:

- MCG failure only;

- MCG failure occurred, and then SCG failure;

- MCG failure occurred, and SCG has already failed.

For NR-U

Proposal 7: Add a new RA purpose of LBT on SpCell.

RAN3/RAN2 work split

Proposal 8: RAN2 first to discuss the features as below:

- MDT and L2 measurement part of NPN enhancement;

- SgNB RACH report for (NG)EN-DC;

- Fast MCG recovery enhancement.

Reference

- RP-221825 Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) CMCC

- R3-205662 LS to RAN2 on RACH report for SgNB RAN3

- RAN3_114-e_agenda_20211111_EOM

- TS38.321 Medium Access Control (MAC) protocol specification v17.1.0

- R2-2006412 Corrections on MDT and SON in NR Huawei, Ericsson, HiSilicon