3GPP TSG-RAN WG2 Meeting #119-e                                                    R2- 2207512

Electronic, 17th – 26th August, 2022

Source:              CATT

Title:	             Consideration on UE Assistance Information

Agenda Item:	8.3.2

Document for:	Discussion and Decision

Introduction

A new study item was agreed in RAN#94-e addressing “Network energy savings” [1]. The following is RAN2/3 related objective in the SI:

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

This contribution discusses our consideration on UE assistance information for network energy saving.

Discussion

RAN1 has already started the first discussion on the SI of network energy saving in RAN1#109e. Some agreements were achieved [2]. UE assistance information associated RAN1 agreements are as follows:

Agreement Further study techniques and enhancements for increasing time domain energy saving opportunities by the gNB, including (but not limited to) the following aspects: […] - Mechanism to utilize potential energy saving states or sleep modes and the transition between states from leveraging cell on/off opportunities - including studies of waking up gNB due to user traffic, or user density, or gNB receiving wake up signal - including technique to allow discovery and measurement of cells in sleep or dormant states - UE assistant information facilitating BS time domain adaptation Agreement Further study techniques and enhancements for the adaptation of number of spatial elements of the gNB, including (but not limited to) the following aspects: […] - feedback/assistance information from the UE required for support dynamic spatial element adaptation - for example, CSI measurement and reports, SR, etc - signaling methods, including reduced signaling, for enabling dynamic spatial element adaptation […] Agreement Further study techniques and enhancements on assistance information from the UE to aid the gNB to perform energy saving techniques - Some examples of assistance information are, but not limited to: - preferred SSB configurations, - indication of semi-static UL channel transmissions, - indication of UE’s buffer status for UL channel transmissions, - UE traffic information such as service priority, delay tolerance, data rate, data volume, traffic type, time criticality, and packet size(s), - coverage, mobility status, location. - conditions for triggering the assistance information from the UE

According to the associated RAN1 agreements, some UE assistance information is strongly related to specific NES techniques. For example: in order to wake up cells in sleep or dormant state, the UE needs to send a wake up signal. Or in order to assist the network to wake up some cells, the UE can perform discovery and measurement of cells in sleep or dormant states. As it is part of a specific network energy saving technique, such UE assistance information should be discussed under certain NES techniques.

Proposal 1: If UE assistance information is part of a specific network energy saving technique, it should be discussed and evaluated under the topic of the specific network energy saving technique.

In addition, RAN2 can consider the general UE assistance information (UAI) framework to assist network for energy saving. Different from UE assistance info for UE power saving, in order to make proper decision on getting a cell to sleep or “off”, NW needs to have an exhaustive UAI picture from all UEs in the cell. Hence, it is not feasible to collect exhaustive UAI in this case. But a cell may wake up or turn to work normal for some specific UEs only. For example: some UEs with large traffic to be transmitted, or some UEs entering an area could require to wake up a sleeping cell. Traffic information, or “expected itinerary/destination” information allowing an estimation of UE trace can be used to assist the network in estimating whether to wake up a cell. However, at least, traffic info may also be obtained from CN. So RAN2 can study further which info is necessary to be provided from the UE to assist NES.

Proposal 2: RAN2 to study further the necessary common UE assistance information for network energy saving.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: If UE assistance information is part of a specific network energy saving technique, it should be discussed and evaluated under the topic of the specific network energy saving technique.

Proposal 2: RAN2 to study further the necessary common UE assistance information for network energy saving.

References

- RP-213554, New SI: Study on network energy savings for NR, RAN#94-e, Huawei

- Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0