3GPP TSG-RAN WG2 Meeting #119-e	R2-2207523

# Electronic meeting, 17th - 29th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3274 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on reception of RRCReconfigurationSidelink message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In subclause 5.8.9.1.3, it is unclear that Rx UE accepts the SL DRX configuration is in the case of UE can comply with the SL DRX but be unable to comply with all other configurations or in the case of UE can comply with the SL DRX and all other configurations. - In subclause 5.8.9.1.3, the agreement that non-DRX will be applied if SL DRX is rejected for SL UC has not been specified for RX UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 5.8.9.1.3, move the procedure that Rx UE accepts the SL DRX configuration to the procedure where UE can comply with all SL configurations. - In subclause 5.8.9.1.3, for RX UE, add the description that the UE will not apply the SL DRX if it rejects the SL DRX configuration. Impact analysis Impacted functionality: NR sidelink communication Inter-operability: For change 1, no UE Inter-operability will be introduced. For change 2, if one UE implements this CR but the other UE does not, the receving UE may have different understanding on the SL DRX applied or not with the trasmitting UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For change1, the Rx UE will accept the SL DRX configuration in the case of UE can comply with the SL DRX but be unable to comply with the other configuration, and Rx UE will configure lower layers to perform the configured sidelink DRX operation. But in the following procedure, the Rx UE will apply the prior configuration because UE is unable to comply with the other configuration and RRCReconfigurationFailureSidelink message is transmitted, which is conflict to that Rx UE accepts the SL DRX configuration. For change 2, Tx UE and Rx UE may have different understanding to SL DRX applied or not, the SL communication will be low qualified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.8.9.1.3	Reception of an RRCReconfigurationSidelink by the UE

The UE shall perform the following actions upon reception of the RRCReconfigurationSidelink

<skip>

1>	if the RRCReconfigurationSidelink message includes the sl-LatencyBoundIUC-Report:

2>	apply the configured sidelink IUC report latency bound;

1>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfigurationSidelink (i.e. sidelink RRC reconfiguration failure):

2>	continue using the configuration used prior to the reception of the RRCReconfigurationSidelink message;

2>	set the content of the RRCReconfigurationFailureSidelink message;

3>	submit the RRCReconfigurationFailureSidelink message to lower layers for transmission;

1>	else:

2> if the RRCReconfigurationSidelink message includes the sl-DRX-ConfigUC-PC5, and

3> if the UE rejects the sl-DRX-ConfigUC-PC5:

4> include the sl-DRX-ConfigReject in the RRCReconfigurationCompleteSidelink message;

4> consider no sidelink DRX to be applied for the corresponding sidelink unicast communication;

3> else if the UE accepts the sl-DRX-ConfigUC-PC5:

4> configure lower layers to perform sidelink DRX operation according to sl-DRX-ConfigUC-PC5 for the associated destination as defined in TS 38.321 [3];

2>	set the content of the RRCReconfigurationCompleteSidelink message;

3>	submit the RRCReconfigurationCompleteSidelink message to lower layers for transmission;

NOTE 1:	When the same logical channel is configured with different RLC mode by another UE, the UE handles the case as sidelink RRC reconfiguration failure.

NOTE 2: It is up to the UE whether or not to indicate the rejection to the peer UE for a received sidelink DRX configuration.

END OF CHANGE