3GPP TSG-RAN WG2 119-e	R2-2207535

17th August – 26th August 2022

GTW

Source:                    	NTT DOCOMO, INC.

Title:			Discussion on L1L2 mobility

Document for:        	Discussion and decision

Agenda Item:         	8.4.2.2	Candidate Solutions

- Introduction

In Rel18 WID of Further NR mobility Enhancements, the following objective was agreed [1]. In this contribution, we discuss about applicable scenario of L1L2 mobility, measurement and simultaneous setting with legacy mobility.

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

  - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- Timing Advance management [RAN1, RAN2]

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 2: FR2 specific enhancements are not precluded, if any.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

- Discussion

As mentioned in justification of WID, in legacy L3 mobility procedure, RRC reconfiguration is triggered by L3 measurements. In addition, MAC reset, RLC re-establishment, and PDCP re-establishment/data recovery are triggered in Reconfiguration with Sync as defined in TS 38.331 [1]. For inter-CU inter-cell handover, it may be difficult to avoid L3 signalling and L1L2 reset. On the other hands, for intra-CU inter-cell handover (intra-DU inter-cell handover or inter-DU inter-cell handover), it is expected that avoiding L1 L2 reset or partial resetting will reduce the interruption time and achieve the scope of this WI. It is assumed that intra-DU can further omit various resets compared to inter-DU. But L1L2 mobility mechanisms should be compatible in both cases.

Observation1: In Legacy intra-CU inter-cell handover (intra-DU inter-cell handover or inter-DU inter-cell handover), MAC reset, RLC re-establishment, and PDCP re-establishment/data recovery is triggered.

Proposal1: RAN2 prioritize to study intra-CU inter-cell handover (intra-DU inter-cell handover or inter-DU inter-cell handover)

In order to trigger L1L2 based inter cell mobility, it is assumed that measurements must be performed. To trigger legacy mobility, there are two types of measurements: L1 measurement used for L1 beam management and L3 measurement used for L3 mobility. It is expected that use of each of the existing measurements causes gain reduction or cannot be used as it is. An example of the reason is as follows. L1 measurement cannot measure the quality of neighbouring cells. L3 measurement is based on L3 filtering to prevent ping pong due to short-term quality changes as defined in TS38.331[3]. Use of L3 filtering and L3 signaling will cause delays and lower gain of L1L2 mobility. Therefore, for L1L2 mobility, L1 measurement and L3 measurement need to be optimized, and RAN2 should study about it.

Proposal2: RAN2 should study L1 or L3 measurement optimization to trigger L1L2 based inter-cell mobility.

There are possible scenarios in which setting L1L2 mobility and L3 mobility simultaneously would be effective. One is when L1L2 mobility fails. In addition to reinitiating or reconfiguring L1L2 mobility, it is possible to fall back to L3 mobility. To fall back to L3 mobility, it is necessary to set up L3 mobility in advance. Other case is when Inter-CU and Intra-CU mobility are evaluated at the same time. In this scenario, L1L2 mobility and L3 mobility are evaluated at the simultaneous time to efficiently perform mobility. Therefore, RAN2 should study if simultaneous use of L1L2 mobility and L3 mobility has any performance gain, and if so, discuss associated RAN2 requirements accordingly.

Proposla3: RAN2 should study if simultaneous use of L1L2 mobility and L3 mobility has any performance gain, and if so, discuss associated RAN2 requirements accordingly.

- Conclusion

Based on the above discussion, we have the following observations and proposals:

Observation1: In Legacy intra-CU inter-cell handover (intra-DU inter-cell handover or inter-DU inter-cell handover), MAC reset and RLC and PDCP re-establishment, PDCP data recovery is triggered.

Proposal1: RAN2 prioritize to study intra-CU inter-cell handover (intra-DU inter-cell handover or inter-DU inter-cell handover)

Proposal2: RAN2 should study L1 or L3 measurement optimization to trigger L1L2 based inter-cell mobility.

Proposla3: RAN2 should study if simultaneous use of L1L2 mobility and L3 mobility has any performance gain, and if so, discuss associated RAN2 requirements accordingly.

- References

- RP-213565

- TS 38.331 5.3.5 RRC reconfiguration

- TS 38.331 5.5.3.2 Layer 3 filtering