3GPP TSG-RAN WG2 Meeting #119 electronic                               R2-2207583

Online, Aug 17th – 29th, 2022

Source:         ZTE, Sanechips

Title:            Discussion on the framework of TEG timing error margin

Agenda item:    6.11.2.6

Document for:	Discussion and Decision

- Introduction

In RAN2#118e, the following LS [1] from RAN4 is received:

- The framework of UE/TRP Rx TEG: - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF. - For UE that supports multiple Rx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Rx TEG is M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID. - The RRM accuracy requirements corresponding to the candidate timing error margin values {TE1, TE2, …, TEN} will be defined in Perf part. - The framework of UE/TRP Rx TEG can be also applied for UE/TRP RxTx TEG - Note: if additional issues are identified based on RAN1/2 progress, then this agreement can be revised.

In RAN2#119e, a new LS [2] from RAN4 is received:

- The framework of UE/TRP Tx TEG is defined as below: - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to gNB or LMF. - For UE that supports multiple Tx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Tx TEG is M, which means the timing error difference between the transmission occasions of same or different SRS resources within the same Tx TEG is within the margin M. - The applicability of reported UE Tx TEG is limited to the transmission occasions of same or different SRS resources within the validity time defined by RAN2 e.g. based on the time stamp information. - Candidate timing error margins for UE/TRP Rx TEGs: - 0Tc, 2 Tc, 4 Tc, 6 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 56 Tc, 64 Tc, 72 Tc, 80 Tc. - For UE/TRP Tx TEGs, Use the same candidate timing error margins as UE/TRP Rx TEG. - The reported value for Tx TEGs, Rx TEGs and RxTx TEGs can be different. - The reported value for Tx/Rx/RxTx TEGs can be different at different times.

The framework of UE/TRP Rx/Tx/RxTx TEG and corresponding margin value should be captured in the spec based on RAN4’s LSs, including 37.355, 38.331 and 38.455. This contribution discusses about how to capture the signaling in these spec.

- Discussion

The timing error margin value is the timing error difference within a Rx/Tx/RxTx TEG. Between TEGs, the timing error difference may be large and no spec restriction is applied for the TEG pair. Further from RAN4’s LSs, we observe that:

- When UE or TRP supports multiple Rx TEGs, the timing error margin value for each Rx TEG should be the same value, e.g. M1;

  - If time varies, M1 may change to a different value, e.g. M1’;

- When UE or TRP supports multiple Tx TEGs, the timing error margin value for each Tx TEG should be the same value, e.g. M2;

  - If time varies, M2 may change to a different value, e.g. M2’;

- When UE or TRP supports multiple RxTx TEGs, the timing error margin value for each RxTx TEG should be the same value, e.g. M3;

  - If time varies, M3 may change to a different value, e.g. M3’;

- M1, M2, M3, M1’, M2’ and M3’ may not be equal;

- The possible candidate value of M1, M2, M3, M1’, M2’ and M3’ should be the same, that is, they choose values from the same pool {0Tc, 2 Tc, 4 Tc, 6 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 56 Tc, 64 Tc, 72 Tc, 80 Tc};

Therefore from our point of view, the timing error margin value is a single value for all the Rx TEGs (or all Tx TEGs, or all RxTx TEGs) at a certain time (or time period), rather than timing error margin is a list containing multiple values, each value corresponds to a Rx TEG (or a Tx TEG, or a RxTx TEG). That is to say, timing error margin value is not associated with certain Rx/Tx/RxTx TEG ID.

Observation 1: At UE side or at TRP side, the timing error margin value is a single value for all the Rx TEGs (or all Tx TEGs, or all RxTx TEGs) at a certain time (or time period), and the value may vary with time.

Furthermore to design the signaling, the followings should be acknowledged:

- UE Tx TEG error margin value is reported to gNB (38.331) or LMF (37.355);

- UE Rx/RxTx TEG error margin value is reported to LMF (37.355);

- TRP Tx TEG error margin value is reported to LMF (38.455), then LMF include the TRP Tx TEG error margin value in the assistance data (37.355);

- TRP Rx/RxTx TEG error margin value is reported to LMF (38.455).

Since 38.455 changes should be left to RAN3, the detailed signaling design of 37.355 and 38.331 is presented in the following sub-sessions.

Observation 2: UE Tx TEG error margin value should be added in 38.331, UE Tx TEG error margin value, UE Rx TEG error margin value, UE RxTx TEG error margin value and TRP Tx TEG error margin value should be added in 37.355.

Because UE/TRP Rx/Tx/RxTx TEGs share a same candidate value list, to increase the protocol readability, we suggest to introduce a new IE TEGTimingErrorMargin-r17 in 37.355, then this IE can be quoted at several other places. The example of TEGTimingErrorMargin-r17 is:

–	TEGTimingErrorMargin

The IE TEGTimingErrorMargin defines the timing error margin value of TEG(s). Value tc0 corresponds to 0 Tc, tc2 corresponds to 2 Tc and so on.

-- ASN1START

TEGTimingErrorMargin-r17 ::= ENUMERATED {tc0, tc2, tc4, tc6, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc56, tc64, tc72, tc80}

-- ASN1STOP

The draft CR of the above changes is also provided in R2-2207581 [3].

Proposal 1: Support to introduce a new IE in 37.355 to define the timing error margin value of TEG(s) as in R2-2207581.

Another question is for LPP UE measurements reporting, whether the TEG timing error margin should be associated with each LPP measurement report, or with each measurement instance in a LPP measurement report. In RAN1#109e [4], the following agreement is achieved:

Agreement In the reply LS to RAN4 (cc RAN2/RAN3), - Ask RAN4 whether UE Rx/RxTx TEG margins are provided to LMF as UE capability, or as LPP signalling parameters outside of UE capability signaling. If RAN4 considers UE Rx/RxTx TEG margins are provided to LMF as LPP signalling parameters outside of UE capability signaling, further ask RAN4 the following questions: - Whether a single timing error margin value is provided per Rx TEG/RxTx TEG type in a single LPP message, even if it has multiple measurement instances; - Whether the timing error margin values for a Rx TEG/RxTx TEG type in different LPP messages can be different; - RAN1 understands the TRP Rx/RxTx TEG margins are provided to the LMF via an NRPPa message and which message to contain the TEG margins is up to RAN3.

RAN1 asked RAN4 this issue but no explicit answer is received. To the common understanding, the intension of different measurement instances is to restrict measurements in a time period where timing error of a single TEG does not change. That is to say, measurement instances can be divided by the time order. Furthermore, the RAN4’s reply LS mentioned ‘The reported value for Tx/Rx/RxTx TEGs can be different at different times’, based on the above analysis and the consideration of flexibility, we think for UE reporting, it is better to contain timing error margin value in each measurement instance, rather than each measurement report.

Proposal 2: For LPP UE reporting, support to contain the timing error margin value in each measurement instance.

2.1 LPP signaling of UE Rx/Tx/RxTx TEG with the margin value

UE Rx TEG ID is reported in DL-TDOA and multi-RTT case 3, and UE RxTx TEG ID is reported in multi-RTT case 1 and case 2.

For DL-TDOA method, The Rx TEG timing error margin should be embedded in NR-DL-TDOA-SignalMeasurementInformation. Since RAN4 has clearly said that ‘The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID’, the Rx TEG timing error margin value should not be associated with each RSTD measurements but set parallel to dl-PRS-ReferenceInfo-r16, for all the RSTD measurements (and corresponding Rx TEG IDs) in a NR-DL-TDOA-SignalMeasurementInformation. The example is shown below:

–	NR-DL-TDOA-SignalMeasurementInformation

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...,

[[

nr-DL-TDOA-UE-RxTEG-TimingErrorMargin-r17   TEGTimingErrorMargin-r17, OPTIONAL, --Cond UERxTEG

]]

}

For multi-RTT method, since there are three cases of TEG reporting, the TEGTimingErrorMargin should apply toRx TEG, Tx TEG, RxTx TEG, respectively. The reference design is shown as below:

–	NR-Multi-RTT-SignalMeasurementInformation

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3	]]

[[

nr-Multi-RTT-UE-TEG-TimingErrorMargin-r17    SEQUENCE {

nr-Multi-RTT-UE-RxTEG-TimingErrorMargin-r17  TEGTimingErrorMargin-r17  OPTIONAL, --Cond Case3

nr-Multi-RTT-UE-TxTEG-TimingErrorMargin-r17  TEGTimingErrorMargin-r17 OPTIONAL, --Cond Case2-3

nr-Multi-RTT-UE-RxTxTEG-TimingErrorMargin-r17   TEGTimingErrorMargin-r17  OPTIONAL, --Cond Case1-2

}

]]

}

The draft CR of the above changes and corresponding condition description and field description is also provided in R2-2207581 [3].

Proposal 3: Support to add UE Rx TEG timing error margin in LPP signaling NR-DL-TDOA-SignalMeasurementInformation as in R2-2207581.

Proposal 4: Support to add UE Rx/Tx/RxTx TEG timing error margin in LPP signaling NR-Multi-RTT-SignalMeasurementInformation as in R2-2207581.

2.2 LPP signaling of TRP Tx TEG with the margin value

TRP Tx TEG is configured in assistance data in NR-DL-PRS-TRP-TEG-Info, which includes all PRS resource and TRP Tx TEG association of all the TRPs and all the frequency layers. Since TRP Tx TEG timing error margin value should be associated with each TRP (no matter how many TRP Tx TEGs are associated with this TRP), the example design is shown as below:

–	NR-DL-PRS-TRP-TEG-Info

-- ASN1START

NR-DL-PRS-TRP-TEG-Info-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-TRP-TEG-InfoPerFreqLayer-r17

NR-DL-PRS-TRP-TEG-InfoPerFreqLayer-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-TRP-TEG-InfoPerTRP-r17

NR-DL-PRS-TRP-TEG-InfoPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17							INTEGER (0..255),

nr-PhysCellID-r17						NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r17						NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-r17							ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

dl-PRS-TEG-InfoSet-r17					SEQUENCE (SIZE(1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-TEG-InfoPerResourceSet-r17,

...,

[[

nr-TRP-TxTEG-TimingErrorMargin-r17   TEGTimingErrorMargin-r17, OPTIONAL, -- Need ON

]]

}

DL-PRS-TEG-InfoPerResourceSet-r17 ::= SEQUENCE (SIZE(1..nrMaxResourcesPerSet-r16)) OFs

DL-PRS-TEG-InfoElement-r17

DL-PRS-TEG-InfoElement-r17 ::= SEQUENCE {

dl-prs-trp-Tx-TEG-ID-r17		INTEGER (0..maxNumOfTRP-TxTEGs-1-r17),

...

}

-- ASN1STOP

The draft CR of the above changes and corresponding field description is also provided in R2-2207581 [3].

Proposal 5: Support to add TRP Tx TEG timing error margin in LPP signaling NR-DL-PRS-TRP-TEG-Info as in R2-2207581.

2.3 RRC signaling of UE Tx TEG with the margin value

UE can report Tx TEG and SRS association in RRC signaling UEPositioningAssistanceInfo, as RAN4’s indication on UE Tx TEG timing error margin, UE selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to gNB or LMF. Therefore, UE Tx TEG timing error margin value is also needed in the RRC signaling. The example design is shown as below:

–	UEPositioningAssistanceInfo

UEPositioningAssistanceInfo message

-- ASN1START

-- TAG-UEPOSITIONINGASSISTANCEINFO-START

UEPositioningAssistanceInfo-r17 ::= SEQUENCE {

criticalExtensions                  CHOICE {

uePositioningAssistanceInfo-r17     UEPositioningAssistanceInfo-r17-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEPositioningAssistanceInfo-r17-IEs ::= SEQUENCE {

ue-TxTEG-AssociationList-r17            UE-TxTEG-AssociationList-r17        OPTIONAL,

lateNonCriticalExtension                OCTET STRING                        OPTIONAL,

nonCriticalExtension                     UE-TxTEG-TimingErrorMargin-v17xx-IEs                        OPTIONAL

}

UE-TxTEG-TimingErrorMargin-v17xx-IEs ::= SEQUENCE {

UE-TxTEG-TimingErrorMarginValue-v17xx-IEs              ENUMERATED {tc0, tc2, tc4, tc6, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc56, tc64, tc72, tc80}               OPTIONAL,

nonCriticalExtension         SEQUENCE {}                                       OPTIONAL,

}

UE-TxTEG-AssociationList-r17 ::= SEQUENCE (SIZE (1..maxNrOfTxTEGReport-r17)) OF UE-TxTEG-Association-r17

UE-TxTEG-Association-r17 ::=        SEQUENCE {

ue-TxTEG-ID-r17                     INTEGER (0..maxNrOfTxTEG-ID-1-r17),

nr-TimeStamp-r17                    NR-TimeStamp-r17,

associatedSRS-PosResourceIdList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16,

servCellId-r17                      ServCellIndex                            OPTIONAL

}

NR-TimeStamp-r17 ::= SEQUENCE {

nr-SFN-r17           INTEGER (0..1023),

nr-Slot-r17          CHOICE {

scs15-r17            INTEGER (0..9),

scs30-r17            INTEGER (0..19),

scs60-r17            INTEGER (0..39),

scs120-r17           INTEGER (0..79)

},

...

}

-- TAG-UEPOSITIONINGASSISTANCEINFO-STOP

-- ASN1STOP

Since the new IE UE-TxTEG-TimingErrorMargin is added in the UEPositioningAssistanceInfo message, this should also be described in the corresponding procedure description, i.e. section 5.7.14.3. The example is shown below:

- 5.7.14.3 Actions related to transmission of UEPositioningAssistanceInfo message The UE shall set the contents of the UEPositioningAssistanceInfo message as follows: 1> if ue-TxTEG-RequestUL-TDOA-Config in RRCReconfiguration message is configured with periodicReporting; 2> for all the association changes store ue-TxTEG-Association corresponding to each ue-TxTEG-ID with nr-TimeStamp; 2> include the results in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message on expiry of each configured period; 2> include one timing error margin value for all the UE Tx TEGs containing in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message. 1> else if ue-TxTEG-RequestUL-TDOA-Config in RRCReconfiguration message is configured with oneShot: 2> identify the ue-TxTEG-Association corresponding to each ue-TxTEG-ID with nr-TimeStamp; 2> include the results in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message only one time. 2> include one timing error margin value for all the UE Tx TEGs containing in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message. The UE shall submit the UEPositioningAssistanceInfo message to lower layers for transmission.

The draft CR of the above changes is also provided in R2-2207582 [5].

Proposal 6: Support to add UE Tx TEG timing error margin in RRC signaling UEPositioningAssistanceInfo and complete the corresponding procedure description as in R2-2207582.

- Conclusion

Observation 1: At UE side or at TRP side, the timing error margin value is a single value for all the Rx TEGs (or all Tx TEGs, or all RxTx TEGs) at a certain time (or time period), and the value may vary with time.

Observation 2: UE Tx TEG error margin value should be added in 38.331, UE Tx TEG error margin value, UE Rx TEG error margin value, UE RxTx TEG error margin value and TRP Tx TEG error margin value should be added in 37.355.

Proposal 1: Support to introduce a new IE in 37.355 to define the timing error margin value of TEG(s) as in R2-2207581.

Proposal 2: For LPP UE reporting, support to contain the timing error margin value in each measurement instance as in R2-2207581.

Proposal 3: Support to add UE Rx TEG timing error margin in LPP signaling NR-DL-TDOA-SignalMeasurementInformation.

Proposal 4: Support to add UE Rx/Tx/RxTx TEG timing error margin in LPP signaling NR-Multi-RTT-SignalMeasurementInformation as in R2-2207581.

Proposal 5: Support to add TRP Tx TEG timing error margin in LPP signaling NR-DL-PRS-TRP-TEG-Info as in R2-2207581.

Proposal 6: Support to add UE Tx TEG timing error margin in RRC signaling UEPositioningAssistanceInfo and complete the corresponding procedure description as in R2-2207582.

- References

- R2-2204139  LS on the UE/TRP TEG framework  LS in  Rel-17  From: RAN4  To: RAN1, RAN2.

- R2-2206946  LS on the Tx TEG framework  LS in  Rel-17  From: RAN4  To: RAN1, RAN2, RAN3.

- R2-2207581  Correction on UE Rx/Tx/RxTx TEG and TRP Tx TEG timing error margin in 37.355, 2022-08

- Report of 3GPP TSG RAN WG1 meeting #109-e, Online, May, 2022.

- R2-2207582  Correction on UE Tx TEG timing error margin in 38.331, 2022-08.