3GPP TSG-RAN WG2 Meeting #119 electronic          	          R2-2207595

Online, August 17-29, 2022

Agenda Item:	6.1.5

Source: 	Huawei, Xiaomi, CBN, HiSilicon

Title: 	PDCP state variables handling during multicast MRB suspend

Document for:	Discussion and decision

- Introduction

In the last meeting, it was agreed by RAN2 to set the initial RX_NEXT to 0, and configure the initial RX_DELIV by multicastHFN-AndRefSN in RRC signaling. Based on this, in this contribution, we will discuss the operations of receiving PDCP entity in TS 38.323 for multicast MRB when a PDCP entity suspend request is received from upper layers.

- Discussion

Currently, it is specified in TS38.323:

b) RX_DELIV For broadcast MRBs, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs, the initial value of RX_DELIV is set by multicastHFN-AndRefSN in TS 38.331 [3]. NOTE 3: For broadcast MRBs, the initial value of the HFN part of RX_DELIV is set by UE implementation.

From the above description, we can see that the initialization of parameters for multicast MRB is different from that of SRB and DRB. But according to the specification, when the UE configured with multicast MRB(s) receives RRCRelease with suspendConfig, it will suspend all SRB(s), DRB(s) and multicast MRB(s), except SRB0, and request the receiving PDCP entity to perform as follows:

When upper layers request a PDCP entity suspend, the receiving PDCP entity shall: - if t-Reordering is running: - stop and reset t-Reordering; - deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression; - set RX_NEXT and RX_DELIV to the initial value.

There is no problem with suspending multicast MRBs. But unlike SRB and DRB, it is not reasonable to initiate RX_DELIV for the multicast MRB when suspending it.

Suspending a multicast MRB is mainly because the corresponding MBS session is deactivated. It is also gNB’s implementation to not suspend the multicast MRB for some UEs, in which case the PDCP state variables are not initialized. Therefore, the PDCP state variables shouldn’t be initialized when suspending a multicast MRB for some UEs. Otherwise there will be misalignment of PDCP state variables between inactive UEs and connected UEs.

Observation1: Initializing the PDCP state variables when suspending a multicast MRB may cause misalignment of PDCP window maintaining between inactive UEs and connected UEs.

Besides, when request of PDCP entity suspend is received, the UE may not have a multicastHFN-AndRefSN value to set RX_DELIV to, which will cause unexpected PDCP operation failure for the multicast MRB. And it is also impossible to initialize during resume as the multicastHFN-AndRefSN can only be configured in case of multicast MRB setup as below.

multicastHFN-AndRefSN-r17      BIT STRING (SIZE (32))         OPTIONAL    -- Cond SetupOnlyMRB

Even if it can be allowed to configure during resume, no initialization procedure is supported in PDCP specs.

Observation2: Initializing the PDCP state variables when suspending a multicast MRB is not possible according the specification.

As a matter of fact, as service may continue in a while from where it was stopped, it is better that the previous PDCP state variables are not initialized and no extra UE behavior is needed. In this case, the PDCP state variables can be continued to be used when the multicast MRB is resumed.

Proposal: When upper layers request a PDCP entity suspend for the multicast MRB, no specific behavior is needed for RX_NEXT and RX_DELIV, i.e. no initialization for RX_NEXT and RX_DELIV.

- Conclusion

Based on the above discussion, we recommend RAN2 to adopt the following proposal:

Observation1: Initializing the PDCP state variables when suspending a multicast MRB may cause misalignment of PDCP window maintaining between inactive UEs and connected UEs.

Observation2: Initializing the PDCP state variables when suspending a multicast MRB is not possible according the specification.

Proposal: When upper layers request a PDCP entity suspend for the multicast MRB, no specific behavior is needed for RX_NEXT and RX_DELIV, i.e. no initialization for RX_NEXT and RX_DELIV.

The corresponding Text Proposal is given in Annex.

Reference

- 3GPP TS 38.323 V17.1.0 “Packet Data Convergence Protocol (PDCP) specification”

Annex: TP to TS 38.323

--------------Start of change--------------

### 5.1.4	PDCP entity suspend

When upper layers request a PDCP entity suspend, the transmitting PDCP entity shall:

-	set TX_NEXT to the initial value;

-	discard all stored PDCP PDUs;

When upper layers request a PDCP entity suspend, the receiving PDCP entity shall:

-	if t-Reordering is running:

-	stop and reset t-Reordering;

-	deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	set RX_NEXT and RX_DELIV to the initial value, except for the multicast MRB.

--------------End of change--------------