3GPP TSG-RAN WG2 Meeting #119 electronic					         R2-2207631

Online, August, 2022

Source: 	vivo

Title:  	Remaining issues on validity timer in NR NTN

Agenda Item:	6.10.3.2.3

Document for:	Discussion and Decision

Introduction

In RAN2#118e meeting [1], RAN2 achieved the following agreements regarding validity timer for neighbour cell:

2.  The following IEs/parameters are broadcast per neighbour cell in NTN:

Ephemeris,

DL and UL polarization,

Epoch time of assistance information

Validity duration

FFS how to handle the validity timer for neighbour cell. FFS if epoch time can be same or different. FFS about other parameters

3.  RAN2 signalling supports having the epoch time of the serving cell applicable to the neighbour cell’s assistance information (i.e. the same epoch time can be used for both cells).

The FFS on how to handle the validity timer for neighbour cell(s) (i.e. UE behaviour on how to use them) failed to be resolved in the last meeting. Current Spec [2] only has procedures for the validity timer of the serving cell. In addition, based on the summary of [offline-106] [3] in the last meeting, we understand that there was another FFS that failed to be resolved in the last meeting, regarding whether the validity duration of the serving cell can be reused as the neighbour cell’s assistance information.

In this contribution, we will discuss the remaining issues on validity timer for neighbour cell, mainly including the following two issues. Based on our proposals, a TP is provided to show what Spec changes are needed from our perspective.

- Issue 1: UE behaviour on handing neighbour cell validity timers;

- Issue 2: The configuration of neighbour cell validity timers

Discussion

UE behaviour on handing of neighbour cell validity timers

According to the current Spec [2], NTN-NeighCellConfig which contains satellite assistance information  (e.g., satellite ephemeris and common TA parameters) and validity timer information for neighbour cells has already been introduced in SIB19. But the corresponding neighbour cell validity timer related procedures are now missing in the Spec.

Observation 1: The validity timer configuration for neighbour cell(s) has already been specified in NTN-NeighCellConfig, but related UE behaviours on how to handle these timers are still missing in the Spec.

Similar to the validity timer for the serving cell, if the satellite assistance information of a neighbour cell is broadcast, UE needs to maintain the corresponding neighbour cell validity timer. In other words, upon receiving SIB19, the UE shall start or restart the validity timer for each neighbour cell with the corresponding duration ntn-UlSyncValidityDuration from the subframe indicated by the associated epochTime.

Proposal 1: Upon receiving SIB19, the UE shall start or restart the validity timer for each neighbour cell with the corresponding duration ntn-UlSyncValidityDuration from the subframe indicated by the associated epochTime.

Neighbour cell’s associated satellite assistance information assists UE in performing neighbour cell measurements. Idle/inactive UEs can autonomously adjust the SMTCs based on this assistance information; connected UE reports service link propagation delay difference between the serving cell and neighbour cell(s) based on UE location and ephemeris to assist the NW in configuring the appropriate SMTCs.

A validity duration configured by the network for satellite assistance information indicates the maximum time during which the UE can apply the satellite assistance information without having to reacquire new satellite assistance information. Invalid assistance information of neighbour cells will only affect the UE measurement, but not UL synchronisation for a UE’s actual UL transmissions. Upon any validity timer for a neighbour cell’s expiry, the UE shall re-acquire that neighbour cell’s satellite assistance information, and this applies to both connected UEs and idle/inactive UEs. As the satellite assistance information for the neighbour cells is broadcast in SIB19, upon the expiry of the validity timer for any neighbour cell, the UE shall re-acquire SIB19.

Proposal 2: When the validity timer for any neighbour cell expires, UE shall re-acquire SIB19.

According to the current Spec, UE should attempt to re-acquire SIB19 before the end of the serving cell validity timer. In our understanding, UE should also attempt to re-acquire SIB19 before the end of any neighbour cell validity timer. That is, UE should attempt re-acquire SIB19 before the end of any validity timer, including serving cell validity timer and neighbour cell validity timer(s).

Proposal 3: UE should attempt re-acquire SIB19 before the end of any validity timer, including serving cell validity timer and neighbour cell validity timer(s).

Configuration aspects of neighbour cell validity timers

In the last meeting, it was agreed that RAN2 signalling supports the epoch time of the serving cell applicable to also the neighbour cell’s assistance information (i.e. the same epoch time can be used for both serving and neighbour cells). However, despite this agreement specific to epoch time, RAN2 failed to achieve a similar agreement regarding the validity duration. Since the validity timer information for neighbour cell can be the same as or different from that of the serving cell based on RAN2#117e agreement [4], we think RAN2 signalling should also support the validity duration of the serving cell that is applicable to the neighbour cell’s assistance information. More specifically, If ntn-UlSyncValidityDuration is absent in ntn-Config provided via NTN-NeighCellConfig, the UE uses ntn-UlSyncValidityDuration from the serving cell’s satellite assistance information.

Proposal 4: If ntn-UlSyncValidityDuration is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses validity duration configured for the serving cell.

Text proposals following proposal 1 ~proposal 4 are attached in the Annex for TS 38.331. RAN2 is suggested to adopt the TP if P1~P4 above are agreeable.

Proposal 5: If P1 ~ P4 are agreed, adopt the TP in the Annex.

Conclusions

Based on the analyses given above, we have the following observations and proposals:

Observation 1: The validity timer configuration for neighbour cell(s) has already been specified in NTN-NeighCellConfig, but related UE behaviours on how to handle these timers are still missing in the Spec.

Proposal 1: Upon receiving SIB19, the UE shall start or restart the validity timer for each neighbour cell with the corresponding duration ntn-UlSyncValidityDuration from the subframe indicated by the associated epochTime.

Proposal 2: When the validity timer for any neighbour cell expires, UE shall re-acquire SIB19.

Proposal 3: UE should attempt re-acquire SIB19 before the end of any validity timer, including serving cell validity timer and neighbour cell validity timer(s).

Proposal 4: If ntn-UlSyncValidityDuration is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses validity duration configured for the serving cell.

Proposal 5: If P1 ~ P4 are agreed, adopt the TP in the Annex.

Reference

- Report of 3GPP TSG RAN WG2 meeting #118e, Online

- TS 38.331 Radio Resource Control (RRC) protocol specification, V17.1.0

- R2-2206210, Report from [AT118-e][106][NTN] CP issues – second round (Nokia)

- Report of 3GPP TSG RAN WG2 meeting #117e, Online

- Annex—TP for TS 38.331

TEXT PROPOSAL for PROCEDURE

##### 5.2.2.4.21	Actions upon reception of SIB19

Upon receiving SIB19, the UE shall:

1>	start or restart T430 for serving cell with the corresponding duration ntn-UlSyncValidityDuration from the subframe indicated by epochTime;

1> if a neighbour cell’s satellite assistance information is provided:

2> start or restart T430 for the neighbour cell with the corresponding duration ntn-UlSyncValidityDuration from the subframe indicated by epochTime

NOTE:	UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime for any cell (including serving cell and neighbour cell(s)) by UE implementation.

#### 5.2.2.6	  T430 expiry

The UE shall:

1>	if T430 for serving cell expiry and if in RRC_CONNECTED:

2>	inform lower layers that UL synchronisation is lost;

2>	acquire SIB19 as defined in clause 5.2.2.3.2;

2>	upon successful acquisition of SIB19:

3>	inform lower layers that UL synchronisation is obtained;

1>	 if T430 for any neighbour cell expiry:

2>	acquire SIB19 as defined in clause 5.2.2.3.2.

#### –	SIB19

SIB19 contains satellite assistance information.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch. The validity duration, ntn-UlSyncValidityDuration of serving cell’s satellite assistance information, is mandatory present.

ntn-NeighCellConfigList Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId.

referenceLocation Reference location of the serving cell provided via NTN quasi-Earth fixed system and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]..

t-Service Indicates the time information on when a cell provided via NTN quasi-Earth fixed system is going to stop serving the area it is currently covering. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). This field is excluded when determining changes in system information, i.e. changes of t-Service should neither result in system information change notifications nor in a modification of valueTag in SIB1. The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field.

#### –	NTN-Config

The IE NTN-Config provides parameters needed for the UE to access NR via NTN access.

NTN-Config information element

-- ASN1START

-- TAG-NTN-CONFIG-START

NTN-Config-r17 ::=             SEQUENCE {

epochTime-r17                  EpochTime-r17                                                            OPTIONAL,  -- Need R

ntn-UlSyncValidityDuration-r17 ENUMERATED{ s5, s10, s15, s20, s25, s30, s35,

s40, s45, s50, s55, s60, s120, s180, s240, s900}              OPTIONAL,  -- Need R

cellSpecificKoffset-r17        INTEGER(1..1023)                                                         OPTIONAL,  -- Need R

kmac-r17                       INTEGER(1..512)                                                          OPTIONAL,  -- Need R

ta-Info-r17                    TA-Info-r17                                                              OPTIONAL,  -- Need R

ntn-PolarizationDL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need R

ntn-PolarizationUL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need R

ephemerisInfo-r17              EphemerisInfo-r17                                                        OPTIONAL,  -- Need R

ta-Report-r17                  ENUMERATED {enabled}                                                     OPTIONAL,  -- Need R

...

}

EpochTime-r17 ::=              SEQUENCE {

sfn-r17                        INTEGER(0..1023),

subFrameNR-r17                 INTEGER(0..9)

}

TA-Info-r17 ::=                 SEQUENCE  {

ta-Common-r17                  INTEGER(0..66485757),

ta-CommonDrift-r17             INTEGER(-257303..257303)                                                 OPTIONAL,  -- Need R

ta-CommonDriftVariant-r17      INTEGER(0..28949)                                                        OPTIONAL   -- Need R

}

-- TAG-NTN-CONFIG-STOP

-- ASN1STOP

NTN-Config field descriptions

EphemerisInfo This field provides satellite ephemeris either in format of position and velocity state vector or in format of orbital parameters. This field is excluded when determining changes in system information, i.e. changes of XXX should neither result in system information change notifications nor in a modification of valueTag in SIB1.

epochTime Indicate the epoch time for assistance information (i.e. Serving satellite ephemeris in IE ephemerisInfo and Common TA parameters). When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. If this field is absent, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses epoch time from the serving satellite ephemeris. In case of handover, this field is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the target cell. This field is excluded when determining changes in system information, i.e. changes of epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.

cellSpecificKoffset The CellSpecific_K_offset is a scheduling offset used for the timing relationships that need to be modified for NTN [see TS 38.2xy]. The unit of K_offset is number of slots for a given subcarrier spacing of 15 kHz. If the field is absent UE assumes value 0.

kmac K_mac is a scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. It is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH [see TS 38.2xy]. If the field is absent UE assumes value 0. For the reference subcarrier spacing value for the unit of K_mac in FR1, a value of 15 kHz is used. The unit of K_mac is number of slots for a given subcarrier spacing.

ntn-PolarizationDL If present, this parameter indicates polarization information for Downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL If present, this parameter indicates Polarization information for Uplink service link. If not present and ntnPolarizationDL is present, UE assumes a same polarization for UL and DL.

ntn-UlSyncValidityDuration A validity duration configured by the network for uplink synchronization assistance information (i.e. Serving satellite ephemeris and Common TA parameters) which indicates the maximum time during which the UE can apply assistance information without having acquired new assistance information. The unit of ntn-UlSyncValidityDuration is second. This parameter applies to both connected and idle mode UEs. If this field is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses validity duration from the serving satellite assistance information. This field is excluded when determining changes in system information, i.e. changes of ntn-UlSyncValidityDuration should neither result in system information change notifications nor in a modification of valueTag in SIB1. ntn-UlSyncValidityDuration is only updated when at least one of epochTime, ta-Info, ephemerisInfo is updated.

ta-Common ta-Common is a network-controlled common timing advanced value and it may include any timing offset considered necessary by the network. ta-Common with value of 0 is supported. The granularity of ta-Common is 4.072 × 10^(-3) μs. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-Common should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-CommonDrift Indicate drift rate of the common TA. The granularity of TACommonDrift is 0.2 × 10^(-3) μs⁄s Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-CommonDrift should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-CommonDriftVariant Indicate drift rate variation of the common TA. The granularity of TACommonDriftVariation is 0.2×10^(-4) μs⁄s^2. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-CommonDriftVariant should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-Report When this field is included in SIB19, it indicates TA reporting is enabled during Random Access due to RRC connection establishment, RRC connection reestablishment and RRC connection resume. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during Random Access due to reconfiguration with sync (see TS 38.321 [3], clause x.x.x).

TEXT PROPOSAL END