3GPP TSG-RAN WG2 Meeting #119 electronic					         R2-2207635

Online, August, 2022

Source: 	vivo

Title:  	Discussion on mobility and service continuity enhancement

Agenda Item:	8.7.4

Document for:	Discussion and Decision

Introduction

According to the WID approved in RAN#96 [1], one of the objectives is to specify NTN-TN and NTN-NTN mobility and service continuity enhancements as follows:

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4]

  - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

  - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

  - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

  - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

In this contribution, we will provide our initial analyses on this topic, regarding both the enhancements to IDLE/INACTIVE mobility and enhancements to CONNECTED mobility.

Discussion

Idle/Inactive mobility enhancements

According to the objectives cited above, one objective is to enhance the time-based and location-based cell reselection operation for the earth-moving cell case. Earth-moving cell’s coverage keeps moving over the earth surface over time. In order to adopt Rel-17 distance-based measurement mechanism accordingly to the earth-moving cell, a UE should determine its distance from a time-variant reference location of an earth-moving cell at each moment. This is different from what was discussed in Rel-17 quasi-earth fixed cell case where a quasi-fixed reference location is provided without such time-variance attribute. Therefore, on top of Rel-17 location-based cell reselection mechanism, how the UE acquires/determines time-variant reference location of the earth-moving cell needs to be resolved in Rel-18.

From our perspective, there could be several ways to address this issue. For example, one possible solution is that a rule on how the reference location changes over time (e.g. the reference location as a function of time) could be configured to the UE by the NW, so that the UE can calculate real-time reference location based on the rule and the current time. As another example, the UE may be configured by the NW with the reference location which needs to be updated/re-acquired after a validity time (e.g. after validity timer expiry, or after a period of time configured by the NW). RAN2 needs to discuss the solution to be adopted for the UE to determine the time-variant reference location for the location-based cell reselection enhancement.

Proposal 1: RAN2 discusses how the UE determines time-variant reference location of the earth-moving cell (e.g. UE calculation based on a rule on how reference location changes over time, acquiring/re-acquiring the reference location based on a validity time, etc.).

Once the UE determines the reference location in the earth-moving cell, we think the location-based measurement mechanism for cell reselection as specified in Rel-17 NTN for quasi-earth fixed cell case can be reused in the earth-moving cell case as well. That is, if the distance from the serving cell’s reference location (which is determined by the UE taking into account the time-variance attribute) is larger than a threshold configured, the UE should start measurement for cell reselection.

Proposal 2: For the earth-moving cell case, the UE should start measurement for cell reselection, if the distance from the serving cell’s reference location determined/acquired by the UE is larger than a threshold configured.

With respect to the time-based cell reselection, the stop time of the quasi-fixed cell is common for all UEs camping in the cell regardless of the reason for stopping serving the area (i.e., due to service-link switch or feeder-link switch). So, broadcasting one single stop time (i.e., t-Service) to reflect the loss of coverage of the serving cell in both cases is sufficient in the quasi-fixed cell case in Rel-17.

However, in the earth-moving cell case, although the stop time in a cell due to feeder-link switch is still the same for all UEs distributed in the cell as in the quasi-earth fixed cell case, UEs distributed in different places in the cell may face different stop time due to service-link switch. Therefore, the stop time due to service-link switch and that due to feeder-link switch may need different handling. Specifically, considering its similarity to the stop time introduced for the quasi-fixed earth cell (i.e. common to all UEs), handling of the stop time caused by the feeder-link switch in earth moving cell may follow a similar mechanism specified in Rel-17, i.e. a configured t-Service with related procedure in TS 38.304. By contrast, for the stop time due to service-link switch, the UE may need to derive the stop time based on the time-variant reference point of the serving cell and its current location, and carries out measurements for cell reselection when it determines that the stop time of the serving cell is getting close. Also, from a system-level perspective, some UEs in an earth moving-cell (e.g. those with high mobility) may start measurements due to the stop time caused by service-link switch, whereas some other UEs (e.g. those staying still) at the same time may initiate the measurements due to the stop time caused by feeder-link switch. This also justifies the need of differentiated handling of these two types of stop time.

To this end, we propose that the stop time due to both feeder-link switch and service-link switch should be considered for the time-based cell reselection enhancement, and two types of stop time should be supported in the earth-moving cell case. Specifically, the UE shall start neighbour measurements before either type of the stop time is reached (e.g. though the UE may not necessarily be made aware of whether a stop time is caused by service-link switch or feeder-link switch).

Proposal 3: For the time-based cell reselection enhancement in the earth moving cell case, two types of stop time (which intends to handle service-link switch and feeder-link switch respectively) should be supported.

Proposal 4: The UE should start neighbour cell measurements before either type of the stop time is reached.

Proposal 5: Similar to the t-Service, the gNB configures a serving cell stop time that covers the stop time due to feeder-link switch in the earth-moving cell case. RAN2 further discusses how the UE determines the stop time due to service-link switch.

Another objective for the IDLE/INACTIVE mobility enhancement is power saving for the NTN-TN mobility. The NTN cell can be much larger than the TN cell, with the radius of NTN cell able to be up to thousands of kilometres. If there are both TN and NTN deployments in a certain area, NTN cells will broadcast information about TN neighbour cells, which applies to all UEs camping in the NTN cell. But for a UE which is at a position with only NTN coverage, performing measurements on TN-only frequencies will result in meaningless power consumption, since UEs in such a position will not have a chance to reselect to the TN cell. In order to reduce UE power consumption, we propose that when the UE is not in an area within/adjacent to TN coverage, the UE does not perform measurement on the corresponding TN frequency.

Proposal 6: A UE with an NTN serving cell does not measure the TN-only frequency when it is not within/adjacent to the area with TN coverage.

Connected mobility enhancements

The objective of connected mode mobility is to reduce the signalling overhead for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell. We think that there could be the following two directions that can be considered for RAN2 discussion.

One direction is to save the L3 mobility signalling due to the service-link switch in NTN with quasi-fixed cells. Specifically, there was the agreement in Rel-17 that such service-link switch implies PCI change, which aims to make this scenario directly covered by L3 mobility in Rel-17. From our perspective, since the typical case for the service-link switch is that only the satellite that provides the coverage to the area is changed but the gNB that the satellites are connected with remains the same, changing the cell and initiating L3 mobility to a UE within the area is not actually necessary. Instead. such service-link switch for the quasi-earth fixed cells could be considered as changing of coverage provided by different satellites, which could be modelled as changing of beams/BWPs rather than L3 handover (see below figure). In this way, a UE may only need to perform DL/UL sync. to the new satellite covering this area and thus is not necessarily reconfigured for most of the parameters. Therefore, signalling related to reconfigurations for these unchanged parameters during L3 mobility procedures can be saved.

Figure.1 a) Rel-17 agreement; b) Change of Beam/BWP.

Proposal 7: RAN2 discusses how to support service-link switch without initiating L3 mobility (e.g. by BWP/beam switch) for the quasi-earth fixed cell case.

Another direction is to provide the common part of the handover signalling via broadcast or multi-cast manner. It can be observed that LEO satellites are typically featured as high-speed motion, the relative speed of the satellite to the earth can be as high as 7.56 km per second. Almost all UEs covered by the NTN cell need to perform handover frequently in very short period, regardless of whether the UE is moving or stationary. In addition, the coverage area of NTN cell service is much larger than that of TN cells, the number of UEs served will also be large. If the existing handover signalling (i.e., dedicated signalling from the NW to UE) is adopted, such frequent handover would result in significant signalling overhead.

So, we think one effective method to reduce the overall signalling overhead for a large number of UE due to frequent handover is to send the common part of the HO command via broadcast signalling. As an example, considering feeder-link switch occurs in quasi-fixed cell and earth-moving cell, all UEs in the cell need to perform handover to another cell almost simultaneously. Usually, the target cell of most UEs may be the same in such scenario, and the target cell-related configuration (e.g., spCellConfigCommon in ReconfigurationWithSync) is the same for these UEs at least. In addition, since CHO is adopted in NTN, the CHO-related configuration (e.g., trigger events and candidate cells) can also be sent via broadcast manner. On the other hand, the handover configurations of UEs in different locations in an NTN cell (e.g., cell center and cell edge) may be different, so providing configuration via multicast (e.g., via P-RNTI) can be also considered.

To sum up, we have the following proposal for the connected mode mobility enhancement:

Proposal 8: The common part of the HO command can be sent via broadcast or multicast manner.

Conclusions

Based on the analyses given above, we have the following proposals:

Proposal 1: RAN2 discusses how the UE determines time-variant reference location of the earth-moving cell (e.g. UE calculation based on a rule on how reference location changes over time, acquiring/re-acquiring the reference location based on a validity time, etc.).

Proposal 2: For the earth-moving cell case, the UE should start measurement for cell reselection, if the distance from the serving cell’s reference location determined/acquired by the UE is larger than a threshold configured.

Proposal 3: For the time-based cell reselection enhancement in the earth moving cell case, two types of stop time (which intends to handle service-link switch and feeder-link switch respectively) should be supported.

Proposal 4: The UE should start neighbour cell measurements before either type of the stop time is reached.

Proposal 5: Similar to the t-Service, the gNB configures a serving cell stop time that covers the stop time due to feeder-link switch in the earth-moving cell case. RAN2 further discusses how the UE determines the stop time due to service-link switch.

Proposal 6: A UE with an NTN serving cell does not measure the TN-only frequency when it is not within/adjacent to the area with TN coverage.

Proposal 7: RAN2 discusses how to support service-link switch without initiating L3 mobility (e.g. by BWP/beam switch) for the quasi-earth fixed cell case.

Proposal 8: The common part of the HO command can be sent via broadcast or multicast manner.

Reference

- RP-221819, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements