3GPP TSG-RAN WG2 Meeting #119-e                      R2-2207647

Electronic, Aug 17th – Aug 26th, 2022

Agenda Item:	8.6.2

Source: 	Transsion Holdings

Title:            Discussion on disabling of HARQ feedback for IoT-NTN

Document for:	Discussion and Decision

- Introduction

At the RAN#95e and RAN#96e meetings, a new Work Item was approved for IoT NTN. The WID objectives concerning RAN2 are listed in the table below:

- Performance Enhancements: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Mobility Enhancements: - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Discontinuous coverage: - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

In this contribution, we provide our views on disabling of HARQ feedback to proceed the work item.

- Discussion

- 2.1 Performance Enhancements Enhancements

- In last RAN1#109e meeting, there have been some agreements listed below on disabling of HARQ feedback:

Agreement For IoT NTN, to configure/indicate enabling/disabling on HARQ feedback for downlink transmission, one or more of the following options can be considered: - Option 1: per HARQ process via UE specific RRC signaling - Option 2: per HARQ process via SIB signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly determined by existing configured/indicated parameter(s) (e.g., repetition number, TBS) - Option 5: per HARQ process via MAC CE - Other options or combinations are not excluded Note: Option(s) for eMTC and NBIoT can be separately discussed. Agreement For IoT NTN, further study the potential issues due to enabling/disabling on HARQ feedback for downlink transmission - Issue A: SPS PDSCH - Issue B: (N)PDSCH/(N)PDCCH scheduling restriction - Issue C: HARQ feedback for scheduling multiple TB - Issue D: HARQ bundling for eMTC HD-FDD - Issue F: NPRACH capacity - Issue G: Serving cell/satellite change during data transfer (FFS: for eMTC and/or NB-IoT) - Other issues are not excluded Note: The “Issues” in common for eMTC and NB-IoT can be separately discussed.

It has proposed many options in RAN1 on how to disable HARQ feedback for downlink transmission. This topic has already been discussed in R17 NR-NTN and the option 1 i.e. per HARQ process via UE specific RRC signaling has been accepted by NR-NTN. According to our understanding, the HARQ process of eMTC is more like NR, which supports at most 14 HARQ processes in FDD, so the agreed solution for NR-NTN can be considered as baseline for eMTC. In this way the data such as RRC signaling or MAC CE command that need high reliability can use the enabled HARQ process while the data such as service traffic that need high data rate can use the disabled HARQ process.

However for NB-IoT, to reduce cost and complexity it supports at most 2 HARQ processes(only when the UE supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter npdsch-MultiTB-Config). So enabling/disabling HARQ feedback per HARQ process may not be suitable for NB-IoT as there maybe only 1 HARQ process. Disabling HARQ feedback will prevent the eNB from knowing if some important signing has been received by the UE, which will cause state mismatch between UE and NW, so other options may be need to be investigated for NB-IoT with only 1 HARQ process. Repetition transmission has been introduced for NB-IoT to enhance coverage. As repetition is some kind of downlink retransmission and UE ensures the reliability of downlink transmission by merging all repetition demodulations. In the same channel condition, the higher repetition is, the better the reliability is. Therefore, NW can decide whether to disable HARQ feedback based on the downlink scheduled repetitions. On the other hand, the HARQ stalling in NTN is caused by the long propagation time between the UE and the eNB. The long propagation time of UE is reflected in the parameter Koffset which is UE specific and adjusted by NW, if the propagation time is not long then the HARQ stalling may not exists, so the Koffset can be a factor to decide whether enable/disable HARQ feedback.

- Re-use the solutions introduced in Rel-17 NR NTN, i.e. per HARQ process via UE specific RRC signaling on HARQ feedback enabling/disabling for downlink transmission for eMTC as a baseline, minimum necessary changes to adapt them to eMTC.

- Enabling/disabling HARQ feedback per HARQ process may not suitable for NB-IoT, RAN2 need to study other options e.g. based on downlink repetition or Koffset.

- 3 Conclusions

In this contribution we discussed issues related to disabling of HARQ feedback and made the following proposals:

- Re-use the solutions introduced in Rel-17 NR NTN, i.e. per HARQ process via UE specific RRC signaling on HARQ feedback enabling/disabling for downlink transmission for eMTC as a baseline, minimum necessary changes to adapt them to eMTC.

- Enabling/disabling HARQ feedback per HARQ process may not suitable for NB-IoT, RAN2 need to study other options e.g. based on downlink repetition or Koffset.

- 4 References

- “3GPP TR 38.821 V16.1.0”

- “Draft_Minutes_report_RAN1#109-e_v030.docx”

- “RP-221806 Revised WID on IoT NTN enhancements”

- “3GPP TS 38.300 V17.1.0”