3GPP TSG-RAN WG2 Meeting #119-e                      R2-2207649

Electronic, Aug 17th – Aug 26th, 2022

Agenda Item:	8.6.4

Source: 	Transsion Holdings

Title:  	Discussion on enhancement to discontinuous coverage for IoT NTN

Document for:	Discussion and Decision

- Introduction

At the RAN#95e and RAN#96e meetings, a new Work Item was approved for IoT NTN. The WID objectives concerning RAN2 are listed in the table below:

- Performance Enhancements: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Mobility Enhancements: - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Discontinuous coverage: - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

In this contribution, we provide our views on enhancement to discontinuous coverage to proceed the work item.

- Discussion

- 2.1 Further enhancement to discontinuous coverage

- In R17 IoT-NTN, in order to enable the UE to save power during discontinuous coverage periods in RRC_IDLE, the network provides satellite assistance information (e.g. satellite ephemeris parameters, the start-time of upcoming satellite's coverage) to enable the UE to predict when coverage will be provided by the upcoming satellites.

Since NTN cells provide much bigger coverage then TN cells, the number of UEs served by a NTN cell is much larger than that of TN cell. For the (quasi-)earth fixed cells, as the network will broadcast the start-time of the upcoming satellite’s coverage, there will be a large number of UEs trying to access the network at the start-time of the upcoming satellite’s coverage which will result in a serious RACH congestion. In current specification, the network can provide a backoff indicator at RAR message to instruct the UEs to delay a random period to resend the preamble when RACH congestion happens. However this may cause another issue as IoT-NTN UEs need to acquire the SIB31(-NB) to get uplink synchronization before accessing the network and the uplink sync duration is limited by the ul-SyncValidityDuration. If it expires, the UE needs to reacquire SIB31(-NB). The specification has specified that IoT UEs are not required to acquire system information in connected state, which means reacquire SIB31(-NB) in connected will cause service interruption as the IoT-NTN UEs need to leave connected mode to acquire SIB31(-NB) and get uplink re-sync. If the UE spends too much time on RACH during RACH congestion, then there will be a limited time for data service as the ul-SyncValidityDuration may expire in connected state. So it needs to avoid RACH congestion when the next satellite starts serving the area rather than relieve congestion when RACH congestion happens. For example the current serving satellite can instruct the UE to delay a period to access the upcoming satellite, the delay period can be generated base on traffic load or just a random value.

- RAN2 needs to consider the RACH congestion issue as a large number of UEs may try to access the NTN cell at the next satellite’s service start time in discontinuous coverage scenario.

- When out of coverage, the UE is not required to perform AS functions. That means that the UEs will not monitor paging when in its predicting discontinuous coverage period. However the core network will initiate the paging procedure for EPS services when needed, there is an chance that the paging procedure will initiated during the UE’s predicting discontinuous coverage period which will result in paging failure. So the CN may need to avoid paging UE when in UE’s predicting discontinuous coverage period. As this is more core network’s work and SA2 are studying this issue, we can wait for the conclusion from SA2.

- Wait for SA2’s conclusion on the paging issues under discontinuous coverage.

- 3 Conclusions

In this contribution we discussed issues related to discontinuous coverage and made the following proposals:

- RAN2 needs to consider the RACH congestion issue as a large number of UEs may try to access the NTN cell at the next satellite’s service start time.

- Wait for SA2’s conclusion on the paging issues under discontinuous coverage.

- 4 References

- “3GPP TR 38.821 V16.1.0”

- “Draft_Minutes_report_RAN1#109-e_v030.docx”

- “RP-221806 Revised WID on IoT NTN enhancements”

- “3GPP TS 38.300 V17.1.0”