3GPP TSG-RAN WG2 Meeting #119 electronic	                     R2-2207657

Online, Aug, 2022

Agenda Item:	8.4.2.2

Source: 	OPPO

Title:  	Initial Considerations on L1/L2 mobility

Document for:	Discussion and Decision

Introduction

In RAN#94e, a new work item on further NR mobility enhancements was approved [1]. One of the objectives is to study the mechanism and procedures of L1/L2 based inter-cell mobility for latency reduction. The objective in WID is shown below.

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

  - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- Timing Advance management [RAN1, RAN2]

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 2: FR2 specific enhancements are not precluded, if any.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

In this contribution, we would like to provide our considerations on L1/L2 mobility from the following aspects:

- Scenarios of L1/L2 mobility.

- Candidate configurations for L1/L2 mobility

- L1/L2 signaling to support dynamic switching

Discussion

Scenarios

Figure 1: Mobility in intra-CU scenario

Regarding the deployment scenarios to be discussed for L1/L2 mobility, the following note is included in WID.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

Multiple deployments are in scope as depicted in note. Before getting into details, it is essential to firstly identify if there is any scenario-specific issue, for which extra discussions/solutions are needed. The conclusions can help us to determine whether a unified solution is enough or some scenarios can be prioritized when designing the procedure of L1/L2 mobility in our future work.

Inter-frequency and intra-frequency

In legacy handovers, target cell and source cell can be either inter-frequency or intra-frequency. To make HO decision, cell qualities of serving cell and neighbour cells are evaluated based on L3 measurement results. For inter-frequency neighbour cells, measurement gaps shall be configured for UE to perform measurements given that UE may not able to support more than one RF chains to operate on different frequencies simultaneously. During a measurement gap, UE performs inter-frequency measurements and stops UL/DL transmissions with serving cell.

In L1 measurement framework, SSBs/CSI-RSs of serving cell are measured and reported for beam management. In R17 ICBM (Inter-Cell Beam Management), L1-RSRP measurements on SSBs of PCIs different from serving cell is supported. The SSBs of different PCIs shall be same frequency as serving cell, thus no MG is needed.

In summary, current L3 measurement configuration is complete to cover inter-frequency measurement on neighbour cells while L1 measurement mechanism cannot. Therefore, if L1 measurement is preferred for L1/L2 mobility, further enhancements are needed such that inter-frequency operations can be feasible.

Inter-frequency measurements on neighbour cells have already been supported in L3 measurement configuration. While for L1 measurement which is configured for CSI report, only intra-frequency case is enabled.

Inter-DU and intra-DU

As for intra-CU inter-DU and inter-DU, it regards to the architecture of network as shown in the figure below. With different deployments, operations to support L1/L2 mobility are not exactly same at both UE side and network side.

Figure 2: Overall architecture for separation of gNB-CU-CP and gNB-CU-UP

Compared to intra-DU case, inter-DU case will introduce much more impacts in RAN3, which may relate to F1/E1 interface. The impacts may include, e.g., PCell change procedure, UP tunnel switch, data forwarding, neighbouring cell detection and configuration, etc.

For the cells managed by the same DU, RLC and MAC configuration can be reused, accordingly, UE may only need to update L1 configurations for L1/L2 mobility. By contrast, the procedures such as MAC reset, RLC re-establishment and PDCP data recovery may be required by UE for inter-DU case which is not beneficial to latency reduction.

For intra-CU inter-DU and intra-DU scenario, the operations are different on both UE side and network side. Separate discussions are needed for two cases in both RAN2 and RAN3.

As can be observed, more standardization works would be involved for intra-CU inter-DU and inter-frequency scenarios, hence it is suggested to prioritize intra-DU and intra-frequency case.

Intra-DU and intra-frequency scenarios can be prioritized in the study of L1/L2 mobility.

Serving cell change within one CG

A cell group consists of one SpCell and at least one SCell. For dual-connectivity, UE can be served by two cell groups, i.e., MCG and SCG. In legacy HO/CHO, UE can perform MCG with or without SCG change based on RRC signalling, L1/L2 mobility aims to achieve lower latency, simultaneous change of MCG and SCG would introduce extra complexity to either configurations or operations. Therefore, serving cell change within one CG is supported as specified in scenario description, i.e., serving cell change within MCG or SCG.

For L1/L2 mobility, serving cell change within one CG is studied, i.e., serving cell change within MCG or SCG.

For serving cell change within on CG, source cell can indicate UE to perform PCell/PSCell change with or without SCell change during handover. Other than handover case, NW is also flexible to indicate UE to add/modify/release a SCell.

Same scenarios can be applied for L1/L2 mobility since either PCell/PSCell or SCell may be not qualified any more due to UE mobility. Therefore, the following cases shall be supported for serving cell change within one CG:

- PCell change w/ or w/o SCell change.

- PSCell change w/ or w/o SCell change

- SCell change

Serving cell change in L1/L2 mobility shall support the following cases:

- PCell change with or without SCell change.

- PSCell change with or without SCell change.

- SCell change.

Configurations for candidates

For L1/L2 mobility, multiple candidates can be pre-configured via RRC signalling for fast serving cell switching. In this paper, we would like to further discuss how to provide these configurations to UE.

As specified in TS 38.331, RRCReconfiguration message is used to provide essential parameters needed for serving cell change, including security information, cell group configurations, etc. Since only intra-CU scenario (security key refresh is not required) will be studied for L1/L2 mobility, configurations for each candidate are mainly focused on DU-related information, e.g., what in cell group configurations can be taken as reference. Cell group information is configured through CellGroupConfig IE and the details are copied as below.

Therein, the yellow highlighted IE is applied to provide SpCell configuration and the blue highlighted ones are used for SCell operations such as add/modification/release. Network can indicate UE to perform different type of serving cell change by comprising either or both IEs. By taking legacy RRC signalling as baseline, candidates for L1/L2 mobility can be configured as the following ways:

- Alt1: Each candidate is configured as a single cell, i.e., like SpCellConfig or SCellConfig.

- Alt2: Each candidate is configured as a cell group, i.e., like CellGroupConfig.

To reduce signalling overhead, the way of delta configuration can be considered, e.g., Each candidate is associated with a delta configuration compared to a reference. Further discussions for this can be triggered in stage3 work.

For L1/L2 mobility, each candidate can be configured as a Cell group or a single serving cell.

L1/L2 signalling for dynamic switching

According to WID, dynamic switch mechanism based on L1/L2 signalling will be studied. In this section, we would like to provide our considerations on information that should be included in L1/L2 signalling.

Most basically, L1/L2 signalling shall be able to indicate which candidate to switch considering that multiple candidates are configurable. On the other hand, UE can be configured to perform L1 measurements and reporting for each candidate. TCI state associated with SSB/CSI-RS of candidate can be also included in L1/L2 signalling to provide initial beam information for reception and receiving in target serving cell.

Based on the discussions above, at least the following information can be indicated by L1/L2 signalling:

- Candidate information

- TCI state

For design of L1/L2 signalling, at least the following information shall be embodied:

  - Candidate information

  - TCI state

Conclusion

Based on the discussion in section 2 we have following observations:

Observation 1	Inter-frequency measurements on neighbour cells have already been supported in L3 measurement configuration. While for L1 measurement which is configured for CSI report, only intra-frequency case is enabled.

Observation 2	For intra-CU inter-DU and intra-DU scenario, the operations are different on both UE side and network side. Separate discussions are needed for two cases in both RAN2 and RAN3.

Observation 3	For L1/L2 mobility, serving cell change within one CG is studied, i.e., serving cell change within MCG or SCG.

Based on the discussion above, we have the following proposals:

Proposal 1	Intra-DU and intra-frequency scenarios can be prioritized in the study of L1/L2 mobility.

Proposal 2	Serving cell change in L1/L2 mobility shall support the following cases:

	PCell change with or without SCell change.

	PSCell change with or without SCell change.

	SCell change.

Proposal 3	For L1/L2 mobility, each candidate can be configured as a Cell group or a single serving cell.

Proposal 4	For design of L1/L2 signalling, at least the following information shall be embodied:

a.	Candidate information

b.	TCI state

Reference

- RP-221799  Revised WID on Further NR mobility enhancements    MediaTek Inc