3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2207700

Online, 17th August – 26th August, 2022

Agenda item:		8.9.3

Source:		Lenovo

Title:		Discussion on Service continuity in U2N relay case

Document for:		Discussion and Decision

- Introduction

In RAN #94-e meeting, 3GPP agreed a new work item “NR sidelink relay enhancements” [1] and the WID was revised [2] in RAN #95-e meeting. One of objectives is to Specify mechanisms to enhance service continuity for single-hop L2 U2N relay:

Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

  - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

  - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

  - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

  - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

In this contribution, we discuss potential procedures for service continuity in L2 U2N relay case.

- Discussion

Figure 1: L2 U2N Remote UE switching from indirect to direct Uu cell

The procedure for L2 U2N Remote UE switching from indirect to direct Uu cell is specified in stage 2 specification see Figure1. The procedure in Figure1 can be reused for the Uu procedure for Inter-gNB indirect-to-direct path switching. The procedure for L2 U2N Remote UE switching from direct to indirect path is specified as well, which can be reused for Uu procedure for Inter-gNB direct-to-indirect path switching.

Regarding Xn interface, CP message e.g handover request and handover request acknowledge can be reused for Inter-gNB case. In addition, SN STATUS TRANSFER message associated with data forwarding also can be reused.

Proposal 1: Rel-17 procedure for path switching from indirect to direct path can be considered as baseline for Uu interface for Inter-gNB indirect-to-direct path switching.

Proposal 2: Rel-17 procedure for path switching from direct to indirect path can be considered as baseline for Uu interface for Inter-gNB direct-to-indirect path switching.

Proposal 3: Legacy Xn message e.g., handover request, handover request acknowledge, and SN STATUS TRANSFER can be reused for Inter-gNB case.

In Rel-17 Intra-gNB handover of remote UE, target relay UE with RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED can be supported. In Inter-gNB case, the remote UE can report the measurement results of candidate relay UE at the state of RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED. Source gNB can select one target relay based on the measurement report as legacy and transmit the handover request to target gNB. Once remote UE receives handover command including one target relay UE ID and neighbor cell ID, the remote UE performs Inter-gNB handover and establish PC5 link with the target relay UE.

Proposal 4: Relay UE with RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED can be selected as target relay UE as Rel-17.

In legacy handover for normal UE with direct path, only one Cell ID is added in the handover request. In Inter-gNB handover of remote UE, one cell ID is also needed. However, we need to discuss whether multiple candidate relay UEs are allowed to be added in handover request or not. The several options are listed as follows.

- Option 1: Cell ID is added. No candidate relay UE is selected by source gNB. Measurement result associated with candidate relay UE can be included.

  - In option 1, target cell is responsible for target relay UE selection since target cell is aware of the candidate relay UE e.g loading of candidate relay UE. Therefore, target cell can select one or more than one target relay UE.

- Option 2: Cell ID and only one candidate relay UE belonging this cell is allowed to be added in the handover request.

- Option 3: Cell ID and multiple candidate relay UEs belonging to this cell are allowed to be added in the handover request.

  - Regarding option 3, target gNB can select one target relay UE to be included in handover request acknowledge. It is also possible to select more than one candidate relay UE, which could be helpful for reliability. For example, when the remote UE establishes PC5 to the first candidate relay UE with idle state, the relay UE reselects to another cell. In legacy, it will result in re-establishment. If multiple relay UEs are included in handover command. The remote UE can access gNB via the second candidate relay UE, which can avoid re-establishment.

Proposal 5: RAN2 is suggested to discuss the number of candidate relay UE included in handover request by source gNB.

- Option 1: Cell ID is added. No candidate relay UE is selected by source gNB. Measurement result associated with candidate relay UE can be included. Target cell is responsible for target relay UE selection.

- Option 2: Cell ID and only one candidate relay UE belonging this cell is allowed to be added in the handover request.

- Option 3: Cell ID and multiple candidate relay UEs belonging to this cell are allowed to be added in the handover request.

Proposal 6: RAN2 is suggested to discuss whether multiple candidate relay UEs can be included in handover request acknowledge by target gNB.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: Rel-17 procedure for path switching from indirect to direct path can be considered as baseline for Uu interface for Inter-gNB indirect-to-direct path switching.

Proposal 2: Rel-17 procedure for path switching from direct to indirect path can be considered as baseline for Uu interface for Inter-gNB direct-to-indirect path switching.

Proposal 3: Legacy Xn message e.g., handover request, handover request acknowledge, and SN STATUS TRANSFER can be reused for Inter-gNB case.

Proposal 4: Relay UE with RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED can be selected as target relay UE as Rel-17.

Proposal 5: RAN2 is suggested to discuss the number of candidate relay UE included in handover request by source gNB.

- Option 1: Cell ID is added. No candidate relay UE is selected by source gNB. Measurement result associated with candidate relay UE can be included. Target cell is responsible for target relay UE selection.

- Option 2: Cell ID and only one candidate relay UE belonging this cell is allowed to be added in the handover request.

- Option 3: Cell ID and multiple candidate relay UEs belonging to this cell are allowed to be added in the handover request.

Proposal 6: RAN2 is suggested to discuss whether multiple candidate relay UEs can be included in handover request acknowledge by target gNB.

References

RP-213585 New WID on NR sidelink relay enhancements.

RP-221010 Revised WID on NR sidelink relay enhancements.