3GPP TSG-RAN WG2 #119e	Tdoc R2-2207725

Electronical meeting, 2022-08-17 -- 2022-08-29

Agenda Item:	8.14.2

Source:	Ericsson

Title:	Discussion on QoE for MBS

Document for:	Discussion

# 1	Introduction

Multicast and Broadcast Service (MBS) was specified in LTE and NR, in [1], and Quality of Experience (QoE) was specified for Virtual Reality (VR), Multimedia services (MTSI) and Dynamic Adaptive Streaming over HTTP (DASH) in [2]. The WID [3] for Release 18, entitled “Enhancement on NR QoE management and optimizations for diverse services”, aims at introducing enhanced QoE support for diverse services, such as MBS. This contribution addresses the following objective defined in [3]: “Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]”.

# 2	Discussion

## 2.1	On defining MBS with respect to QoE

Multicast and Broadcast Service (MBS) is a point-to-multipoint service in which data is transmitted from a single source entity to multiple recipients. In TS 38.300 [2], MBS is defined as a service, enabling delivery of the same service and the same content data simultaneously to a dedicated set of UEs or to all UEs in a geographical area, for the Multicast service or for the Broadcast service, respectively.

An overview of the delivery methods for MBS traffic is presented in Figure 1.

Figure 1: An overview of traffic delivery methods for MBS

In general, MBS is a communication service for delivering some content to a group of users, and it can be used to deliver content pertaining to more than one service type. Since MBS is intended for downlink traffic, conversational services which imply both downlink and uplink traffic (e.g., MTSI) cannot be delivered as MBS traffic. On the other hand, application level communication services such as DASH video streaming could be delivered via a Multicast session, and the same holds for non-real time distribution of Virtual Reality content.

MBS is an important communication service which enables delivery of the same critical as well as non-critical content. The use cases for the MBS can be traffic updates (accidents, signs, light status, speed notifications, etc.), warning messages, public safety, etc.

MBS is a communication service for delivering the content of diverse application layer services, such as, e.g., VR and DASH. MBS is hence not an application layer service type.

Support QoE and RVQoE measurement collection for MBS, regarding MBS as a communication service, not as an application layer service type.

Given that MBS is a communication service (i.e., a means for data delivery) rather than an application layer service, there is no actual "experience" of MBS to measure the quality of. Hence, there is no real sense in sending a QoE report for an MBS session without specific consideration of the delivered content. Of interest is rather the way the MBS communication service impacts the QoE of the application layer service that is carried by the MBS communication service. Thus, it becomes natural to include the MBS related QoE metrics in the QoE configuration and QoE report of each application layer service session that is carried by MBS.

MBS related QoE metrics are pointless without an application layer service context.

Include MBS related QoE metrics in the QoE configuration and the QoE report(s) for the application layer service session that is carried by MBS.

## 2.2	QoE measurements in RRC_INACTIVE and RRC_IDLE

As described in TS 38.300, a UE can receive a multicast communication service only in RRC_CONNECTED state, while the broadcast communication service can be received by a UE irrespective of its RRC state. That means that a UE can receive a broadcast service in either RRC_CONNECTED, RRC_INACTIVE, or RRC_IDLE state. During multicast MBS sessions, the MBS data can be delivered via 5GC Individual MBS traffic delivery method, while in both multicast MBS and broadcast MBS sessions, the MBS data can be delivered via 5GC shared traffic delivery method.

When there is no data to be sent to a UE for a multicast MBS session, the gNB may transfer a UE to RRC_INACTIVE or RRC_IDLE state. When a UE is in an RRC_INACTIVE state, the UE INACTIVE AS context is still stored in a UE and in the RAN. However, all Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs) are suspended, so that no RRC signalling, or data transmission or reception takes place. When a UE is entering the RRC_IDLE state, the UE context is released. Nonetheless, a UE is monitoring the paging channel, in case some data packet is arriving in DL. In the case of data being available in DL, the paging procedure is initiated, and it subsequently leads to the resuming of the session and the UE moving to an RRC_CONNECTED mode. On the other hand, a UE may receive MBS configuration for the broadcast session and broadcast communication service in either RRC_CONNECTED, RRC_INACTIVE, or RRC_IDLE states.

Upon entering the RRC_INACTIVE state, the QoE configuration is stored in the UE INACTIVE AS context, while upon entering the RRC_IDLE state the UE releases the QoE configuration. Entering either of those states prevents a UE from continuing the QoE measurements if the measurement configuration does not include conditions for doing so in those states. Before entering an RRC INACTIVE or IDLE state the UE needs to receive an RRCReconfiguration message instructing it to perform the measurements and store them while in INACTIVE/IDLE states. The actual instruction could be part of the QoE configuration, preferably in the measConfigAppLayerContainer which is created by the O&M system, in the MeasConfigAppLayer IE. These measurements can be logged and kept until a UE is in an RRC CONNECTED state and can forward the measurement reports to a gNB [4].

Broadcast MBS data can be delivered to a UE irrespective of its RRC state.

Allow QoE measurements to be performed in RRC_IDLE and RRC_INACTIVE state.

# Conclusion

In the previous sections we made the following observations:

Observation 1	MBS is a communication service for delivering the content of diverse application layer services, such as, e.g., VR and DASH. MBS is hence not an application layer service type.

Observation 2	MBS related QoE metrics are pointless without an application layer service context.

Observation 3	Broadcast MBS data can be delivered to a UE irrespective of its RRC state.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support QoE and RVQoE measurement collection for MBS, regarding MBS as a communication service, not as an application layer service type.

Proposal 2	Include MBS related QoE metrics in the QoE configuration and the QoE report(s) for the application layer service session that is carried by MBS.

Proposal 3	Allow QoE measurements to be performed in RRC_IDLE and RRC_INACTIVE state.

# References

3GPP TS 23.247, Architectural enhancements for 5G multicast-broadcast services, v17.3.0, 2022-06

3GPP TS 38.300, NR and NG-RAN Overall Description, v17.0.0, 2022-03

3GPP WID RP-221803, Enhancement on NR QoE management and optimizations for diverse services, 2022-06

3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v17.0.0, 2022-03