3GPP TSG-RAN WG2 #119e	Tdoc R2-2207726

Electronic meeting, 2022-08-17 - 2022-08-29

Agenda Item:	8.4.3

Source:	Ericsson

Title:	NR-DC with selective activation

Document for:	Discussion, Decision

# 1	Introduction

The Rel-18 work on further NR mobility enhancements is described in the WID in [1]. It includes, among others, an objective related to enhancements for PSCell mobility in case of NR-DC configurations (objective 2):

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

The WID in [1] also includes the following justification related to the above objective:

In Rel-17 Conditional PSCell change (CPC)/Conditional PSCell addition (CPA), a CPC/CPA-configured UE has to release the CPC/CPA configurations when completing random access towards the target PSCell. Hence the UE doesn’t have a chance to perform subsequent CPC/CPA without prior CPC/CPA reconfiguration and re-initialization from the network. This will increase the delay for the cell change and increase the signaling overhead, especially in the case of frequent SCG changes when operating FR2. Therefore, MR-DC with selective activation of cell groups aims at enabling subsequent CPC/CPA after SCG change, without reconfiguration and re-initialization on the CPC/CPA preparation from the network. This results in a reduction of the signalling overhead and interrupting time for SCG change.

In this contribution we discuss our understanding on the aspects to improve for the above objective and solutions that should be considered in the work.

# 2	Discussion

## 2.1	NR-DC with selective activation

As described in the WID ([1]) the focus for NR-DC with selective activation is the case of frequent SCG changes when operating in FR2, where the delays for cell changes and the signalling overhead should be reduced. The work item also includes other parts for mobility enhancements in Rel-18, such as the introduction of L1/L2 based inter-cell mobility mechanisms and procedures as well as enhancements to support CHO with candidate SCGs. Considering that the scope of the work item thus is very large, we should try to avoid that we end up discussing several different solutions for objective 2 (NR-DC with selective activation).

Only a single solution should be defined in Rel-18 for objective 2 (the NR-DC with selective activation of the cell groups).

The objective 2 in the WID [1] is to specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements to allow subsequent cell group changes without the need to reconfigure/re-initiate CPC/CPA. The corresponding justification in [1] clarifies that “MR-DC with selective activation of cell groups aims at enabling subsequent CPC/CPA after SCG change, without reconfiguration and re-initialization on the CPC/CPA preparation from the network”. The subsequent cell group changes should thus consist of conditional changes (CPC/CPA).

In order to support subsequent CPC/CPA procedures, without the corresponding reconfiguration/re-initialization from the network, the UE thus needs to already have those CPC/CPA configurations available after performing other cell group changes. The addressed scenarios should then include that the UE can be moving between multiple PSCells, including back to a previous serving PSCell, without the need for reconfiguration/re-initialization. The most straightforward mechanism is to enable this by keeping, i.e. not releasing, existing CPC/CPA configurations after performing a cell group change.

The selective activation of cell groups should correspond to support of subsequent conditional changes (CPC/CPA) after a cell group change.

The subsequent conditional changes (CPC/CPA), without reconfiguration and re-initialization, should be enabled by keeping, i.e. not releasing, already existing conditional reconfigurations after performing a cell group change.

Objective 2 is described as applicable for “at least for SCG”. A related question is then whether also corresponding mechanisms for the MCG should be considered in the Rel-18 work. Considering the large scope in the work item and that these enhancements are targeting FR2 and that it can be expected that MCG is typically on FR1, we think that only the SCG should be targeted in the work on objective 2 in Rel-18 (NR-DC selective activation). This means that the work in Rel-18 should only consider keeping CPC or CPA configurations, i.e. not CHO configurations, at execution of other mobility procedures.

Only selective activation of SCGs should be targeted in the Rel-18 work for objective 2, i.e. not for MCGs. This means that the work in Rel-18 should only consider keeping CPC or CPA configurations, i.e. not CHO configurations, at execution of other mobility procedures.

Legacy PSCell change and Conditional PSCell Change (CPC) are supported together, e.g. so that the UE can be configured with CPC for some candidate target PSCells but can anyway receive a configuration for a legacy PSCell change. The legacy PSCell change could then be to one of the candidate target PSCells or to another target PSCell and is then executed directly. The possibility to keep a conditional reconfiguration for the SCG (e.g. CPC) should therefore be supported both for the case where a UE executes another CPC configuration and when it executes a legacy PSCell change.

Whether it should be possible to keep a conditional reconfiguration for the SCG (CPC or CPA) in case there is a change of MCG/PCell should be discussed further and can be left as TBD for now.

The mobility procedures where it should be possible to keep a conditional reconfiguration for the SCG (e.g. CPC) should correspond to any type of SCG mobility procedure, i.e. both CPC execution and normal PSCell changes should be applicable. TBD whether it is applicable also in case there is a change of MCG, i.e. at a CHO execution or a normal HO.

## 2.3	CPC configuration within CPA/CPC

The objective 4 in the WID [1] is to specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC. It can thus be assumed that there then will be support for a CHO configuration that includes CPA/CPC.

Today it is not allowed to include the field conditionalReconfiguration in the condRRCReconfig, i.e. a new conditional reconfiguration within the conditional reconfiguration. This can be seen in the field description for condRRCReconfig in 38.331:

condRRCReconfig

The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config. For CPA and for CPC, the RRCReconfiguration message contained in condRRCReconfig cannot contain the field scg-State.

It is also described in 37.340 (clause 10.6) that CPC configurations cannot be included in conditional reconfigurations:

CPC configuration in HO command, in PSCell addition/change command or in conditional configuration (i.e CPA, CPC or CHO configuration) is not supported.

By adding support for CPC/CPA configurations within a CHO configuration, the above restrictions regarding inclusion of a CPC configuration within a conditional reconfiguration would be removed. The corresponding support for CPC configuration within a CPC or a CPA configuration would then also be possible unless new, more specific, restrictions are to be introduced in the specification. We therefore think that if support for CPC/CPA within CHO is introduced, RAN2 should consider to also support that CPC can be configured within a CPC/CPA configuration.

In case support for CPC or CPA within a CHO configuration is specified, the possibility to also support a CPC configuration within a CPC or CPA configuration may be supported without additional specification work.

RAN2 to consider support for inclusion of CPC configuration within a CPC or CPA configuration, in case CPC/CPA configuration is supported within CHO configuration.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In case support for CPC or CPA within a CHO configuration is specified, the possibility to also support a CPC configuration within a CPC or CPA configuration may be supported without additional specification work.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Only a single solution should be defined in Rel-18 for objective 2 (the NR-DC with selective activation of the cell groups).

Proposal 2	The selective activation of cell groups should correspond to support of subsequent conditional changes (CPC/CPA) after a cell group change.

Proposal 3	The subsequent conditional changes (CPC/CPA), without reconfiguration and re-initialization, should be enabled by keeping, i.e. not releasing, already existing conditional reconfigurations after performing a cell group change.

Proposal 4	Only selective activation of SCGs should be targeted in the Rel-18 work for objective 2, i.e. not for MCGs. This means that the work in Rel-18 should only consider keeping CPC or CPA configurations, i.e. not CHO configurations, at execution of other mobility procedures.

Proposal 5	The mobility procedures where it should be possible to keep a conditional reconfiguration for the SCG (e.g. CPC) should correspond to any type of SCG mobility procedure, i.e. both CPC execution and normal PSCell changes should be applicable. TBD whether it is applicable also in case there is a change of MCG, i.e. at a CHO execution or a normal HO.

Proposal 6	RAN2 to consider support for inclusion of CPC configuration within a CPC or CPA configuration, in case CPC/CPA configuration is supported within CHO configuration.

# References

RP-221799, Revised WID on Further NR mobility enhancements, MediaTek Inc., 3GPP TSG RAN Meeting #96, June 6-9, 2022