3GPP TSG-RAN2 Meeting #119-e          	          R2-2207738

Electronic meeting, 17th - 29th August, 2022

Agenda Item:	8.4.2.2

Source: 	Huawei, HiSilicon

Title: 	Solutions for L1/L2 mobility

Document for:	Discussion and decision

Introduction

The WID [1] on further NR mobility enhancement was approved in RAN plenary meeting #96 and one of the objectives is to specify the mechanism and procedures of L1/L2 based inter-cell mobility, to reduce the mobility latency and overhead and interruption time. In the remaining part of the contribution, we discuss the general procedure of L1/L2 handover and the candidate solutions for L1/L2 mobility.

Discussion

Configuration of candidate targets for L1/L2 mobility

The handover preparation phase:

As mentioned in the WID, configuration of multiple candidate target cells should be provided to UE in the handover preparation phase, to allow fast application of configurations when the L1/L2 handover is triggered. Considering that it is the gNB-CU who stores the neighbour relationship, especially the relationship of neighbour cells in the inter-DU mobility case, it is better for the gNB-CU to determine the candidate cells, taking the L3 measurement report into consideration. Before the gNB-CU sends the pre-configuration for the candidate cells to the UE, the gNB-CU needs to obtain the configurations of candidate cells from the gNB-DU in the intra-DU mobility case or from the target gNB-DU in the inter-DU mobility case.

Observation 1: The gNB-CU is more suitable to determine the candidate cells than the gNB-DU during the preparation phase for pre-configuration.

When it comes to the contents of pre-configuration for candidate cells, there are several aspects to take into consideration, such as the applicable scenarios, the latency and the signalling overhead. We describe below several ways to provide the configuration:

- RRC reconfiguration: Each candidate target for L1/L2 handover can be a full RRCReconfiguration message, similarly to the CHO case.

- Cell group configuration: Because the L1/L2 mobility can only be applied in the intra-CU handover scenario, the PDCP configuration (i.e. pdcp-Config) and the measurement configuration (i.e. measConfig) of the source cell can be reused in the candidate cells, so it is sufficient to provide the CellGroupConfig for each target configuration in some scenario.

- Serving cell configuration: assuming cell group-level parameters need not be changed, the configuration could include a number of SpCellConfig and SCellConfig, which surely works in the intra-DU handover scenario. Apparently, RLC config is not be able to be updated during L1/L2 mobility in this approach.

- Partial serving cell configuration: alternatively, for each serving cell L1/L2 handover can change a limited set of parameters of serving cell configuration and the UE is provided, for each target cell, with such parameters. This may be coupled with inter-cell beam management, either the Rel-17 or the Rel-18 version, but this may also be separate.

- Common channel configuration for additional PCIs: in inter-cell beam management, the UE can be configured with additional PCIs for each serving cell and TCI states can refer to SSBs with such PCIs, so that, without any change of configuration, dedicated channels will be handled by a different cell. In order to make a handover, the UE should switch to the common channels of the other cell, so that configuration should be provided to the UE. Avoiding interruption of dedicated channel transmission may require some restrictions on the differences between common channels of different cells and/or scheduling restrictions on the network side.

Obviously, if less parameters can change with L1/L2 handover, this puts some restriction on the network deployments where L1/L2 mobility can be used. But this may reduce the signalling size and the storage in the UE. If this also reduces the handover latency, it could be useful to have such restrictions, but the details needs further discussion.

Proposal 1a: RAN2 confirm that the configurations of candidate cell(s) are pre-configured to UE before the handover command in Rel-18 L1/L2 cell mobility.

Proposal 1b: Discuss which part of RRC configuration can be updated/changed for Rel-18 L1/L2 cell mobility, considering the applicable deployment scenarios, the latency and the signalling overhead, e.g.:

- Option 1: an RRC reconfiguration: the RRCReconfiguration can include all RRC parameters

- Option 2: Cell group configuration: a CellGroupConfig IE is provided

- Option 3: Serving cell configuration: alternative SpCellConfig and/or SCellConfig

- Option 4: Common channel configuration for additional PCIs: for the additional PCIs in ICBM, the corresponding common channel parameters (i.e. servingcellConfigCommon)

The handover execution phase:

In Rel-16, both network-controlled and UE-triggered handover are supported. For UE-triggered handover, the UE applies execution conditions relying on L3 measurements but as discussed in [1], L1 measurements are far more frequent, L1 measurements defined for inter-cell beam management could already be used and, combined with a L1/L2 handover command, could greatly reduce the handover latency. In scenarios where L1 measurements cannot be used, execution conditions could be considered, but since the latency would be larger, we propose to focus only on network-controlled handover.

Proposal 2: There is no execution condition(s) in the pre-configuration, since the L1/L2 mobility in Rel-18 is controlled by L1/L2 signalling.

Handover decision

Furthermore, to further decrease the delay of L1/L2 handover procedure, L1 measurement configuration and report can be applied to determine the target cell of L1/L2 handover. After receiving the L1 measurement reports, the gNB-DU can decide when to trigger the L1/L2 handover and which is the target cell among the candidate cells.

Proposal 3: The L1/L2 handover decision (i.e. when to switch and which cell to switch) is made by the gNB-DU in intra-DU handover scenario and by the source gNB-DU in inter-DU handover scenario.

Handover command

The gNB-DU can use the L1/L2 handover command to indicate to the UE the target cell and optional beam information (e.g. TCI state). The beam information can help the UE to complete the beam alignment in advance. Additional contents could also be considered. Therefore, RAN2 should determine the contents of the handover command before selecting L1 signalling or L2 signalling, as DCI may not suitable to carry a lot of information.

Proposal 4a: The L1/L2 handover command indicates the target configuration to use, among the list of previously received configurations.

Proposal 4b: RAN2 first determine the contents of the L1/L2 handover command, before deciding between L1 signalling vs. L2 signalling.

Switch optionally without RACH

As discussed in [2], eliminating the RACH delay during a handover procedure can significantly reduce the handover delay. There is already one scenario where the target PCell is a current SCell with UL, in which the TA towards the current SCell is already.

Proposal 5: Support the procedures/solutions to avoid RACH during R18 L1/L2 mobility, at least when the UL TA towards the target cell is available before handover.

In LTE with RACH-less HO procedure, only timing adjustment indication that TA of the target cell equals to 0 or reusing the TA from the source cell is allowed. However, if only the RACH-less TA obtain mechanism of LTE is considered, the usage of RACH-less is limited. Therefore, further enhancement is needed in RAN1.

Proposal 6: Ask RAN1 to work on solutions to obtain the TA towards the candidate target cell before L1/L2 handover.

Subsequent L1/L2 mobility

If the UE releases the L1/L2 mobility configurations when completing the access to the target cell, then the UE doesn’t have a chance to perform “subsequent L1/L2 HO” without RRC reconfiguration and re-initialization from the network.

For easy to understanding, the following Figure 3 shows an example of intra-DU subsequent handover procedure. The procedure of the inter-DU handover case is quite similar to that of intra-DU handover case.

Figure 1. The procedure of intra-DU L1/L2 subsequent handover

First time handover procedure:

- Handover preparation phase during the first handover procedure. Same as step 1~ step 6 in Figure 2.

- Handover execution phase during the first handover procedure. Same as step 7~ step 10 in Figure 2.

- Same as step 11 in Figure 2.

After completing the access to the target cell, the UE and the gNB maintain the configuration of the candidate cells and previous source cells, instead of releasing them to support the subsequent L1/L2 handover.

Subsequent handover procedure:

For subsequent L1/L2 handover, the handover preparation phase does not need to be performed again. The network can trigger UE to execute the subsequent L1/L2 HO after the first time L1/L2 HO execution, which does not have to be RRC configured. The UE and gNB-DU can perform the subsequent L1/L2 handover based on the configuration of candidate cells or source cell for the first L1/L2 handover in step 1.

4/5/6/7/8.  Same as step 7, 8, 9, 10, 11 in Figure 2. The gNB-DU makes the handover decision and sends the handover command, the target configuration indicated in the subsequent L1/L2 handover command is one of configuration of the candidate cells pre-configured in step 1 or the source cell for the first handover. The UE performs subsequent handover to the target cell using the L1/L2 handover configuration configured in step 1 or the previous source cell configuration.

9.  The gNB-CU can instruct the UE to release the L1/L2 handover configuration via the RRCReconfiguration message.

From the above procedure, it is noted that if the UE supports subsequent L1/L2 handover, the UE does not need to perform the handover preparation phase in the subsequent handover procedure. Based on the analysis of [2], this saves above 20 ms of latency. In addition, this reduces the signalling overhead of RRC reconfiguration over the air interface. Therefore, we think that subsequent L1/L2 HO can be supported to further improve the mobility performance.

Proposal 7: L1/L2 mobility can support the “subsequent L1/L2 HO”, i.e. NW can trigger UE to execute the subsequent L1/L2 HO after the first time L1/L2 HO execution, which does not have to be RRC reconfigured.

Conclusion

Observation 1: The gNB-CU is more suitable to determine the candidate cells than the gNB-DU during the preparation phase for pre-configuration.

Proposal 1a: RAN2 confirm that the configurations of candidate cell(s) are pre-configured to UE before the handover command in Rel-18 L1/L2 cell mobility.

Proposal 1b: Discuss which part of RRC configuration can be updated/changed for Rel-18 L1/L2 cell mobility, considering the applicable deployment scenarios, the latency and the signalling overhead, e.g.:

- Option 1: an RRC reconfiguration: the RRCReconfiguration can include all RRC parameters

- Option 2: Cell group configuration: a CellGroupConfig IE is provided

- Option 3: Serving cell configuration: alternative SpCellConfig and/or SCellConfig

- Option 4: Common channel configuration for additional PCIs: for the additional PCIs in ICBM, the corresponding common channel parameters (i.e. servingcellConfigCommon)

Proposal 2: There is no execution condition(s) in the pre-configuration, since the L1/L2 mobility in Rel-18 is controlled by L1/L2 signalling.

Proposal 3: The L1/L2 handover decision (i.e. when to switch and which cell to switch) is made by the gNB-DU in intra-DU handover scenario and by the source gNB-DU in inter-DU handover scenario.

Proposal 4a: The L1/L2 handover command indicates the target configuration to use, among the list of previously received configurations.

Proposal 4b: RAN2 first determine the contents of the L1/L2 handover command, before deciding between L1 signalling vs. L2 signalling.

Proposal 5: Support the procedures/solutions to avoid RACH during R18 L1/L2 mobility, at least when the UL TA towards the target cell is available before handover.

Proposal 6: Ask RAN1 to work on solutions to obtain the TA towards the candidate target cell before L1/L2 handover.

Proposal 7: L1/L2 mobility can support the “subsequent L1/L2 HO”, i.e. NW can trigger UE to execute the subsequent L1/L2 HO after the first time L1/L2 HO execution, which does not have to be RRC reconfigured.

References

- RP-221799, Revised WID on Further NR mobility enhancements.

- R2-2207637, L1/L2 mobility target performance enhancements, Huawei, HiSilicon

Annex: General L1/L2 mobility procedure

From the perspective of C-plane handling, the L1/L2 handover procedure consists of handover preparation, execution and completion phase, which is consistent with Rel-16 RRC managed handovers.

The procedure of intra-DU handover and inter-DU handover can be shown in the Figure 2 and Figure 3, respectively.

Figure 2. The procedure of intra-DU L1/L2 handover

- The gNB-CU configures L3 measurements and the UE reports according to the measurement configuration.

- The gNB-CU makes the L1/L2 handover preparation decision and may determine the candidate cells of L1/L2 handover based on the measurement reports from the UE.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU, to request the addition of candidate cells for L1/L2 handover.

- The gNB-DU replies to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

- The gNB-CU transmits the L1/L2 handover configuration by sending an RRCReconfiguration message to the UE. The L1/L2 handover configuration includes a list of target configurations and each target configuration includes one PCell and optionally multiple associated SCells.

- When receiving the RRCReconfiguration message with the L1/L2 handover configuration, the UE stores the L1/L2 handover configuration, applies other fields of the RRC reconfiguration and replies the gNB with an RRC reconfiguration complete message.

- The UE performs L1 measurements to the L1 measurement configuration and transmits the L1 measurement report to the gNB-DU. The L1 measurement configuration can be provided in the RRCReconfiguration message in steps 5/6.

- The gNB-DU decides to trigger the L1/L2 handover, based on the L1 measurement reports. Similarly, in the case of inter-DU L1/L2 handover, the source gNB-DU makes the L1/L2 handover decision.

- The gNB-DU sends the L1/L2 handover command via MAC CE or DCI to the UE. The L1/L2 handover command at least indicates the target configuration that is to be applied from the L1/L2 handover configuration.

- After receiving the L1/L2 handover command, the UE synchronizes to the target cell (if not already done) and completes the handover procedure. The UE performs the random access to the target PCell if the UE does not have a valid TA for the target PCell. Otherwise, the random access procedure can be avoided and replaced with a RACH-less procedure to further reduce the latency of mobility as we analysed in [2], for example a procedure similar to the RACH-less handover procedure in LTE or another enhanced solution.

- After the UE has completed the access to the target PCell, the gNB-DU informs the gNB-CU of which cell the UE has successfully accessed via an ACCESS SUCCESS message. If needed, the gNB-CU performs data forwarding from the source gNB-DU to the target gNB-DU according to the indication (in the case of inter-DU handover).

- The gNB-CU can instruct the UE to release the L1/L2 handover configuration via the RRCReconfiguration message.

The procedure for the inter-DU handover case is quite similar to the procedure for the intra-DU handover case, as shown as in the Figure 3. One difference is that, in the inter-DU handover case, the target gNB-DU must reserve the resources for the configured L1/L2 handover targets (e.g. the resources for CFRA). The similar issue also exists in Rel-16 CHO procedure. To avoid this, a solution would be that, before sending the L1/L2 handover command, the source gNB-DU informs the target gNB-DU via the gNB-CU but this solution would introduce extra delay, which goes against the motivation of introducing the L1/L2 mobility.

Observation: Introducing exchanges between the source gNB-DU and the target gNB-DU would increase the latency of L1/L2 mobility.

Figure 3. The procedure of inter-DU L1/L2 handover

Observation: Based on the procedures of intra and inter-DU handover, the L1/L2 procedure at least includes the pre-configuration of candidate cells, L1 measurement configuration/reporting, L1/L2 Handover Command and the access to the target cell with or without RAHC, from the perspective of RAN2.