3GPP TSG-RAN WG2 Meeting #119-e		R2-2207786

E-Meeting, 17th – 29th Aug, 2022

Agenda Item:	8.3.2

Source:	vivo

Title:	Discussions on time domain techniques for network energy saving

Document for:	Discussion and Decision

Introduction

According to RAN1#109-e meeting, the following agreements were made[1]:

Agreement Further study techniques and enhancements for increasing time domain energy saving opportunities by the gNB, including (but not limited to) the following aspects: - potential methods of reducing/adapting transmission/reception of common channels/signals, e.g. SSB, SIB1, other SI, paging, PRACH, and its impact to initial access procedure, cell (re)selection, handover, synchronization and measurements performed by the idle/inactive/connected UE; - potential methods of reducing transmission/reception of common channels/signals can include no- or reduced-transmission/reception, increased periodicity, enablement of on-demand transmission/reception of common channels/signals, or offloading of common channels/signals to other carriers or use of light or relaxed versions of common channels /signals - potential methods of reducing/adapting transmission/reception of periodic and semi-persistent signals and channels configuration such as CSI-RS, group-common/UE-specific PDCCH, SPS PDSCH, PUCCH carrying SR, PUCCH/PUSCH carrying CSI reports, PUCCH carrying HARQ-ACK for SPS, CG-PUSCH, SRS, positioning RS (PRS), etc. - semi-static and/or dynamic cell on/off in one or more granularity, e.g. /subframe/slot/symbol; some examples are: - Cell/network node activation request by the UE, for example using signal/channel from UE for gNB’s wake-up request - enhancements to L1/L2 based mobility to efficiently enable a network node (e.g. TRP, repeater) on/off operation within a cell (within network energy saving SI scope) - signaling enhancements for indication of semi-static and/or dynamic cell/subframe/slot/symbol on/off duration - support of periodic and/or on-demand reference signal(s) from the gNB to aid discovery of a cell; - dynamic adaptation of UE C-DRX configurations in a UE-group or cell-specific manner - Mechanism to utilize potential energy saving states or sleep modes and the transition between states from leveraging cell on/off opportunities - including studies of waking up gNB due to user traffic, or user density, or gNB receiving wake up signal - including technique to allow discovery and measurement of cells in sleep or dormant states - UE assistant information facilitating BS time domain adaptation

In this contribution, we investigate various aspects considering scenarios and potential issues for time domain network energy saving (NES) techniques.

Discussion

General description

In RAN1#109-e meeting, various time domain techniques for NES were raised, which can be generally summarized as ‘No-, reduced- or period-increased transmissions of DL Common and dedicated periodic/semi-persistent signals and receptions of UL channels/signals for gNB/cell including gNB/cell on/off status’. And the methods to realize it can be categorized as:

- gNB/cell direct implementation:

gNB/cell directly enables ‘No-, reduced- or period-increased transmission/reception’ without considering network’s traffic and UE’s assistance. In this case, we only need to evaluate what issues it may raise from UE side (loss of initial access opportunity for UE, energy consumption raise in other gNB/cell instead, etc)and methods to tackle with, e.g. common/dedicated channels/signals adaptation.

- on demand transmission/reception:

gNB/cell enables ‘No-, reduced- or period-increased transmission/reception’ with the information provided by UE(UAI, CSI report, etc.) or by UE’s request (WUS for gNB/cell’s wake-up request, dedicated signaling)

In the follow sections, we briefly review some time domain methods for network energy saving and their related issues.

Common/dedicated channels/signals adaptation

According to current specification, many common signals support to be period-adjusted. Take SSB for example, the period of SSB can be 5, 10, 20, 40, 80, 160 ms. For the sake of UE initial cell search, the default period of SSB is set to 20 ms. If a cell chooses to enter certain sleep mode, it can change the period of SSB to 160ms and indicate it to the UE with SIB1, which does not affect the camped UEs, but may lead cell searching UE to take longer time to camp on it with respect to different UE implementation.

- The increased SSB period of a cell may cause a cell searching UE taking longer time to camp on it, but not prevent the camped UE from staying camping on it.

As change of SSB period also affects RRM/RLM measurement, RAN4’s inputs on this is identically necessary. We suggest to study the impact of it to RAN2, e.g. IDLE UE measurement, but not the method detail so soon, which can be also contributive to the other WGs regarding the decision about whether to support such feature or not.

- For SSB period adaptation, prioritize studying potential RAN2 impact over the method details discussing.

As for the paging/PRACH adaptation, they may be submissive to the method of SSB/SIB1 period adaptation and the sleep modes of gNB/cell, e.g. if a cell is in a turned-off or quasi-off sleep mode, it may not able to perform paging or PRACH reception. For example, certain sleep modes of gNB/cell can compromise the procedure of paging/PRACH, e.g. tx/rx shut down in certain frame/slots, or UE may be unable to camp on an energy-saving cell with other energy-saving techniques enabled. Therefore, the related discussion for which can be deprioritized.

- Wait for more RAN1 inputs on the definition of gNB/cell sleep mode before studying paging/PRACH adaptation from RAN2’s perspective.

WUS for gNB/cell’s wake-up request

In NR Rel-16 UE power saving WI, a PDCCH-based wake-up signal at BS side was designed to assist UE power saving by indicating whether UE needs to be active for the upcoming DRX on duration. Similar concept can apply to network energy saving case, i.e. gNB/cell activation by UE wake up signal (WUS).

Legacy cell activation/deactivation mechanism

In NR Rel-17, SA designs cell activation/deactivation mechanism for capacity booster cell overlaid by candidate coverage cell(s) in TS 28.310 [2]. As described in TS 28.310, load-based activation/deactivation procedures for Cell B are specified to save network energy consumption. The cell deactivation of Cell B may be triggered in case that light traffic in Cell B is detected. Cell A providing basic coverage may request cell B re-activation based on its own cell load information or neighbour cell load information.

Fig. 1: Cell activation of capacity booster cell (Cell B) by candidate coverage cell (Cell A)

Motivation and issues of UE wake up signal (WUS)

When evaluating the legacy mechanism, the most difficult part is how to activate the turned-off or quasi-off sleep cell when there is real need.

On one hand, in HetNet case as illustrated in Fig. 1, legacy load-based cell activation may not be accurate since candidate cell (i.e. Cell A) may not know whether the increased load exists in the area of switching-on capacity booster cell (i.e. Cell B). As an extreme case, it may occur that almost all the traffic loads are coming from UEs outside Cell B coverage but inside Cell A coverage when Cell A sends activation request to Cell B which results in a waste of network energy saving. Besides, if there are several capacity booster cells within one candidate coverage cell, all of them need to be activated due to unknown information of load source. Thus, this is really a waste of network energy and a more accurate activation mechanism is needed in this HetNet case.

On the other hand, in non-HetNet case as illustrated in Fig. 2, there is no good mechanism to wake up the cell if it is deactivated when there is no or little load. In other words, the cell don’t dare to be deactivated even when there is no any load since it can’t be woken up when needed (e.g. a UE moves to its area). If the cell is deactivated in this case, the UE performance will be degraded a lot. The only choice is to switch on all cell BSs in this non-HetNet case which results in large network energy consumption.

Therefore, WUS for gNB/cell’s wake-up request may be a good choice to serve both cases, providing information for the network to implement more accurate and efficient cell on/off.

Fig. 2: Potential UE wake up signal procedure

From RAN2’s perspective, the following issues about WUS for gNB/cell’s wake-up request can be discussed:

issue 1: Whether the UE WUS mechanism can be applicable for UE in different state, i.e. IDLE/INACTIVE/CONNECTED mode?

It is possible that UE in any state can use WUS to send wake-up request to a neighbor cell in sleep mode. If a UE is in IDLE/INACTIVE state, when conditions for neighbour cell measurement are met, UE can send WUS to try waking up a sleep neighbour cell for neighbour cell assessment. If a UE is in CONNECTED state, instead of blindly waking up the sleep neighbour cell through Xn interface without knowing if the UE is really approaching it, the serving cell can require the UE to send WUS to the sleep neighbour cell, if the measurement to neighbour cell satisfy the need for hand-over, the serving cell can indicate hand-over to the UE, knowing that the UE is indeed in the vicinity of the just-waken-up neighbour cell.

issue 2: How does the UE identify a sleep gNB/cell?

Two possible ways for identifying a sleep gNB/cell include:

1. a gNB/cell sends long-period SSB (e.g. SSB with 160 ms periodicity);

2. a gNB/cell sends periodic and/or on-demand reference signal(s) to aid discovery of it;

Either way enables UE to discover a sleep gNB/cell, and sends WUS if it maintains available WUS configurations. Yet, other ways to aid discovery of a sleep gNB/cell are not excluded

issue 3: How does the gNB/cell monitor WUS?

From gNB/cell’s side, it can continuously monitor WUS or set up periodical WUS monitoring occasions. This depends on how we evaluate the power consumption of gNB/cell reception and the UE performance upon such network energy saving case.

For example, if companies agree that the rx power consumption is relatively small compared to the tx power, continuous monitoring can be considered. It enable more WUS opportunities, from which gNB/cell can quickly response to all possible UE requests for gNB/cell activation.

issue 4: Potential gNB/cell’s reaction to WUS

In Rel-16 UE Power Saving, the UE should monitor the PDCCH occasion following the WUS occasion once it detects a WUS indicating PDCCH monitoring. However, the gNB/cell might not necessarily be turned on at the instance that it receives a WUS from UE. On one hand, the gNB can treat UE WUS as a kind of UE assistance information, the gNB can decide to switch on once it detects enough amount of UE sending WUS. On the other hand, there can be some activated gNB nearby or a gNB providing coverage remains available, hence there’s no need for such energy-saving gNB to be activated. Moreover, the gNB can notice the UE that it refuses to switch on.

The way gNB/cell reacts to UE WUS can affect the UE WUS procedure design. It is beneficial that the possible gNB/cell reactions to UE WUS can reach a consensus.

- Discuss the below issues regarding WUS for gNB/cell’s wake-up request.

(1) Whether the UE WUS mechanism can be applicable for UE in different state, i.e. IDLE/INACTIVE/CONNECTED mode;

(2) How does the UE identify a sleep gNB/cell;

(3) How does the gNB/cell monitor WUS;

(4) Potential gNB/cell’s reaction to WUS;

Conclusion

Based on the discussion, we have the following observation and proposals:

- The increased SSB period of a cell may cause a cell searching UE taking longer time to camp on it, but not prevent the camped UE from staying camping on it.

- For SSB period adaptation, prioritize studying potential RAN2 impact over the method details discussing.

- Wait for more RAN1 inputs on the definition of gNB/cell sleep mode before studying paging/PRACH adaptation from RAN2’s perspective.

- Discuss the below issues regarding WUS for gNB/cell’s wake-up request:

(1) Whether the UE WUS mechanism can be applicable for UE in different state, i.e. IDLE/INACTIVE/CONNECTED mode;

(2) How does the UE identify a sleep gNB/cell;

(3) How does the gNB/cell monitor WUS;

(4) Potential gNB/cell’s reaction to WUS;

References

- Chair's notes RAN1#109-e v19

- TS 28.310 v17.3.0