3GPP TSG RAN WG2 Meeting #119-electronic               	R2-2207829

# Online meeting, 17th 26th August 2022

Agenda Item: 	8.2.3

Source:		Sony

Title:			Considerations on solution for integrity of RAT dependent positioning techniques

Document for:	Discussion & Decision

# Introduction

A new release-18 study item on expanded and improved NR positioning was approved in RANP#94e meeting in December 2021 [1].

The study item has several objectives, in which one of them is related to Integrity

Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

RAN2 has not discussed any Rel-18 topics yet, and the upcoming RAN2#119e meeting is the first one to discuss amongst other this topic.

In this contribution, we discuss potential solutions for RAT dependent integrity.

# Discussion

## Background

The application layer of a UE may contain applications that are highly depending on certain accuracy being provided from a location system, but also other applications or users that may require an accurate positioning estimate of the UE. Furthermore, the location system may also need a view on to what level the positioning measurement are reliable.

In release 17 features to support GNSS integrity was included based on the GNSS integrity concepts. From SID [1] it was agreed to study integrity solutions for also a RAT dependant positioning. There concepts might be preferred to reuse as much as possible from also for RAT dependant integrity concept.

The basic definition of Positioning Integrity is that it is a measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts.

There is a need to have integrity to ensure the provided positioning estimate is fulfilling various levels of positioning requirements (accuracy, latency) for the intended service level.

In Rel-17 a number of error sources were identified and listed related to GNSS error sources, e.g. based on orbital, clock, code bias, phase bias Ionosperic or Tropospheric errors, that may affect the positioning measurements.

In this paper we make some reflections related to error sources, validity time for reporting and the possibility for adaptation of RAT-dependent positioning based on input from non-RAT dependent methods.

## Discussion on RAT dependent integrity

It is relevant to discuss good metrics as e.g. error sources to signal to the LoCation Services (LCS) supporting similar concept as of GNSS integrity. However, we see the LPP [2] messages for RequestLocationInformation, ProvideLocationInformation, RequestAssistanceData and ProvideAssistanceData, could be complemented and also used for RAT dependent based integrity KPIs/results.

Observation 1: The LPP messages for RequestLocationInformation, ProvideLocationInformation, RequestAssistanceData and ProvideAssistanceData, can be used for RAT dependent based integrity KPIs/results

In Rel-17, some of the error sources identified has been identified for GNSS. fFor RAT-dependent positioning methods, similar error sources can be identified. Additionally, other sources might be relevant such as, mobility or movements of the UE, low SNR, high temperature, clock drift, few or weak PRS signals, NLOS aspects, etc.

Observation 2: GNSS error sources can be the basis for exploring further error sources related to RAT-dependent positioning.

Proposal 1: Consider RAT-dependent specific error sources e.g., related to reception of PRS transmissions, UL-SRS transmissions and NLOS aspects.

We see the time duration from request the positioning information to receiving integrity of the qualified positioning results as important. The application side which triggers the positioning request may know the positioning requirements and therefore it should be possible to request the positioning information to be reported from within a time window with a maximum time duration. The integrity metrics should be communicated to LCS as soon as ready within the maximum time duration (i.e., as limit). Additionally, it could make sense with a priority order of the required positioning methods to be performed to make sure the best positioning measurement is reported within the valid time duration. Furthermore, if one positioning method delivered a result fulfilling the expected level of trust there could be an option to abort the other methods and only report the selected result.

Proposal 2: Consider a maximum reporting time for positioning measurements in which the important/prioritized measurement data should be available within the maximum reporting time.

Additionally, there is also an opportunity of using integrity for adapting the RAT based positioning. We argue it is not useful to perform positioning estimations that are anyway not used by the application due to the trustworthiness of the estimations or any error source in the RAT based positioning system. Similarly, there might be less use of performing any RAN based localization if a better estimation of the position is provided by RAT-independent based method. Thereby we consider letting the UE have ability to adapt its position measurements and estimations based on other integrity metrics in a device. In other words, integrity KPIs/results from e.g GNSS could be used to set operating conditions for RAT based positioning. Thereby we propose to have the option to signal RAT-independent based integrity KPIs/results to RAN nodes to assist in positioning method selection/enable/disable or even act as input to resource scheduling for positioning reference signals.

Proposal 3: Consider RAT-independent integrity KPIs/results be available for adapting the RAT-dependent positioning methods and integrity reporting.

# Summary

In this contribution, we have discussed our view on RAT dependent integrity. Our observation and proposals are listed below:

Observation 1: The LPP messages for RequestLocationInformation, ProvideLocationInformation, RequestAssistanceData and ProvideAssistanceData, can be used for RAT dependent based integrity KPIs/results

Observation 2: GNSS error sources can be the basis for exploring further error sources related to RAT-dependent positioning.

Proposal 1: Consider RAT-dependent specific error sources e.g. related to reception of PRS transmissions, UL-SRS transmissions and NLOS aspects.

Proposal 2: Consider a maximum reporting time for positioning measurements in which the important/prioritized measurement data should be available within the maximum reporting time.

Proposal 3: Consider RAT-independent integrity KPIs/results be available for adapting the RAT-dependent positioning methods and integrity reporting.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021

- LPP, 37.355, “LTE Positioning Protocol (LPP), Rel-17