3GPP TSG RAN WG2 Meeting #119-electronic               	R2-2207831

# Online meeting, 17th 26th August 2022

Agenda Item: 	8.5.2

Source:		Sony

Title:			Considerations on XR awareness

Document for:	Discussion & Decision

# Introduction

A new Rel-18 study item was approved at RANP#94 in December 2021, including the following objectives. The study is to be based on RAN1, Rel-17 TR 38.838 [2], SA4 work in SP-210043 [3], and SA2 work as per SP-220705 [4].

The study is to be based on Rel-17 TR 38.838, on corresponding Rel-17 work from SA4 (as per SP-210043) and on Rel-18 work from SA2 (as per SP-220705). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; Dynamic scheduling/grant enhancements

RAN2 has not discussed any Rel-18 topics yet, and the upcoming RAN2#119e meeting is the first one to discuss amongst other this topic.

In this contribution, we discuss potential solutions for XR awareness, particularly related to PDU set information.

# Discussions on XR awarness

## Background

In SA2, the study FS_XRM has started including KI#4 (PDU Set integrated packet handling) and KI#5 (Differentiated PDU Set Handling). SA2 have in for example solution #50 defined a Relative QoS within a PDU Set that allow RAN to optimize the BLER (Block Error Rate) within a PDU Set, TS 23.700-60 [5].

The study in Rel-18 in is ongoing and is planned to be ready in August 2022.

SA2 is still working on the study and has not concluded the study or started the normative work.

## Discussion on Priority of PDUs within a PDU set.

XR transmission is typically involving a video transmission. A video transmission, particularly a video transmission with high quality, may require large packet size. A large packet size is typically divided into sub-packets or PDUs, belonging to a PDU set. Each sub-packet is transmitted from the source application over the internet via the UPF and the RAN connection to the application in the UE.

Assuming a PDU Set contains the same type of content/importance, then typically for XRM service, a PDU Set needs to be delivered as a whole and if some pieces are not delivered in time, then the complete PDU Set should be dropped. i.e. if not all PDU´s within the PDU set are delivered, then the packets delivered successfully, are transmitted in vain.

Observation 1: In order to not waste radio resources, if the initial packets in a PDU set are delivered successfully, the delivery of the remaining of the PDUs should be delivered with increasingly priority, in order to not have wasted the initial packets, if not the remaining packets were delivered.

Hence from a radio resource/bandwidth perspective, the last PDU´s within a PDU set should have higher importance compared to the first or initial PDU´s within the same PDU Set.

If the PDU Set contains multiple sub-types of flows such as video, audio, haptic data or sensor data as defined in some solutions then the same is valid for each of the sub-types.

Observation 2: In case there are different types of PDUs the different types shall be handled separately with different priorities.

During normal transmission the NG-RAN calculates the needed modulation or coding rate to achieve a specific BLER (Block Error Rate) to fulfill the desired QoS of the PDU. The existing solutions expect the QoS to be equal for all PDUs with the same importance within the PDU Set.

There is a need to improve the resource efficiency in 5GS, e.g. in NG-RAN by allowing NG-RAN to be aware of where within the PDU Set it transmit PDUs in order for NG-RAN/gNB to be able to adjust the reliability level within the PDU Set so that the reliability is less in the beginning compared to the end in the same PDU Set.

Proposal 1: NG-RAN/gNB should be informed about potential differentiation of reliability levels within a PDU set from Core Network nodes, including AMF and UPF.

Depending on solution, this may be mainly to be discussed by SA2 and RAN3 for CN-gNB signalling. Potential impact for RAN2, may be related to new ways to handle QoS and that RAN needs to be aware of the PDU set since RAN should be able to handle different reliability in the beginning and end of the PDU Set.

Proposal 2: Send LS to SA2/RAN3, that RAN2 like to follow such discussions, that RAN needs to be aware of the PDU set and that RAN2 is notified if any RAN2 impact is identified.

## Early feedback for a PDU within the same PDU set

If we assume the current protocol structure, the application layer PDU generated at XR codec will be segmented into smaller PDUs of size 1500 octets (to fit the transmission size of IP packet). In addition, PDCP PDU may then be segmented by RLC sub-layer to fit the MAC transport block (TB). MAC TB is then fed to HARQ entity and HARQ will perform its own segmentation. It is clear that segments of a PDU set may not be traceable on lower layers due to e.g., segmentation in RLC and HARQ and then multiplexing of data in MAC sub-layer. So, the feedback for successful/unsuccessful PDU transmission/reception may be implemented in PDCP layer (once the PDU is processed and combined through different layers). However, the feedback in PDCP layer might be too late because there are delays related to RTT (Round Trip Time) over the air interface, processing time in each sub-layer in the UE and gNB, transport delay between CU (PDCP), DU (RLC, MAC) and probably RU (PHY layer) on the network side.

The other approach could be to allow to implement early lower layer feedback. This may need to configure certain restrictions so that the application layer PDU segment is identifiable on lower layers. These restrictions imply that a PDU segment is mapped to a specific DRB, logical channel and MAC TB where MAC TB contains RLC segments of the same PDCP PDU, and a specific HARQ process number (one or more) is reserved for this TB. This way, a segment of PDU set can be traced through different layers. Hence, the transmitter waits for feedback before recycling to the same HARQ process. In addition, it is worth propagating the feedback at HARQ layer to upper layers so that data from RLC/PDCP buffer can be discarded. This may save unnecessary processing of packets. Note that transmitting more than one TB containing data related to different logical channels or different PDUs from either the same or different PDU set should be maintained as much as possible.

Proposal 3: Consider a mechanism for early lower layer feedback, e.g., by configuring restrictions via RRC such that application PDU segment is identifiable at lower layer, like HARQ.

# Summary

In this contribution, we have discussed our view on potential XRxxxx in NR. Our observation and proposals are listed below:

Observation 1: In order to not waste radio resources, if the initial packets in a PDU set are delivered successfully, the delivery of the remaining of the PDUs should be delivered with increasingly priority, in order to not have wasted the initial packets, if not the remaining packets were delivered.

Observation 2: In case there are different types of PDUs the different types shall be handled separately with different priorities.

Proposal 1: NG-RAN/gNB should be informed about potential differentiation of reliability levels within a PDU set from Core Network nodes, including AMF and UPF

Proposal 2: Send LS to SA2/RAN3, that RAN2 like to follow such discussions, and to be notified if any RAN2 impact is identified.

Proposal 3: Consider a mechanism for early lower layer feedback, e.g., by configuring restrictions via RRC such that application PDU segment is identifiable at lower layer, like HARQ.

# References

- RP-213587, “Study on XR Enhancements for NR””, RANP #94e, December 2021

- TR38.838,

- SP-210043, SA4

- SP-220705, SA2

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services