3GPP TSG RAN WG2#119-e	           R2-2207865

Online, 17th – 29th August 2022

Agenda item:	8.2.2

Source: 	Lenovo

Title: 	On SL Positioning Architecture and Procedures

Document for:	Discussion

# Introduction

The Rel-18 NR positioning evolution SID was agreed upon during the RAN#94-e [1] meeting, where one of the objectives included the potential solutions to enable SL Positioning. The following highlighted SID objectives were outlined to support the discussion:

SL Positioning SID Objectives: - Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution provides a detailed discussion into the potential areas to develop in order to enable a SL Positioning framework.

# Types of SL Positioning/Ranging Location Requests

In the context of Uu positioning, according to TS 23.273, different types of location requests were defined based on the type of positioning application/service including Network Induced Location Request (NI-LR), Mobile Terminated Location Request (MT-LR), Mobile Originated Location Request (MO-LR), Immediate Location Request and Deferred Location Request. Each of these requests are able support different service and requirements.

In a similar manner, RAN2 can derive the type of positioning models, e.g., UE-based and/or UE-assisted approaches based on the type of supported SL Positioning/Ranging location requests.

Proposal 1: RAN2 to coordinate with SA2 on the supported types of SL Positioning/ranging requests, which is related to the triggering of a SL positioning/session.

# SL Positioning Models

The traditional 3GPP Uu positioning framework consists of 2 models, whereby the position estimate of a UE is computed, which includes:

- Uu Model 1 - UE-assisted positioning: The configuration and positioning calculation entity is the LMF

- Uu Model 2 - UE-based positioning: The configuration entity is the LMF, while the positioning calculation entity is the UE.

These aforementioned models only catered to in-coverage UEs, with the involvement of the NG-RAN, AMF and LMF.

Considering the distributed nature of SL communications as well as additional partial coverage and out of coverage scenarios to be supported for SL positioning, it is important to distinguish the difference between a configuration entity and positioning calculation entity.

The configuration entity may configure the assistance data (including time-frequency resources in which to measure the SL positioning measurements), e.g., LMF, gNB, RSU, target-UE, anchor UE, etc. On the other hand, the positioning calculation entity is responsible for SL measurement requests as well as the reception of the SL measurements and finally the final computation of the location estimate, e.g., LMF, gNB, RSU, target-UE, anchor UE, etc. The entities(s) involved may also vary depending on the type of coverage scenario.

Observation 1: In SL Positioning, it is important to distinguish between the configuration and positioning calculation entity depending on the coverage scenario.

Therefore, the following 4 SL positioning models are to be considered during the study:

- SL Positioning Model 1a - UE-assisted positioning: The configuration entity is a UE, while the positioning calculation entity is a network node comprising either of a LMF or gNB.

- SL Positioning Model 1b – UE-assisted positioning: The configuration entity and positioning calculation entity is a network node comprising either of the LMF or gNB

- SL Positioning Model 2a – UE-based positioning: The configuration entity and positioning calculation entity is a UE.

- SL Positioning Model 2b - UE-based positioning: The configuration entity is a network node comprising either of a LMF or gNB, while the positioning calculation entity is a UE.

The above models mentioned ensure that SL positioning is supported in in-coverage, partial coverage as well as out-of-coverage scenarios.

Proposal 2: Support the following 4 different models for SL positioning, which distinguish the configuration entity and positioning calculation entity for in-coverage, and partial and out-of-coverage scenarios:

- SL Positioning Model 1a - UE-assisted positioning (UE-configured)

- SL Positioning Model 1b – UE-assisted positioning (network-configured)

- SL Positioning Model 2a – UE-based positioning (UE-configured)

- SL Positioning Model 2b - UE-based positioning (network-configured)

# SL Positioning Protocol and Architecture

## Architecture

The Uu positioning architecture is primarily based on the involvement of the LMF and as such can only operate in-coverage scenarios with a network deployment supporting an LMF. In the case of SL positioning, the use cases span across different vertical applications (e.g., V2X, Public Safety, IIoT and Commercial ranging), which have different requirements in terms of coverage scenario operation. The proposed SL positioning architecture should also support partial and out-of-coverage scenarios. This aspect was extensively studied during the RAN study in TR 38.845 [2], where the SL positioning framework should cater to all coverage scenarios.

Observation 2: The designed SL Positioning architecture should cater to all use cases including V2X, Public Safety, IIoT and Commercial ranging across all coverage scenario, i.e., in-coverage, partial coverage and out-of-coverage.

Furthermore, due to the distributed nature of SL positioning there are multiple entities which may act as configuration and positioning calculation entities, e.g., RSU, gNB, target-UE, anchor-UE, other assistance UEs, positioning reference units, etc, which is different from the Uu operation where the LMF was responsible for both positioning configuration and positioning calculation. It is, therefore, important to consider the involvement of such entities within the SL positioning architecture.

Observation 3: Aside from the LMF, other entities which may be involved in a SL positioning session may include RSU, gNB, target-UE, anchor-UE, other assistance UEs, positioning reference units, etc.

The following architecture options may be further studied by RAN2 (also in consultation with SA2):

| Architecture Options | High-level Advantages | High-level Challenges |
| --- | --- | --- |
| Option 1: LMF dependent SL positioning architecture | - For in-coverage UEs performing SL positioning/ranging the LMF may be involved and can leverage the re-use of the existing Uu positioning framework (LPP). - Can be leveraged for improving positioning in the case Uu and SL positioning measurements are required. | - A standalone SL positioning architecture for partial coverage and out-of-coverage UEs may be required -For SL positioning/ranging, LMF involvement may increase positioning latency due to configuration since gNB performs resource allocation for SL communications |
| Option 2: LMF independent SL positioning architecture (PC5-only architecture) | - May not require a UE to support LPP to support SL Positioning UE - SL positioning/ranging can be independent of whether a network deployment supports an LMF or not. - Same SL positioning architecture and procedures can be used across all coverage scenarios. | - Additional coordination will be required for configuration, measurement & reporting of both Uu and SL positioning measurements to improve both Uu positioning accuracy and SL positioning accuracy for in-coverage scenarios. |

Proposal 3: RAN2 to further study the feasibility of both LMF-dependent and LMF-independent SL positioning architectures. SA2 coordination may be required.

## Protocol Design

LPP serves as point-to-point positioning protocol or UE-terminated protocol between a location server (E-SMLC, LMF or SLP) and a target device (UE or SET) in order to position the target device using position-related measurements obtained by one or more reference sources, which may include GNSS (RAT-independent methods) or NG-RAN node for RAT-dependent methods. LPP covers the initiation or maintenance or termination of one or more LPP sessions, management of different transactions and other aspects to enable different location procedures. Normally, the NAS layer (control plane) or OMA SUPL UPL (user-plane) may be utilised to transfer such LPP messages. The protocol layering is illustrated in Figure ??.

Figure 1: LPP Protocol Layer on the Uu interface [TS 38.305]

Observation 4: LPP offers the framework to exchange messages required to enable Uu positioning.

However, on the SL or PC5 interface the operations have thus far catered towards SL communication and require additional feasibility studies for enabling SL positioning/ranging. Two protocol stacks for PC5 may be distinguished on the control plane:

Figure 2: C-Plane SL Protocol Stack [TS38.300]

Further RAN2 studies are required in the context of a new or existing protocol design with respect to SL positioning/ranging satisfying the following basic functionality:

- Support initiation/maintenance/termination of a SL positioning session

- Distinguish between multiple SL positioning/ranging sessions between one or more UEs/network entities

- Support groupcast and unicast SL positioning/ranging

- A request/response pairing of SL positioning/ranging messages or enable one or more "unsolicited" SL positioning/ranging messages

- Transfer of capabilities, SL positioning configurations (including resource configurations), positioning-related reports or positioning estimates based on a configured SL positioning QoS and one or more positioning techniques.

- Enable parallel and multiple procedures to allow for one or more positioning attempts of the same target-UE.

- Secure transfer of all SL positioning/ranging messages.

Proposal 4: RAN2 to further study the feasibility of introducing a new protocol layer on the SL or re-use existing functionality to support the at least the following SL positioning/ranging functionality including SL positioning/ranging session management (e.g., initiation/maintenance/termination of a SL positioning session), supporting different positioning cast types, request/response pairing of SL positioning/ranging messages, and message transfer of capabilities, SL positioning configurations (including resource configurations), positioning-related reports, etc. FFS further other aspects for support.

# SL Positioning Techniques

## SL RAT-dependent Positioning techniques

The potential SL positioning/ranging techniques should be able to support different scenarios, especially given that anchor nodes (whose location is usually known) and non-anchor nodes (whose location is usually unknown or only coarsely known) can be considered given the distribute nature of SL communications. The SL RAT-dependent positioning techniques to be supported may be summarized as follows:

- SL timing-based positioning methods

  - SL-TDoA

  - SL-RTT (including one-way and two-way RTT methods)

- SL angular-based positioning methods

  - SL-AoD

  - SL-AoA

- SL Location fingerprinting methods

  - Exploiting RRM measurements such as SL RS RSRP/RSRQ measurements

Furthermore, during RAN1#109-e [3], RAN1 had already made some progress towards this aspect based on the following agreements:

RAN1#109-e Agreement With regards to the Positioning methods supported using SL measurements study further the following methods: - RTT-type solutions using SL - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT - SL-AoA - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study - SL-TDOA - SL-AoD - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu. - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study - Consider in the study at least the following aspects: - Definition(s) of the corresponding SL measurements for each method - Which method is applicable to absolute or relative positioning or ranging, including whether such categorization is needed to be discussed. - For angle-based methods, antenna configuration consideration(s) using practical UE capabilities - Per-panel location, if UE uses multiple panels. - UE’s mobility, especially for V2X scenarios - Impact of synchronization error(s) between UEs - Existing SL measurements (e.g. RSSI, RSRP), and UE ID information etc, may be used. - Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified, or whether there will be a unified SL Positioning method. - Note: When the study of carrier phase positioning and the evaluations of sidelink positioning have progressed, it can be reviewed whether carrier phase for sidelink can be considered in further work. Checkpoint at RAN1#110-e-Bis to see if sufficient information is available for this review. Note: Companies are encouraged to describe the role of SL nodes and their interaction/coordination participating in each method.

As such RAN2 should coordinate with RAN1 on the supported positioning techniques by awaiting further feedback on the feasibility of the different timing-based and angle-based SL positioning techniques.

Proposal 5: RAN2 to coordinate with RAN1 on the study of the supported SL timing-based and angle-based RAT-dependent positioning techniques.

### Anchor UEs/RSUs

The positioning requirements agreed in RAN1#109-e [3], target both absolute and relative positioning requirements. For each of these cases, it is expected that anchor UEs with relative stable mobility patterns and known UE locations, may be configured to participate in SL positioning/ranging session. This is especially required for positioning techniques requiring 2 or more anchor nodes for enhanced positioning performance.

Observation 5: Anchor nodes with known locations such as UEs and RSUs can enable certain SL positioning techniques and in other cases improve positioning performance.

One of the key responsibilities of anchor nodes, would be transmit/receive SL-PRS, perform SL positioning measurements and reporting, as well as any other key positioning-related tasks. In the case of anchor UEs, there are additional issues worth studying including the selection, triggering and configuration of anchor nodes to perform the aforementioned tasks. And the solutions for anchor nodes issues should be studied for both LMF dependent SL positioning architecture and LMF-independent SL positioning architecture as defined in Section 4, to support in coverage, partial coverage and out of coverage scenario. For LMF dependent SL positioning architecture, LMF may need to perform management to the anchor nodes in addition to legacy target UE, while for LMF-independent SL positioning architecture, the anchor nodes management (e.g., include (re)selection, triggering and configuration) may controlled by gNB if in-coverage or determined by Initiator UE that starting/initiating the SL positioning session, the coordination procedure need to be studied.

Proposal 6: RAN2 is suggested to investigate the procedures related to anchor UE(s) with known locations including (re)selection, triggering and configuration when performing sidelink positioning procedures.

## Hybrid Positioning

It also well established that no single positioning technique can satisfy all the target positioning requirements, especially considering different use case have varying positioning requirements. The SL Positioning framework should be able to support and leverage hybrid positioning in order extract the best gains from different interfaces, positioning techniques as well as 3GPP/non-3GPP technologies. Accordingly, we define hybrid positioning based on two categories:

- Hybrid Positioning Model A (using hybrid interfaces) – This hybrid positioning model leverages measurements from both Uu and SL interface to aid and enhance either the Uu positioning estimate (based on the LPP framework) or SL positioning estimate.

- Hybrid Positioning Model B (using hybrid positioning technologies) - This hybrid positioning model exploits the position computation using a combination of RAT-dependent and RAT-independent methods (e.g., GNSS, WLAN, Bluetooth, IMU,etc.). Since RAT-independent methods have already been supported within Uu positioning framework, it can be also straight-forward to enable SL positioning to re-use existing RAT-independent methods without significant additional work.

Proposal 7: RAN2 to further study the support of hybrid positioning under the following models:

- Hybrid Positioning Model A - Hybrid positioning using hybrid interfaces including the support of Uu and SL measurements for SL position calculation.

- Hybrid Positioning Model B - Hybrid positioning using hybrid technologies including the support of RAT-dependent and RAT-independent methods for SL position calculation.

# SL Positioning Configuration and Resource Allocation

## General

In the context of the Uu positioning, the LMF is a key central entity coordinating the PRS/SRS resource configuration with the serving and neighbouring gNBs via the NRPPa interface, while in SL the gNB is responsible for the configuration and resource allocation of all SL UEs, while in-coverage. In order to avoid additional spec. efforts, it is recommended that SL positioning resource allocation mechanism reuse many of the principles that already exist to enable SL communications.

Observation 6: In Uu positioning, the LMF coordinated the transmission configuration of PRS/SRS, while for in-coverage SL UEs, the gNB is the key configuration entity.

It should be possible to extend the current Mode 1 and Mode 2 resource allocation schemes in SL to configure SL PRS in all coverage scenarios, i.e., in-coverage, partial coverage and out-of-coverage. Mode 1 offers a centralized approach to configure SL PRS for in-coverage and partial coverage scenarios, while Mode 2 enables SL PRS configuration by UEs in out-of-coverage scenarios.

Furthermore, during RAN1#109-e [2], related agreement on the following resource allocation issues were made:

RAN1#109-e Agreement With regards to the SL-PRS resource allocation, study the following two schemes: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS - Applicable regardless of the network coverage - FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting UEs (e.g., IUC-like solutions). - Note: Other Schemes are not precluded to be studied - FFS how to handle resource allocation of SL-Positioning measurement report

Proposal 8: SL positioning to re-use some of the existing SL functionality related configuration and resource allocation.

Furthermore, it is yet to be determined whether dedicated or shared resource pools will be configured for SL positioning and is currently under RAN1 discussion:

RAN1#109-e Agreement With regards to the SL Positioning resource allocation, study further the following 2 options for SL Positioning resource (pre-)configuration: - Option 1: Dedicated resource pool for SL-PRS - Include in the study at least the following aspects: - which slots can be used, SL frame structure, SL positioning slot structure, multiplexing of SL-PRS with control information (if included in the same slot) - positioning measurement report - whether a dedicated frequency allocation (e.g., layer/BWP) is needed for SL PRS - resource allocation procedure(s) of SL-PRS - This option may or may not include control information (i.e., configuration/activation/deactivation/triggering of SL-PRS) for the purpose of SL positioning operation - Option 2: Shared resource pool with sidelink communication. - Include in the study at least the following aspects: - co-existence between SL communication and SL positioning, backward compatibility - Multiplexing considerations of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH) and any modifications in the SL-slot structure

Observation 7: Discussions on SL positioning resource pool structure are already in progress in RAN1

## Triggering

The SL PRS transmission(s) can be configured in one of the following ways dynamically, semi-statically or event-triggered. For different coverage scenarios, the configuration and triggering procedure needs to be considered. The triggering may occur from the gNB/UE/LMF side and/or from higher layer signalling. It is expected that the higher-layer may receive a ranging service request, e.g., from the Application layer and further procedures are required to be studied. A further aspect is the alignment and handling of the SL positioning QoS in order for RAN to meet the service for a SL positioning/ranging service in terms of accuracy, latency, reliability, etc.

Proposal 9: RAN2 further study the triggering mechanism and alignment of SL Positioning/ranging QoS for RAN-based procedures to meet the service requirements, since this may be different from the existing SL communication mechanism.

## Configuration

The SL positioning configuration may comprise of all necessary parameters required to support a SL positioning/ranging session. This may include not only time -frequency resources or other physical-layer parameters, but also the type of configured positioning techniques, selected and configured anchor nodes, QoS information, and additional assistance information that may assist the configuration node with the SL positioning configuration.  Figure 3 is a general conceptual overview of the SL positioning configuration, measurement and reporting flow for SL positioning.

Figure 3: SL positioning configuration, measurement and reporting flow for SL positioning

Furthermore, Figure 3 may also align with different positioning models outlined in Section 3.

Proposal 10: RAN2 to further study the transmission of the SL positioning configuration depending on the type of configure node and SL positioning mode (e.g., UE-based or UE-assisted), e.g., gNB, LMF, RSU/Anchor-UE, Target-UE by considering at least:

- UE-specific (via dedicated signalling) or common (via broadcast/groupcast signalling, e.g., SIB) SL positioning configurations.

- UE assistance information to assist the configuration node, e.g., gNB, LMF, UE in supporting SL positioning procedures.

## Mode 1

According to this resource allocation mode, the gNB may configure the SL PRS resources in a centralised manner depending on the configured positioning technique. For techniques requiring relative position estimation amongst multiple pair of UEs (e.g., between an anchor node UE and target-UE), gNB-based resource coordination should be sufficient. The gNB may also coordinate SL PRS for multiple anchor node UEs when performing triangulation using timing-based methods such as SL-TDoA or angular-based position estimation, for the purposes of absolute location estimation.

In addition, depending on the type of location request the gNB may configure one of the following:

- Dynamic grants for one shot SL positioning estimates (based on one shot SL PRS transmissions) (e.g., in the case of immediately triggered requests)

- Configured grant schemes in the case that periodic location requests are required, e.g., Type 1 and 2 CG grants (based on periodic SL PRS transmissions).

Proposal 11: RAN2 to further study Mode 1 coordination of SL PRS resources for one or more UEs participating in a SL positioning session (e.g., one or more anchor UEs and a target-UE).

## Mode 2

Mode 2 is beneficial for scenarios not requiring network infrastructure, where the Initiator UE (UE starting/initiating the SL positioning session) may autonomously select and reserve resource for the purposes of SL positioning. This relies on a pre-configuration of resource pools within a UE. Given the distributed nature of this scheme, Mode 2 can sure stability of SL PRS for a given period by selecting and reserving resources for SL PRS transmission. However, this may depend on the type of SL PRS configuration (e.g., bandwidth, number of symbols, etc.).

The selection triggers may be different in the case of SL PRS when compared to the existing schemes for SL data (TB) transmission, where the selection is dependent on the generation of a new TB or a pre-emption mechanism depending on the SL traffic priority. Similarly, in the case SL Positioning, the QoS may also have an impact on the resource selection mechanism in Mode 2. The sensing window should also be able to accommodate a SL PRS transmission based on the accuracy requirements of the SL positioning service request.

Proposal 12: RAN2 to further study Mode 2 coordination of SL PRS resources for one or more UEs participating in a SL positioning session e.g., one or more anchor UEs and a target-UE) including at least:

- The use of pre-configured resources in out-of-coverage scenarios.

- Initiator UE (e.g., target UE) indicates the SL-PRS resource to other UEs involved in SL Pos (e.g., one or more anchor UEs) over sidelink.

# SL Measurement and Reporting

SL positioning measurements can be performed by the target-UE in a similar fashion to the current Uu positioning framework. The target-UE is usually configured separately to report certain measurements based on the positioning method and can also be extended to the SL positioning framework. In the context of relative positioning, it can be expected that the SL positioning measurements may be performed either by the Initiating or Responding UEs, depending on the type of configured positioning methods, e.g., the two-way SL-RTT requires that both the Initiator and Responder UE perform the timestamp measurements for enhanced accuracy.

It can be noted that depending on the scenario, one or multiple entities may have different responsibilities in terms of transmitting the SL positioning configuration, performing measurements, reporting (if required, e.g., UE-based methods would require optional reporting) and finally the position calculation. It can be further observed that the measurement node/entity may receive the SL-PRS transmissions and then perform the associated SL positioning measurements.

Proposal 13: RAN2 to consider the supported signalling and procedures depending on which node/entity:

- Performs the SL positioning measurements and associated reporting (e.g., if needed for UE-based positioning), e.g., RSU/Anchor-UE, Target-UE

- Performs the SL positioning calculation e.g., gNB, LMF, RSU/Anchor-UE, Target-UE.

NOTE: This has a dependency on the SL Positioning architecture as well as positioning models (e.g., UE-assisted, UE-based) described in Section 3.

The SL Positioning framework should also be capable of supporting different types of reporting of both SL positioning measurements and absolute/relative location estimate. Table 2 shows the various reporting types depending on the reporting configuration.

Table 2: Types of SL Positioning Reporting

| SL Positioning Reporting Types | Overview |
| --- | --- |
| One-shot | - The UE/device performing measurements may report to the positioning calculation entity in a one-shot manner to support immediate reporting. - The UE (positioning calculation) entity may report the absolute/relative location estimate to the requesting node/network entity, e.g., UE, LMF, etc. |
| Triggered | - The UE/device performing measurements may report to the positioning calculation entity based on defined event-based criteria, e.g., based on a location change/cell change. - The UE/device performing measurements may report to the positioning calculation entity upon trigger of a configured event. |
| Periodic | - The UE/device may provide periodic positioning measurement or location estimate reports with the same time interval between two consecutive reports. |

Proposal 14: Support different SL Positioning reporting types including one-shot, triggered and periodic reports.

It is also important to assess the quality of the SL PRS measurements, which were performed, e.g., in the form of confidence intervals relating to a specific measurement, predefined margins of error, or other range reliability metrics. This can be applied to timing, angle, distance/ranging, direction/orientation estimates. This serves as additional assistance information for the positioning calculation entity.

Proposal 15: Measurement quality metric reporting should also be supported to assess quality of SL positioning measurements.

# Conclusion

The following observations regarding potential SL positioning solutions have been noted as part of the discussion:

Observation 1: In SL Positioning, it is important to distinguish between the configuration and positioning calculation entity depending on the coverage scenario.

Observation 2: The designed SL Positioning architecture should cater to all use cases including V2X, Public Safety, IIoT and Commercial ranging across all coverage scenario, i.e., in-coverage, partial coverage and out-of-coverage.

Observation 3: Aside from the LMF, other entities which may be involved in a SL positioning session may include RSU, gNB, target-UE, anchor-UE, other assistance UEs, positioning reference units, etc.

Observation 4: LPP offers the framework to exchange messages required to enable Uu positioning.

Observation 5: Anchor nodes with known locations such as UEs and RSUs can enable certain SL positioning techniques and in other cases improve positioning performance.

Observation 6: In Uu positioning, the LMF coordinated the transmission configuration of PRS/SRS, while for in-coverage SL UEs, the gNB is the key configuration entity.

Observation 7: Discussions on SL positioning resource pool structure are already in progress in RAN1.

The proposals relating to the SL Positioning framework are summarized as follows:

Proposal 1: RAN2 to coordinate with SA2 on the supported types of SL Positioning/ranging requests, which is related to the triggering of a SL positioning/session.

Proposal 2: Support the following 4 different models for SL positioning, which distinguish the configuration entity and positioning calculation entity for in-coverage, and partial and out-of-coverage scenarios:

- SL Positioning Model 1a - UE-assisted positioning (UE-configured)

- SL Positioning Model 1b – UE-assisted positioning (network-configured)

- SL Positioning Model 2a – UE-based positioning (UE-configured)

- SL Positioning Model 2b - UE-based positioning (network-configured)

Proposal 3: RAN2 to further study the feasibility of both LMF-dependent and LMF-independent SL positioning architectures. SA2 coordination may be required.

Proposal 4: RAN2 to further study the feasibility of introducing a new protocol layer on the SL or re-use existing functionality to support the at least the following SL positioning/ranging functionality including SL positioning/ranging session management (e.g., initiation/maintenance/termination of a SL positioning session), supporting different positioning cast types, request/response pairing of SL positioning/ranging messages, and message transfer of capabilities, SL positioning configurations (including resource configurations), positioning-related reports, etc. FFS further other aspects for support.

Proposal 5: RAN2 to coordinate with RAN1 on the study of the supported SL timing-based and angle-based RAT-dependent positioning techniques.

Proposal 6: RAN2 is suggested to investigate the procedures related to anchor UE(s) with known locations including (re)selection, triggering and configuration when performing sidelink positioning procedures.

Proposal 7: RAN2 to further study the support of hybrid positioning under the following models:

- Hybrid Positioning Model A - Hybrid positioning using hybrid interfaces including the support of Uu and SL measurements for SL position calculation.

- Hybrid Positioning Model B - Hybrid positioning using hybrid technologies including the support of RAT-dependent and RAT-independent methods for SL position calculation.

Proposal 8: SL positioning to re-use some of the existing SL functionality related configuration and resource allocation.

Proposal 9: RAN2 further study the triggering mechanism and alignment of SL Positioning/ranging QoS for RAN-based procedures to meet the service requirements, since this may be different from the existing SL communication mechanism.

Proposal 10: RAN2 to further study the transmission of the SL positioning configuration depending on the type of configure node and SL positioning mode (e.g., UE-based or UE-assisted), e.g., gNB, LMF, RSU/Anchor-UE, Target-UE by considering at least:

- UE-specific (via dedicated signalling) or common (via broadcast/groupcast signalling, e.g., SIB) SL positioning configurations.

- UE assistance information to assist the configuration node, e.g., gNB, LMF, UE in supporting SL positioning procedures.

Proposal 11: RAN2 to further study Mode 1 coordination of SL PRS resources for one or more UEs participating in a SL positioning session (e.g., one or more anchor UEs and a target-UE).

Proposal 12: RAN2 to further study Mode 2 coordination of SL PRS resources for one or more UEs participating in a SL positioning session e.g., one or more anchor UEs and a target-UE) including at least:

- The use of pre-configured resources in out-of-coverage scenarios.

- Initiator UE (e.g., target UE) indicates the SL-PRS resource to other UEs involved in SL Pos (e.g., one or more anchor UEs) over sidelink.

Proposal 13: RAN2 to consider the supported signalling and procedures depending on which node/entity:

- Performs the SL positioning measurements and associated reporting (e.g., if needed for UE-based positioning), e.g., RSU/Anchor-UE, Target-UE

- Performs the SL positioning calculation e.g., gNB, LMF, RSU/Anchor-UE, Target-UE.

NOTE: This has a dependency on the SL Positioning architecture as well as positioning models, (e.g., UE-assisted, UE-based) described in Section 3.

Proposal 14: Support different SL Positioning reporting types including one-shot, triggered, and periodic reports.

Proposal 15: Measurement quality metric reporting should also be supported to assess quality of SL positioning measurements.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RAN#94-e, Dec. 2021.

- 3GPP TR 38.845, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases (Release 17), V2.0.0, September 2021.

- R1-2205574, “Session notes for 9.5 (Study on expanded and improved NR positioning)”, Ad-Hoc Chair (Huawei), RAN1#109-e, May 2022