3GPP TSG-RAN WG2 Meeting #119-e	R2-2207887

# Electronic meeting, August 17 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1346 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL DRX behaviour for unicast link establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-07-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following agreements have been reached w.r.t DRX behaviour during unicast establishment procedure. According to #117-e agreement: “For messages delivery after PC5-S DCR message until and including PC5-RRC RRCReconfigurationSidelink message including initial DRX configuration, UE remains in active. FFS on PC5-RRC RRCReconfigurationSidelinkComplete.” #118-e agreement: “TX UE remains active for RRC reconfiguration complete/failure sidelink reception (only for initial RRC reconfiguration sidelink case). If TX UE already applies SL DRX configuration in the direction (RX UE -> TX UE), TX UE follows the current SL DRX configuration.” The above agreed behaviour has not been covered by the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added in section 5.28.2, that ActiveTime includes the time after the transmission of PC5-S Direct Communication Request message and the reception of RRCReconfigurationCompleteSidelink or RRCReconfigurationFailureSidelink message during unicast establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MAC specification is not correct and it may happen that Rx UE is not Active, e.g. monitoring for PSCCH/PSSCH, and able to receive PC5-S/RRC messages during the unicast link establishment procedure and initial DRX configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 5.28	Sidelink Discontinuous Reception (DRX)

### 5.28.1	General

The MAC entity may be configured by RRC with an SL DRX functionality that controls the UE's SCI (i.e., 1st stage SCI and 2nd stage SCI) monitoring activity for unicast, groupcast and broadcast. When using SL DRX operation, the MAC entity shall also monitor SCI (i.e., 1st stage SCI and 2nd stage SCI) according to requirements found in other clauses of this specification.

RRC controls Sidelink DRX operation by configuring the following parameters:

-	sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer: the duration at the beginning of an SL DRX cycle;

-	sl-drx-SlotOffset: the delay before starting the sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer;

-	sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer (except for the broadcast transmission): the duration after the first slot of SCI (i.e., 1st stage SCI and 2nd stage SCI) reception in which an SCI indicates a new SL transmission for the MAC entity;

-	sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer (per Sidelink process except for the broadcast transmission): the maximum duration until an SL retransmission is received;

-	sl-drx-StartOffset: the slot where the SL DRX cycle starts;

-	sl-drx-Cycle/sl-DRX-GC-BC-Cycle: the Sidelink DRX cycle;

-	sl-drx-HARQ-RTT-Timer (per Sidelink process except for the broadcast transmission): the minimum duration before an SL HARQ retransmission is expected by the MAC entity.

### 5.28.2	Behaviour of UE receiving SL-SCH Data

When SL DRX is configured, the Active Time includes the time while:

-	sl-drx-onDurationTimer or sl-drx-InactivityTimer is running; or

-	sl-drx-RetransmissionTimer is running; or

-	period of sl-LatencyBoundCSI-Report configured by RRC in case SL-CSI reporting MAC CE is not received; or

-	the time between the transmission of the request of SL-CSI reporting and the reception of the SL-CSI reporting MAC CE in case SL-CSI reporting MAC CE is received; or

-	Slot associated with the announced periodic transmissions by the UE transmitting SL-SCH Data.

-	the time after the transmission of PC5-S Direct Communication Request message and the reception of RRCReconfigurationCompleteSidelink or RRCReconfigurationFailureSidelink message during unicast establishment.

When one or multiple SL DRX is configured, the MAC entity shall:

1>	if multiple SL DRX Cycles that are mapped with multiple SL-QoS-Profiles of a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:

2>	select sl-drx-Cycle whose length of the sl-drx-cycle is the shortest one among multiple SL DRX Cycles that are mapped with multiple SL-QoS-Profiles associated with the Destination Layer-2 ID:

2>	select sl-drx-onDurationTimer whose length of the sl-drx-onDurationTimer is the longest one among multiple SL DRX onDuration timers that are mapped with multiple SL-QoS-Profiles associated with the Destination Layer-2 ID.

1>	if an sl-drx-HARQ-RTT-Timer expires:

2>	if the data of the corresponding Sidelink process was not successfully decoded or if the HARQ feedback (i.e., negative acknowledgement) is not transmitted for unicast due to UL/SL prioritization:

3>	start the sl-drx-RetransmissionTimer for the corresponding Sidelink process in the first slot after the expiry of sl-drx-HARQ-RTT-Timer.

When the cast type is groupcast or broadcast as indicated by upper layer, the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations:

sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-drx-Cycle (ms).

sl-drx-SlotOffset (ms) = Destination Layer-2 ID modulo the number of slots in one subframe (ms).

1>	if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle) = sl-drx-StartOffset:

2>	start sl-drx-onDurationTimer after sl-drx-SlotOffset from the beginning of the subframe.

1>	if an SL DRX is in Active Time:

2>	monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI) in this SL DRX.

2>	if the SCI indicates a new SL transmission:

3>	if Source Layer-1 ID of the SCI is equal to the 8 LSB of the intended Destination Layer-2 ID and Destination Layer-1 ID of the SCI is equal to the 16 LSB of the intended Source Layer-2 ID and the cast type indicator in the SCI is set to unicast:

4>	start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair after the first slot of SCI reception.

3>	if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-1 ID and the cast type indicator in the SCI is set to groupcast:

4>	select sl-drx-InactivityTimer whose length of the sl-drx-InactivityTimer is the largest one among multiple SL DRX Inactivity timers that are mapped to multiple SL-QoS-Profiles of Destination Layer-2 ID associated with the Destination Layer-1 ID of the SCI; and

4>	start or restart sl-drx-InactivityTimer for the corresponding Destination Layer-2 ID after the first slot of SCI reception.

2>	if the SCI indicates an SL transmission:

3>	if a next retransmission opportunity is scheduled in the SCI:

4>	derive the sl-drx-HARQ-RTT-Timer from the retransmission resource timing of the next retransmission resource in the SCI.

3>	else:

4>	use the sl-drx-HARQ-RTT-Timer configured by upper layers.

3>	if PSFCH resource is not configured for the SL grant associated to the SCI:

4>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

3>	if PSFCH resource is configured for the SL grant associated to the SCI:

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to unicast; or

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and positive-negative acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization;

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and negative-only acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is a positive acknowledgement.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is not scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSFCH resource.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

NOTE:	Void.

3>	stop the sl-drx-RetransmissionTimer for the corresponding Sidelink process.

1>	if an SL DRX Command MAC CE is received for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast:

2>	stop sl-drx-onDurationTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast;

2> stop sl-drx-InactivityTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast.