3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207898

# Elbonia, 17 – 26 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1349 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the matching TB size for 2-step RA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2022-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following text is applied in the current specification text: 4> if the transport block size of the MSGA payload configured in the rach-ConfigDedicated corresponds to the transport block size of the MSGA payload associated with Random Access Preambles group B: where the “corresponds to” is intended to define “matching” size with the two transport blocks. However, “corresponds to” is not usually used to define “matching”, e.g., in terms of exact size and is hence unclear wording in this context. “Matches with” is used for other similar context in the MAC specification and should, hence, be aligned to also herein: “3> if the uplink grant size matches with size of the obtained MAC PDU; and” (…) 4> if the uplink grant size does not match with size of the obtained MAC PDU; and |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the “corresponds to” with “matches with” as used elsewhere in the MAC specification. Impact analysis Impacted functionality: 2-step Random Access. Inter-operability: - If the network is implemented according to the CR and the UE is not, no interoperability issues. - If the UE is implemented according to the CR and the network is not, no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The preamble group selection remains unclear how it should be done in csae of CFRA is configured for 2-step RA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2, 5.1.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.1.2	Random Access Resource selection

If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall:

1>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17); and

1>	if the beamFailureRecoveryTimer (in clause 5.17) is either running or not configured; and

1>	if the contention-free Random Access Resources for beam failure recovery request associated with any of the SSBs and/or CSI-RSs have been explicitly provided by RRC; and

1>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or the CSI-RSs with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the CSI-RSs in candidateBeamRSList is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the CSI-RSs in candidateBeamRSList;

2>	if CSI-RS is selected, and there is no ra-PreambleIndex associated with the selected CSI-RS:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the SSB in candidateBeamRSList which is quasi-colocated with the selected CSI-RS as specified in TS 38.214 [7].

2>	else:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by PDCCH; and

1>	if the ra-PreambleIndex is not 0b000000:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex;

2>	select the SSB signalled by PDCCH.

1>	else if the contention-free Random Access Resources associated with SSBs have been explicitly provided in rach-ConfigDedicated and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else if the contention-free Random Access Resources associated with CSI-RSs have been explicitly provided in rach-ConfigDedicated and at least one CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if the Random Access Resources for SI request have been explicitly provided by RRC:

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	select a Random Access Preamble corresponding to the selected SSB, from the Random Access Preamble(s) determined according to ra-PreambleStartIndex as specified in TS 38.331 [5];

2>	set the PREAMBLE_INDEX to selected Random Access Preamble.

1>	else (i.e. for the contention-based Random Access preamble selection):

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if the RA_TYPE is switched from 2-stepRA to 4-stepRA:

3>	if a Random Access Preambles group was selected during the current Random Access procedure:

4>	select the same group of Random Access Preambles as was selected for the 2-step RA type.

3>	else:

4>	if Random Access Preambles group B is configured; and

4>	if the transport block size of the MSGA payload configured in the rach-ConfigDedicated matches with the transport block size of the MSGA payload associated with Random Access Preambles group B:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

2>	else if Msg3 buffer is empty:

3>	if Random Access Preambles group B is configured:

4>	if the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group;

2>	set the PREAMBLE_INDEX to the selected Random Access Preamble.

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if ra-AssociationPeriodIndex and si-RequestPeriod are configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

1>	else if an SSB is selected above:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured or indicated by PDCCH (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB).

1>	else if a CSI-RS is selected above:

2>	if there is no contention-free Random Access Resource associated with the selected CSI-RS:

3>	determine the next available PRACH occasion from the PRACH occasions, permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, corresponding to the SSB in candidateBeamRSList which is quasi-colocated with the selected CSI-RS as specified in TS 38.214 [7] (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6], corresponding to the SSB which is quasi-colocated with the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the SSB which is quasi-colocated with the selected CSI-RS).

2>	else:

3>	determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers, corresponding to the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS).

1>	perform the Random Access Preamble transmission procedure (see clause 5.1.3).

NOTE 1:	When the UE determines if there is an SSB with SS-RSRP above rsrp-ThresholdSSB or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS, the UE uses the latest unfiltered L1-RSRP measurement.

NOTE 2:	Void.

Next Modified Subclause

### 5.1.2a	Random Access Resource selection for 2-step RA type

If the selected RA_TYPE is set to 2-stepRA, the MAC entity shall:

1>	if the contention-free 2-step RA type Resources associated with SSBs have been explicitly provided in rach-ConfigDedicated and at least one SSB with SS-RSRP above msgA-RSRP-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above msgA-RSRP-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else (i.e. for the contention-based Random Access Preamble selection):

2>	if at least one of the SSBs with SS-RSRP above msgA-RSRP-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above msgA-RSRP-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if contention-free Random Access Resources for 2-step RA type have not been configured and if Random Access Preambles group has not yet been selected during the current Random Access procedure:

3>	if Random Access Preambles group B for 2-step RA type is configured:

4>	if the potential MSGA payload size (UL data available for transmission plus MAC subheader and, where required, MAC CEs) is greater than the ra-MsgA-SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – msgA-PreambleReceivedTargetPower – msgA-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-MsgA-SizeGroupA:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

3>	else:

4>	select the Random Access Preambles group A.

2>	else if contention-free Random Access Resources for 2-step RA type have been configured and if Random Access Preambles group has not yet been selected during the current Random Access procedure:

3>	if Random Access Preambles group B for 2-step RA type is configured; and

3>	if the transport block size of the MSGA payload configured in the rach-ConfigDedicated matches with the transport block size of the MSGA payload associated with Random Access Preambles group B:

4>	select the Random Access Preambles group B.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Random Access preambles group has been selected during the current Random Access procedure):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the earlier transmission of MSGA.

2>	select a Random Access Preamble randomly with equal probability from the 2-step RA type Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group;

2>	set the PREAMBLE_INDEX to the selected Random Access Preamble.

1>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex if configured and ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability among the consecutive PRACH occasions allocated for 2-step RA type according to clause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB);

1>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

2>	select a PUSCH occasion from the PUSCH occasions configured in msgA-CFRA-PUSCH corresponding to the PRACH slot of the selected PRACH occasion, according to msgA-PUSCH-Resource-Index corresponding to the selected SSB;

2>	determine the UL grant and the associated HARQ information for the MSGA payload in the selected PUSCH occasion;

2>	deliver the UL grant and the associated HARQ information to the HARQ entity.

1>	else:

2>	select a PUSCH occasion corresponding to the selected preamble and PRACH occasion according to clause 8.1A of TS 38.213 [6];

2>	determine the UL grant for the MSGA payload according to the PUSCH configuration associated with the selected Random Access Preambles group and determine the associated HARQ information;

2>	if the selected preamble and PRACH occasion is mapped to a valid PUSCH occasion as specified in clause 8.1A of TS 38.213 [6]:

3>	deliver the UL grant and the associated HARQ information to the HARQ entity.

1>	perform the MSGA transmission procedure (see clause 5.1.3a).

NOTE:	To determine if there is an SSB with SS-RSRP above msgA-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

End of Changes