3GPP TSG-RAN WG2 #119 electronic	R2-2207910

Online, 17th August – 26th August, 2022

Source:		NEC

Agenda item:		8.4.3	NR-DC with selective activation cell of groups

Title:		Aspects to improve for the support of subsequent CPC

Document for:		Discussion and decision

# 1. Introduction

In Rel-17 Conditional PSCell change (CPC)/Conditional PSCell addition (CPA), a CPC/CPA-configured UE has to release the CPC/CPA configurations when completing random access towards the target PSCell. Hence the UE doesn’t have a chance to perform subsequent CPC/CPA without prior CPC/CPA reconfiguration and re-initialization from the network. This will increase the delay for the cell change and increase the signaling overhead, especially in the case of frequent SCG changes when operating FR2. Therefore, MR-DC with selective activation of cell groups aims at enabling subsequent CPC/CPA after SCG change, without reconfiguration and re-initialization on the CPC/CPA preparation from the network. This results in a reduction of the signalling overhead and interrupting time for SCG change.

Therefore, one new Rel-18 WI on further mobility enhancements was approved [1], and one of the objective is:

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

In this paper, we provide our view on the aspects to improve for CPC/CPA to support subsequent CPC.

# 2. Discussion

In Rel-17 Conditional PSCell change (CPC)/Conditional PSCell addition (CPA), a CPC/CPA-configured UE has to release the CPC/CPA configurations when completing random access towards the target PSCell. However, one typical scenario would be the UE moves around within the coverage of several micro gNBs and one macro gNB. One example scenario can be seem in Figure 1.

Figure 1 example scenario of NR-DC with selective activation cell of groups

To reduce signalling overhead and interrupting time for SCG/PSCell change, UE continues evaluating the conditional reconfiguration for SCG and subsequent CPC can performed based on previous CPC/CPA configurations which is not released after the previous PSCell change/addition procedure.

Considering that using full configuration is very low efficient in signalling overhead and lossless cannot be supported, for Rel-17 CPA/CPC, delta configuration is supported, and the use of delta configuration in T-SN PSCell preparation is up to NW implementation and no UE specific behaviour is specified. However in case of subsequent CPC, if UE follows the existing delta configuration behaviour, it may results in wrong SCG configuration or even reconfiguration failure. This is because at the network side the delta configuration is built on the basis of the original source SCG. However at the UE side the delta configuration will be applied to the latest source SCG configuration, while the latest SCG configuration changes during the multiple times of subsequent CPC procedure. To give an example, the delta configuration associated with candidate PSCell #1 and candidate PSCell #2 is built on a source SCG configuration. When UE performs CPC to PSCell #1, and then performs CPC to PSCell #2, at the UE side the delta configuration of PSCell #2 would applied to SCG configuration associated with PSCell #1. However at network side, the delta configuration of PSCell #2 should be applied to the source SCG configuration. This results misalignment of the configuration between UE and network side.

Proposal 1: RAN2 discuss whether and how to support delta configuration for subsequent CPC.

As the example scenario in the Figure, the UE moves back and forth within the coverage of several micro gNBs, it is possible that the UE changes to the same PSCell for more than one time. So this implies that the condition configuration for the same candidate PSCell is applied more than one time.

Proposal 2: UE can perform subsequent CPC to the same PSCell for more than one time.

Currently, to update security keys for SN, MN generates the KSN for the SN and sends it to the SN over the Xn-C. To generate the KSN, the MN associates a counter, called sk-Counter. And the KSN is generated based on the security key of MN and sk-Counter. The MN sends the value of the sk-counter to the UE over the RRCRconfiguration. According to TS 33.501 [2], the security of SN only depends on sk-Counter and Key of MN:

TS 33.501 A.16 Derivation of KSN for dual connectivity This input string is used when the MN and UE derive KSN during dual connectivity. The following input parameters shall be used: - FC =0x79, - P0 = Value of the SN Counter as a non-negative integer, - L0 = length of the SN Counter value (i.e. 0x00 0x02). The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

For subsequent CPC, we assume the stored secondary key configuration (i.e. sk-Counter) in CPC configuration is used. So for multiple times of subsequent CPC on the same candidate PSCell, if the same sk-Counter stored in CPC configuration is used, the same security key would be generated. This would results in security key reuse issue (i.e. cipher different packets with same security key and PDCP COUNT value, which is not allowed).

Observation: There would be security key reuse issue if UE uses the same sk-Counter in CPC configuration for multiple times of subsequent CPC to the same candidate PSCell.

Proposal 3: RAN2 asks SA3 to work on security key reuse avoidance in case of multiple times of subsequent CPC to the same candidate PSCell.

Additionally, currently the sk-counter should be monotonically increased by MN for each additional calculated KSN.

TS 33.501 The MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. The MN shall set the SN Counter to ‘1’ after the first calculated KSN, and monotonically increment it for each additional calculated KSN. The SN Counter value '0' is used to calculate the first KSN.

However, for the case of subsequent CPC, since the moving tack of the UE is random, it is not possible for the network to configure appropriate sk-Counter for each candidate PSCell to fulfil the monotonically increment principle. RAN2 should ask SA3 to discuss whether and how to fulfil the monotonically increment of sk-Counter for the support of subsequent CPC.

Proposal 4: RAN2 asks SA3 whether and how to fulfil the monotonically increment of sk-Counter requirement for the support of subsequent CPC.

# 3. Conclusion

In this paper, we provide our view on the aspects to improve for CPC/CPA to support subsequent CPC, and have the following proposals:

Proposal 1: RAN2 discuss whether and how to support delta configuration for subsequent CPC.

Proposal 2: UE can perform subsequent CPC to the same PSCell for more than one time.

Observation: There would be security key reuse issue if UE use the same sk-counter in CPC configuration for multiple times of subsequent CPC to the same candidate PSCell.

Proposal 3: RAN2 asks SA3 to work on security key reuse avoidance in case of multiple times of subsequent CPC to the same candidate PSCell.

Proposal 4: RAN2 asks SA3 whether and how to fulfil the monotonically increment of sk-Counter requirement for the support of subsequent CPC.

# Reference

[1] RP-221799, Revised WI: Mobility Enhancements

[2]TS 33.501, Security architecture and procedures for 5G system