3GPP TSG-RAN WG2 Meeting #119	  R2-2207925

e-Meeting, August, 17-29, 2022

Source: 	   Vodafone

Title: 		   NR support for UAV first steps plus Inter RAT aspects

Document for: Decision

Agenda Item:   8.8.2

# Introduction

A first Objective of a new WI in NR support for UAV is to introduction of UAV support is to:

1. Specify the following enhancements on measurement reports [RAN2]:

- UE-triggered measurement report based on configured height thresholds

- Reporting of height, location and speed in measurement report

- Flight path reporting

- Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously

Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

As per objective, it would be important to summarise what was introduced during LTE time and investigate if any changes for NR are needed.

# Discussion

UE-triggered measurement report based on configured height thresholds

In LTE, there are 2 events defined:

Event H1 (The Aerial UE height is above a threshold)

Event H2 (The Aerial UE height is below a threshold)

The condition to enter/leave these events are defined as:

Ms - Hys  > or < Thresh + Offset

- Ms is the Aerial UE height, not taking into account any offsets.

- Hys is the hysteresis parameter (i.e. h1-Hysteresis as defined within ReportConfigEUTRA) for this event.

- Thresh is the reference threshold parameter for this event given in MeasConfig(i.e. heightThreshRef as defined within MeasConfig).

- Offset is the offset value to heightThreshRef to obtain the absolute threshold for this event. (i.e. h1-ThresholdOffset as defined within ReportConfigEUTRA)

- Ms is expressed in meters.

- Thresh is expressed in the same unit as Ms.

Where:

- h1-ThresholdOffset-r15 INTEGER (0..300), where value 1 means 2 meters, value 2 means 4 meters and so on.

- h1-Hysteresis-r15 INTEGER (1..16)

- heightThreshRef-r15 is an INTEGER (0..31) Value 0 refers to -420m, value 1 refers to –120m, and so on until value 30 refers to 8880m. The actual value is height in meters relative to sea level. Value 31 is reserved

As per our observation most of the drones are expected to fly today at the height of around 100 meters above the ground as much higher height would result in lower number of detected cells. Please also 36.777 chapter H.5.3 .

Observation 1: We currently see no need to change the definition of H1 and H2 events and they could be used as they are defined today in LTE and be used on any height between -400 and 8880 meters.

Reporting of height, location, and speed in measurement report:

In LTE, the UE can report height within the range of INTEGER (-400..8880) with the accuracy of 1 meter.

Height UE:

Indicates height of the UE in meters relative to the sea level. Value 0 corresponds to sea level (i.e., negative value indicates depth of the UE below sea level). Value -400 corresponds to -400 m, value -399 corresponds to -399 m and so on.

Observation 2: We currently see no need to change the definition of the height UE as it can be reported with the accuracy of 1 meter, and this should also be sufficient once the UAV is connected to NR network.

Location Info:

The IE LocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information. This IE includes location information and vertical and horizontal velocity as defined in 36.355/37.335

Observation 3: The current definition provided in 37.335 seems to be sufficient, but they are coming from 37.335 LTE Positioning Protocol (LPP) and it would be advantageable to confirm with location experts that NR positioning data can be re-used 1 to 1 as in LTE or some additional elements would need to be reported.

Flight path reporting:

In LTE the NW is capable of requesting the UE flight path via UEInformationRequest including the fight path configuration. The UAV can include maximum 20 WayPoints and estimated arrival time

It might be discussed if the maximum number of WayPoints could be increased, but the current available procedure can be adapted in NR

Observation 4: We believe that the current procedure used for flight path reporting and configuration could be re-used. If a particular IEs such as number of WayPoints need to be enhanced may be discussed within the study item.

Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously

As it can be seen from 21.915: The RRM measurement framework was extended such that the UE can be configured to trigger a measurement report if an event condition is met for a configurable number of cells. Events applicable for this enhancement are A3, A4 and A5. These enhancements help the eNB to determine that a UE is flying and/or allow to detect that the UE may be causing or experiencing interference.

- In Release 15 3GPP added a new parameter numberOfTriggeringCells to the trigger configuration, in order to minimize the amount of trigger events.

Observation 5:  we believe that similar functionality is also of an advantage for the case UAV is connected over NR. It is probably even of a higher benefit, as the number of cell UAV could observe might be higher as the amount of spectrum available to 5G might grow fast.

Inter RAT Mobility:

With the introduction of 5G support for UAV, Inter RAT mobility for UAV should also be considered as it is very likely UAV would perform Inter RAT mobility and due to the nature of UAV, such mobility might be even more frequent. It should be studied if any existing mechanisms would need to be enhanced.

# Conclusion

Based on observations above, we would like to propose:

Proposal 1: It is proposed to re-use 1 to 1 events H1 and H2 defined in LTE and the height reporting within NR

Proposal 2: The accuracy of 1 m for the Hight information is sufficient and hight IE could be re-used in NR

Proposal 3: The definition of the Location Info seems to be sufficient in LTE, but the definition is often relating to LPP protocol 37.335 LTE Positioning Protocol (LPP) and it needs to be confirmed the location information can be re-used with NR.

Proposal 4: The current procedure used for flight path reporting and configuration could be re-used. It is proposed to study if the number of Way Points need to be enhanced.

Proposal 5: The functionality of the numberOfTriggeringCells should be introduced in NR. Also, a leaving conditions available in LTE should be introduced too.

Proposal 6: It is proposed to discuss if the enhancements to InterRAT mobility mechanism for UAV need to be done.