3GPP TSG-RAN WG2 Meeting #119	R2-2207926

e-Meeting, August, 17-29, 2022

Source: 	Vodafone

Title: 		First Steps for XR handling

Document for: 	Decision

Agenda Item: 	8.5.2

# Introduction

XR applications are becoming more and more important, and it can be assumed that the number of products proving virtual reality, augmented reality or cloud gaming services would increase.

- In Rel. 17, RAN WG 1 studied different traffic models for XR including virtual reality, augmented reality, or cloud gaming and carried out performance evaluations. The corresponding Study can be found in 38.838, where SA4 provided input for the traffic models for XR applications.

- In addition, there is work ongoing in SA2, e.g., in 23.700-60 Study on XR (Extended Reality) and media services, where enhancements e.g., for QoS characteristics are discussed, which might also be very important from RAN2 point of view.

- It is important to highlight that SA4 TR 26.928 where definitions of XR applications, key performance indicators and Quality-of-Experience metrics for relevant XR services are summarised.

In the new RAN WG2 Rel. 18 Study Item “New SID “Study on XR Enhancements for NR” the first objective of the study item “Study on XR Enhancements for NR” is to:

- Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of

- Study how the above information aids XR-specific traffic handling

The aim of this paper is to highlight certain aspects of such characteristics and suggests next steps how to proceed

# Discussion

In the current NR system, once a particular service is requested, the network is establishing a QoS flow corresponding to this service and the RAN (and UE) handle the packets according to the QoS characteristics requested by the Core Network.

Traffic with different QoSs characteristics is normally realised over different DRBs to enable different handling over the radio interface, even it is possible to multiplex traffic with similar QoS characteristics into the same DRB.

In this case, the strictest QoS characteristics would need to be satisfied within such a DRB which might lead to inefficient resource allocation in the system. For the uplink, it is also important to organise the mapping of the DRBs onto the Logical Channel Groups effectively.

The architecture of the PDU/5QI handling can be seen from 23.501 below:

Figure 5.7.1.5-1: The principle for classification and User Plane marking for QoS Flows and mapping to AN Resources

The main attributes on a particular QoSs/5QI today are:

- Resource type: Guaranteed Bit Rate or Non-Guaranteed Bit Rate

- Priority

- Packet Delay Budget

- Packet Error Rate

- Default Maximum Data Burst Volume

- Default Average Window

Once a QoS/5QI flow is established (NAS signalling and AS signalling), the characteristics and their values are provided to the gNB and packets belonging to the same QoS are treated according to the requested characteristics, but there is no differentiation per packet flow/stream within the same QFI/5QI/DRB.

As it can be seen from 38.838, the main traffic models analysed for XR may include multiple streams in uplink and downlink and those streams have different requirements in terms of packet delay budget, periodicity, Importance from application point of view regarding the perceptual quality.

It would also be important to highlight that allocating for different streams different QCI/5QI values, where single streams are related to within a particular XR application, might lead to different jitters as the mapping of different QCIs might lead to different treatments within transport network.

Observation 1: RAN WG2 should study enhancements allowing to handle traffic with different characteristics/streams received from the Core Network with the same 5QI value.

In our view there are several parameters which would be important to know to be able to handle packets within the same 5QI differently:

- Number of streams within a particular QCI/5QI Flow for UL and DL

- Frame Rate per flow in UL and DL

- Periodicity of the packets in UL and DL

- Packet Delay Budget in UL and DL

- Packet Error Rate in UL and DL

- Jitter requirements per stream for DL and possible UL

- Etc

Observation 2: Some of the characteristics might be (more) important for DL and some for UL or both as uplink and downlink data is often very different, e.g.:

- Case 1: video downlink: “pose data” uplink (i.e. speed of head /eye turn and direction of head)

- Case 2: AR, video uplink and a small bit of text downlink saying the name of the person in the video

Uplink Direction:

In uplink the UE should know which data belongs to which flow and could map the data accordingly. This knowledge probably could be provided within QoS rules as defined in 24.501

Once per flow configurations are provided from gNB to UE and there are specifications means to determine to which flow a corresponding packet belongs to, different handling of the packets by the gNB could be realised.

- Example 1:  If different flows have slightly different requirements in terms of PDB or PER, it might be useful to configure different times the UE is waiting for NACK (DRX-RetransmissionTimerUL) or any other related parameters.

- Example 2: The knowledge of the frame rate per flow in DL and UL together with the number of flows in UL and DL, might help to take a better decision regarding the configuration of the CDRX parameters.

- Different or combined configuration of Configured Grants (CGs) could be provided per flow. This might be especially important if the periodicity of the traffic per flow is different, so that the UE could use the available resources in the most optimal way taking the flow characteristics into account once using configured grant resources.

Observation 3: There is a need to know between gNB and UE to which flow the corresponding packet belongs to. Such means could be provided over MAC protocol.

Proposal 1: it is proposed to investigate methods for providing flow characteristics to the radio protocols at the UE for UL data transmission.

Proposal 2: It is proposed to study how MAC header could be used to signal the correspondence of a packet to a particular flow within one DRB.

Downlink Handling:

In case of the optimisations for XR in DL, the gNB needs to know the QCI/5QI characteristics per flow and be able to determine to which flow a particular packet belongs to.

SA2 is also studying now such potential enhancements and it is crucial to align with SA2 to understand which information might be expected e.g., which additional information e.g. in GTP-U headers could be provided and what are current agreements within SA2.

Observation 4: It is important to know what characteristics/methods are agreed in SA2 sofar, to allow the gNB to handle packets differently within the same 5QI.This would also allow to prioritise corresponding enhancements in RAN WG2.

It has also to be noted that SA4 is the group responsible for Multimedia Codecs, Systems and Services and it would be of great importance to receive their view on application related parameters which makes sense to be available to gNB and the reason for it.

Proposal 3: It is proposed to liaise SA2 asking for information which is agreed regarding the enhancements for QCI/5QI characteristics and possible enhancements to the transport protocols for user plane data.

Proposal 4: It is also proposed to contact SA4 to ask for the last enhancements regarding XR services and if there is an intention to provide those to the gNB.

# Conclusion:

This contribution addresses the characteristics of XR traffic and how to satisfy the QoS requirements of XR traffic transmission over the radio interface. The following observations and proposals are made.

Observation 1: RAN WG2 should study enhancements allowing to handle traffic with different characteristics/streams within the same DRB.

Observation 2: Some of the traffic characteristics might be important for DL and some for UL or both.

Observation 3: There is a need to know between gNB and UE to which flow within DRB the corresponding packet belongs to. Such means could be provided over MAC protocol.

Observation 4: Especially for the DL enhancements, but also for UL of XR traffic, it is important to know what characteristics/methods are agreed in SA2 sofar, to allow the gNB to handle packets differently within the same 5QI. This would also allow to prioritise corresponding enhancements in RAN WG2.

Proposal 1: it is proposed to investigate methods for providing flow characteristics to the radio protocols at the UE for UL data transmission

Proposal 2: It is proposed to study how MAC header could be used to signal the correspondence of the packet to a particular flow within the same DRB.

Proposal 3: It is proposed to liaise SA2 asking for information which is agreed regarding the enhancements for QCI/5QI characteristics and possible enhancements to protocols e.g. GTP-U for user plane data.

Proposal 4: It is also proposed to contact SA4 to ask for the last enhancements regarding XR services and if there is an intention to provide those to the gNB.

# References

[1] 38.383: Study on XR (Extended Reality) Evaluations for NR

[2] 23.700-60 Study on XR (Extended Reality) and media services

[3] 26.928: Technical Specification Group Services and System Aspects; Extended Reality (XR) in 5G

[4] 24.501 Non-Access-Stratum (NAS) protocol for 5G System (5GS)

[5] 23.501 System architecture for the 5G System (5GS)