3GPP TSG-RAN WG2 #119e	R2-2207927

Electronic meeting, 2022-08-17 - 2022-08-29

Agenda Item:	6.0.1

Source:	Ericsson

Title:	Corrections for Release-17 NTN RRC

Document for:	Discussion

# 1	Introduction

# 2	Discussion

## 2.1	The validity duration, ntn-UlSyncValidityDuration, is mandatory present in SIB19

In SIB19 field descriptions, one sub-field  of ntn-Config is specified to be mandatory present. However, typically these conditions are specified with ASN1 condition in the respective IE.

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch time. The validity duration, ntn-UlSyncValidityDuration, is mandatory present.

Hence, it is proposed to make the corresponding change in RAN2#119. The change is implemented in RRC correction CR for release 17-NTN in R2-22

Implement the mandatory presence of field ntn-UlSyncValidityDuration as Cond in ntn-Config instead of field description in SIB19.

## 2.2	On-demand delivery of SIB19

RAN2#118 agreed “On-demand SIB19 is not supported for UEs in RRC_CONNECTED state in NR NTN Rel-17”. Hence it is suggested to add:

dedicatedSystemInformationDelivery

This field is used to transfer SIB6, SIB7, SIB8, SIB19 to the UE with an active BWP with no common search space configured or the L2 U2N Remote UE in RRC_CONNECTED. For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is used to transfer the SIBs requested on-demand, except for SIB19.

Add “expect for SIB19” in field description of dedicatedSystemInformationDelivery in “For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is used to transfer the SIBs requested on-demand, except for SIB19.”

## 2.3	Field name hysteresis in condEventD1

The condEventD1 which should follow the definition of eventD1 has field name hysteresis when it should be hysteresisLocation as the corresponding field name in eventD1 is hysteresisLocation:

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17 INTEGER(0.. 65525),

distanceThreshFromReference2-r17 INTEGER(0.. 65525),

referenceLocation1-r17           ReferenceLocation-r17,

referenceLocation2-r17           ReferenceLocation-r17,

hysteresisLocation-r17                   HysteresisLocation-r17,

timeToTrigger-r17                TimeToTrigger

},

Hence it is suggested to update the field name.

Change the field name of hysteresis to hysteresisLocation in condeventD1

Adopt the NTN RRC CR in R2-2207924 as baseline for further NTN discussions.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Implement the mandatory presence of field ntn-UlSyncValidityDuration as Cond in ntn-Config instead of field description in SIB19.

Proposal 2	Add “expect for SIB19” in field description of dedicatedSystemInformationDelivery in “For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is used to transfer the SIBs requested on-demand, except for SIB19.”

Proposal 3	Change the field name of hysteresis to hysteresisLocation in condeventD1

Proposal 4	Adopt the NTN RRC CR in R2-2207924 as baseline for further NTN discussions.