3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2207955

Online, 17th – 29th August, 2022

Title: 	Discussion on MR-DC SCG failure

Source: 	Huawei, HiSilicon

Agenda item:	8.13.2

Document Type:	discussion

1. Introduction

According to the Rel-18 feSONMDT WID [1], the following objective is defined:

- Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback,

- Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

This paper is to address MRO for MR-DC SCG failure scenarios. In Rel-17, only intra-NR SCG failure report was introduced (i.e. for NR-DC), based on the Lses [2][3][4]. In this paper, we’ll give our initial analysis on the scenarios other than NR-DC.

# 2. Discussion

2.1 Background for Rel-17 discussions on MR-DC SCG failure

In Rel-17 SON and MDT enhancements, there were some Lses exchanged between RAN2 and RAN3, and here is a summary:

In the LS [2], RAN3 discussed the solution for the optimization of Pscell change failure for MRO in case of MR-DC, and then RAN3 suggest 5 parameters in the LS to RAN2.

In the LS [3], RAN2 requests RAN3 to confirm the MR-DC scenarios as RAN2 may not have enough time to work on all MR-DC scenarios

In the LS [4], RAN3 has agreed to “Prioritize NR-NR DC only”. And then RAN2 only defined necessary parameters for NR-DC SCG failure enhancement.

In general, MRO for NE-DC, EN-DC, and NG-EN-DC scenarios was initially proposed by RAN3, and RAN2 did not have enough time to progress on them. The possible enhancements can take the LS [1] into account.

2.2 Technical discussions for each MR-DC scenario

2.2.1 NE-DC

For NE-DC, if SCG failure happens, the SCGFailureInformationEUTRA message (defined in TS 38.331) is triggered. In the table below, we provide some analysis regarding the possible impacts.

| Parameters mentioned in the LS [2] | Possible impacts |
| --- | --- |
| 1) CGI of the Source PSCell: the source PSCell of the last SN change. The source PSCell could be E-UTRA cell or NR cell. | It should be about the cell information for LTE-5GC. |
| 2) CGI of the Failed PSCell: the PSCell in which SCG failure is detected or the target PSCell of the failed PScell change. The Failed PSCell could be E-UTRA cell or NR cell. | It should be about the cell information for LTE-5GC. |
| 3) timeSCGFailure: the time elapsed since the last PSCell change initialization until SCG failure. | Can reference timeSCGFailure-r17. |
| 4) connectionFailureType: radio link failure or SN change failure. | The existing field failureType can be re-used. |
| 5) random-access related information set by the PSCell | In LTE and NR, RACH reports have been defined, but the contents are different. In TS 36.331, RACH report has the following definitions: RACH-Report-r16 ::= SEQUENCE { numberOfPreamblesSent-r16 NumberOfPreamblesSent-r11, contentionDetected-r16 BOOLEAN } RACH-Report-v1610 ::= SEQUENCE { initialCEL-r16 INTEGER (0..3), edt-Fallback-r16 BOOLEAN } We think the existing LTE RACH report should be used as much as possible, and the details can be further discussed. |

Proposal 1: It is proposed to discuss MRO for NE-DC SCG failure scenario.

Proposal 2: It is proposed to discuss the following parameters for SCGFailureInformationEUTRA message defined in TS 38.331:

- CGI of the Source PSCell:	cell information for LTE-5GC

- CGI of the Failed PSCell:		cell information for LTE-5GC

- timeSCGFailure:		Can reference timeSCGFailure-r17

- connectionFailureType:		The existing field failureType can be re-used

- RA related info:		LTE RACH report should be used as much as possible

2.2.2 EN-DC/NG-EN-DC

For EN-DC/NG-EN-DC, if SCG failure happens, the SCGFailureInformationNR message (defined in TS 36.331) is triggered. In the table below, we provide some analysis regarding the possible impacts.

| Parameters mentioned in the LS [2] | Possible impacts |
| --- | --- |
| 1) CGI of the Source PSCell: the source PSCell of the last SN change. The source PSCell could be E-UTRA cell or NR cell. | Rel-17 definitions on NR-DC SCG failure can be referenced. |
| 2) CGI of the Failed PSCell: the PSCell in which SCG failure is detected or the target PSCell of the failed PScell change. The Failed PSCell could be E-UTRA cell or NR cell. | Rel-17 definitions on NR-DC SCG failure can be referenced. |
| 3) timeSCGFailure: the time elapsed since the last PSCell change initialization until SCG failure. | Rel-17 definitions on NR-DC SCG failure can be referenced. |
| 4) connectionFailureType: radio link failure or SN change failure. | The existing field failureType can be re-used. |
| 5) random-access related information set by the PSCell | Rel-17 definitions on NR-DC SCG failure can be referenced. |

Proposal 3: It is proposed to discuss MRO for EN-DC/NG-EN-DC failure scenarios.

Proposal 4: It is proposed to discuss the following parameters for SCGFailureInformationNR message defined in TS 36.331:

- CGI of the Source PSCell:	Rel-17 definitions on NR-DC SCG failure can be referenced

- CGI of the Failed PSCell:		Rel-17 definitions on NR-DC SCG failure can be referenced

- timeSCGFailure:		Rel-17 definitions on NR-DC SCG failure can be referenced

- connectionFailureType:		The existing field failureType can be re-used

- RA related info:		Rel-17 definitions on NR-DC SCG failure can be referenced

Based on the above analysis, the spec impacts are summarized in the table below:

|  | RAN2-LTE impacts | RAN2-NR impacts | RAN3 impacts |
| --- | --- | --- | --- |
| NE-DC | No | Yes | For RAN3 discussions |
| EN-DC/NG-EN-DC | Yes | No | For RAN3 discussions |

3. Conclusion

In this contribution, for MR-DC SCG failure, we discuss the scenarios and possible enhancements. It is proposed:

Proposal 1: It is proposed to discuss MRO for NE-DC SCG failure scenario.

Proposal 2: It is proposed to discuss the following parameters for SCGFailureInformationEUTRA message defined in TS 38.331:

- CGI of the Source PSCell:	cell information for LTE-5GC

- CGI of the Failed PSCell:		cell information for LTE-5GC

- timeSCGFailure:		Can reference timeSCGFailure-r17

- connectionFailureType:		The existing field failureType can be re-used

- RA related info:		LTE RACH report should be used as much as possible

Proposal 3: It is proposed to discuss MRO for EN-DC/NG-EN-DC failure scenarios.

Proposal 4: It is proposed to discuss the following parameters for SCGFailureInformationNR message defined in TS 36.331:

- CGI of the Source PSCell:	Rel-17 definitions on NR-DC SCG failure can be referenced

- CGI of the Failed PSCell:		Rel-17 definitions on NR-DC SCG failure can be referenced

- timeSCGFailure:		Rel-17 definitions on NR-DC SCG failure can be referenced

- connectionFailureType:		The existing field failureType can be re-used

- RA related info:		Rel-17 definitions on NR-DC SCG failure can be referenced

4. Reference

- RP-221825, Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), CMCC

- R2-2102639/R3-211332, LS on information needed for MRO in SCG Failure Report, Source: RAN3, To: RAN2

- R2-2109208/R3-214695, Reply LS on RACH report for SgNB and information needed for MRO in SCG Failure Report, Source: RAN2, To: RAN3

- R2-2202120/R3-216159, Reply LS on scenarios need to be supported for MRO in SCG Failure Report, Source: RAN3, To: RAN2