3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2208008

Elbonia, 17 – 26 August 2022

Agenda item:	5.1.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	SPS HARQ feedback dropping for TDD

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In the context of Rel-17 URLLC discussions, RAN1 has introduced HARQ feedback deferral for SPS with a maximum allowed time and after that it will be dropped. It has been discussed in the previous two meetings whether HARQ RTT timer or the drx-RetransmissionTimerDL should be started when HARQ feedback is dropped due to maximum allowed time exceed for HARQ feedback deferral.

It was pointed out the same issue exist in legacy without HARQ feedback deferral without too much discussion. In this contribution, we discuss intended legacy behaviour.

# 2	Discussion

In legacy, the timing for HARQ feedback is semi-statically configured in RRC for SPS. In case it collides with a DL slot, the HARQ feedback is dropped. To minimize the negative impact of HARQ feedback dropping, enhancement of HARQ feedback deferral was introduced in Rel-17 URLLC with more opportunities for transmitting the HARQ feedback and it is only dropped when maximum allowed time is reached.

During the Rel-17 URLLC discussions in RAN2 #117e, the following agreement were made [1]:

7 The UE does not start the drx-HARQ-RTT-TimerDL timer for the dropped SPS HARQ feedback

It was made with the argument that current MAC specification says starting of the drx-HARQ-RTT-TimerDL only happens after the actual transmission:

3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

Furthermore, the following agreement made in RAN2 #118e [2]:

9	When the maximum allowed deferral time of HARQ feedback is reached drx-RetransmissionTimerDL is not started. (no specification change)

The consequence of these agreements was there would be no opportunity for the NW to schedule retransmission until next OnDuration. In legacy without HARQ feedback deferral, the possibility of SPS HARQ feedback being dropped is much higher than with HARQ feedback deferral. Besides, there could be other cases that PHY might drop HARQ feedback transmission which are invisible to MAC, e.g due to collision with preamble transmission, power limited in dual connectivity case etc.

We do not think the intention behaviour for starting the HARQ RTT timer is to consider the actual transmission is dropped or not in PHY. It should rather be depending on the timing configured by RRC for the feedback. The agreement made in Rel-17 actual changed this assumption for legacy. It would be good to clarify the behaviour for legacy.

Proposal 1: Clarify that regardless of whether the actual transmission is dropped in PHY, drx-HARQ-RTT-TimerDL is started after the end of the last symbol of DL HARQ feedback.

# 3	Conclusion

HARQ feedback dropping is discussed in this paper with the following proposals proposed:

Proposal 1: Clarify that regardless of whether the actual transmission is dropped in PHY, drx-HARQ-RTT-TimerDL is started after the end of the last symbol of DL HARQ feedback.

TP for MAC is provided below and MAC CRs for Rel-15/16/17 are provided in [3][4][5].

# TP for MAC

When DRX is configured, the MAC entity shall: 1> if a MAC PDU is received in a configured downlink assignment: 2> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the last symbol of the DL HARQ feedback; 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

# Reference

[1] R2-2203734, Summary of [AT117-e][504][IIoT] QoS Additional Open Issues (Nokia), Nokia

[2] R2-2205021, Corrections on HARQ feedback deferral, Nokia, Nokia Shanghai Bell

[3] R2-2208009, Rel-15 CR on Clarification on HARQ RTT timer in case of HARQ feedback dropping, Nokia, Nokia Shanghai Bell

[4] R2-2208010, Rel-16 CR on Clarification on HARQ RTT timer in case of HARQ feedback dropping, Nokia, Nokia Shanghai Bell

[5] R2-2208011, Rel-17 CR on Clarification on HARQ RTT timer in case of HARQ feedback dropping, Nokia, Nokia Shanghai Bell