3GPP TSG-RAN WG2 Meeting #119 electronic											R2-2208052

Online, 17th – 29th August, 2022

Agenda item:       5.2.2

Source: 	Huawei, HiSilicon

Title: 	Summary on Rel-16 control plane corrections

Document for:	Discussion and Decision

# Introduction

This document summarizes the contributions submitted to AI 5.2.2. Rapporteur’s recommendations on how to treat them are provided.

# Correction CRs to be discussed together with Rapporteur’s Misc CR

The following correction CRs could be discussed together. The agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR and its Rel-17 mirror CR except the change in the last one.

Table 1: Correction CRs to be discussed together offline

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2208045 | Miscellaneous CR on 38.331 | Huawei, HiSilicon | - Modify in clause 5.2.2.4.13 and clause 5.5.3 that the UE shall “perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal or sl-TxPoolExceptional…”. - Modify in clause 5.8.8 that a UE shall “configure lower layers to perform the sidelink resource allocation mode 2 based on random selection (as defined in TS 38.321 [3]) using the pool of resources indicated by sl-TxPoolExceptional for the concerned frequency”. - In clause 6.3.5, “resource indicator (RIV)” is replaced by “resource indicator value (RIV)” and “time resource indicator (RIV)” is repalced by “time resource indicator value (TRIV)”. - In clause 6.3.5, “sl-HARQ-ProcID-Offset indicates the offset used in deriving the HARQ process IDs…” should be replaced by ““sl-HARQ-ProcID-Offset indicates the offset used in deriving the HARQ process ID …”. - Add in clause 6.3.5 that the unit of “sl-PSFCH-ToPUCCH” is slot. - Modify in clause 5.8.9.1.2 that the word“SLRB-PC5-ConfigIndex”in “2> set the SLRB-PC5-ConfigIndex included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB” should be replaced by “slrb-PC5-ConfigIndex”. - Modify in clause 5.8.9.1.3 that the word“SLRB-PC5-ConfigIndex”in “for each SLRB-PC5-ConfigIndex value included in the slrb-ConfigToReleaseList…;”should be replaced by “slrb-PC5-ConfigIndex”. - Modify in clause 5.3.5.14 that the word “SL-DestinationIndex” in the descritions “2> for each SL-DestinationIndex included…” and “remove the entry with the matching SL-DestinationIndex…”, should should be replaced by “sl-DestinationIndex”. |  |
| R2-2208050 | Clarification on PC5 AS security | Huawei, HiSilicon |  | As CT1 LS indicated, PC5-S messages can be transmitted unprotected when NULL security algorithm is used. The changes are specific on what is used when the transmission is considered as unprotected with the addition starting with i.e. |
| R2-2207217 | Correction on null security algorithm | ZTE Corporation, Sanechips |  | The first change could mean the same for implementation as CR above however there might be ambiguity (for specific cases or for all cases, the transmission is allowed?), also it is preferred not to modify legacy texts. The intention of second change seems fine, the wording can be optimized. |
| R2-2208283 | Control plane correction on NR Sidelink enhancements | LG Electronics France |  | Rel-17 RRC CR, shall be in AI 6.15.3. The change is straightforward correction on FD texts, can be included in Rel-17 Misc RRC CR if agreed. |
| R2-2208350 | Correction on LCID assignment for SL LCH | ASUSTeK | first change: second change: | For clause 5.8.9.1a.2.2, the level 2 condition for the "to-be-changed level-3 action" is "else", meaning this action is for the case " for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB addition was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or indicated by upper layers:". For this case, UE will execute the action "assigning a new LCID, in accordance with the sl-MAC-LogicalChannelConfig received in the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR ". So rapp considers the second change is not agreeable, then the first change is not needed either, as the action is covered in 5.8.9.1a.2.2. |
| R2-2208600 | Correction on Missing UE behavior on sidelink reset | vivo |  | Rapp agrees that " set the sl-ResetConfig " is needed according to procedure texts in 5.8.9.1.3, however the change in NOTE 1 may not be needed, as there is only one parameter " ENUMERATED {true}". Not like sl-CSI-RS-Config or sl-LatencyBoundCSI-Report, when UE include sl-ResetConfig in RRCReconfigurationSidelink, the "parameter" of sl-ResetConfig is already chosen. |
| R2-2208217 | Clarifications on PC5 UE capabilities for V2X | Nokia, Nokia Shanghai Bell | first change second change third change | This CR is for 38.306. Rapp can agree that the proposed changes, especially the second, third change, can bring more clarity where "simultaneous transmission/reception" could be vague. |

Proposal 1: Discuss the CRs listed in Table 1 together, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR (except the last one).

# Conclusions

The contributions submitted to AI 5.2.2 are summarized, Rapporteur’s recommendations are as follows.

Proposal 1: Discuss the CRs listed in Table 1 together, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR (except the last one).