3GPP TSG-RAN WG2 Meeting #119e 	R2-2208099

# Online, 17 - 29 August, 2022

Agenda item:	8.8.2

Work Item:	NR_UAV-Core

Source: 	Qualcomm Incorporated

Title: 	Mobility considerations and some performance results

Document for:     Discussion/Decision

# Introduction

RAN#94e approved a WI for NR UAV [1].

3GPP studied support of Aerial Vehicles during Rel-15 LTE and documented the extensive performance evaluation results in TR 36.777 [2]. However, performance of Aerial Vehicles using NR has not been studied.

3GPP defined new enhancements for mobility handling, e.g. CHO, targeting terrestrial UEs in Rel-16 and beyond. These new features were not specifically studied or specified taking UAV use-cases in mind.

Performance of UAVs in NR has not been studied.

New enhancements for mobility handling, e.g. CHO, were introduced after the LTE UAV SI and WI were concluded.

In this contribution, we present some results on mobility performance of aerial vehicles using NR, taking into account the RAN2 assumptions used during Rel-15 SI as baseline.

# Handover rates of NR UAV

Mobility simulations are performed to analyze handover rate comparing between ground/terrestrial UEs and airborne UAVs where the connectivity is provided by NR.

## Simulation Setup

For the NR UAV mobility, each modeled UE starts at a randomly selected location within the center cell in the network, and a bearing angle is selected randomly and uniformly. The UE then moves at the assigned speed in a straight line for the duration of the simulation. The same scenario is repeated multiple runs (with different UE dropping), and statistics is averaged.

The following table shows the parameters used to model the handover for these simulations, where the parameters used for LTE study are taken as baseline [2].

Table 1 Simulation Parameters

| Parameter | Description/Note | - Value |
| --- | --- | --- |
| Cell layout and scenario |  | Hexagonal grid, 61 macro sites, 3 sectors per site ISD = 1732m for RMa |
| Band |  | - 700 MHz band for RMa |
| TimeToTrigger | Time to trigger a measurement report | 160 ms |
| A3Offset | Offset between signal strength of serving and neighboring cells | 2 dB |
| MeasurementInterval | Physical layer measurement interval | 10 ms |
| TMeasurement_Period, Intra | L1 filtering time in TS 38.133 | 200 ms |
| L3RRMCoefficient | Filtering coefficient for layer 3 measurements | 1 |
| Qin | Qin Threshold | -6 dB |
| Qout | Qout Threshold | -8 dB |
| TEvaluate_Qout | Qout evaluation period | L1 samples filtered linearly over a sliding window of 200 ms |
| TEvaluate_Qin | Qin evaluation period | L1 samples filtered linearly over a sliding window of 100 ms |
| T310 | Timer to trigger radio link failure | 1 s |
| N310 | Maximum number of consecutive "out-of-sync" indications from lower layers | 1 |
| T311 | Not used (RLF recovery not simulated) | Not used (RLF recovery not simulated) |
| N311 | Maximum number of consecutive "in-sync" indications from lower layers | 1 |
| HOPreparationDelay | Handover preparation delay | 50 ms |
| HOExecutionDelay | Handover execution delay | 40 ms |
| RSRPError | Standard deviation of RSRP measurement error | 1.22 dB |
| UE height | Height in meter above ground level NOTE: Ground UEs are modelled as being at 2 m height. | Ground, 50 m, 100 m, 300 m (AGL) |
| UE speed | Horizontal speed for terrestrial and aerial UEs | 30 km/h, 60 km/h |
| Duration of each simulation |  | 40 seconds |

## Results

Figure 1 shows the handover rate for the RMa scenario. The handover rate is the number of HO attempts per UE over time.

Figure 1. HO rate for different UE height and speed

## Observations

Based on the above simulation results, we observe the following:

- For the same height, HO rate (attempts) increases with increase in speed as expected.

- HO rate changes with change in height.

- For the same speed, in general the HO rate is higher for UAVs when the height is in the mid range, which is in line with the observations of Rel-15 UAV study This is possibly due to the nulls in-between different vertical lobes, where a UE may be served by a side lobe of a far cell instead of the main lobe of a near cell.

Overall, these observations from the above simulation results can be summarized as:

HO rate increases with increase in speed (as expected).

HO rate depends on height of the UAV. For the considered scenarios, HO rate first went up with increase in height then came down.

Based on the above observations and discussion, we propose:

Taking the above results into account, RAN2 consider enhancements for mobility of NR UAVs, e.g. for CHO.

# Conclusion

In this contribution, we presented some simulation results on handover attempt rate of UAVs. Based on the discussion and simulation results, we have the following observations:

Observation 1.	Performance of UAVs in NR has not been studied.

Observation 2.	New enhancements for mobility handling, e.g. CHO, were introduced after the LTE UAV SI and WI were concluded.

Observation 3.	HO rate increases with increase in speed (as expected).

Observation 4.	HO rate depends on height of the UAV. For the considered scenarios, HO rate first went up with increase in height then came down.

Therefore, we propose:

Proposal 1.	Taking the above results into account, RAN2 consider enhancements for mobility of NR UAVs, e.g. for CHO.

# References

[1] RP-213600, New WID on NR support for UAV (Uncrewed Aerial Vehicles), RAN#94e, Dec. 2021

[2] TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles (Release 15), Technical Report