3GPP TSG-RAN2 Meeting # 119-e	R2-2208206

# Electronic Meeting ,  17th Aug  - 29th Aug 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | CRNum | rev | RevNum | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Total RAN Delay calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#118-e meeting, RAN2 received a reply LS from RAN3 regarding M6 calculations in split-bearer scenario. RAN3 reply in R2-2204498 is quoted below- “RAN3 thanks RAN2 for the reply LS in R2-2111290. Based on the answers in the reply LS, RAN3 studies the following use cases for M6 calculation for split bearers in MR-DC: - Case 1: PDCP duplication is activated within the report interval of M6 - Case 2: PDCP duplication is not activated within the report interval of M6 - Case 3: PDCP transmission mode switches between duplication and non-duplication within the report interval of M6 In order to calculate the M6 in TCE correctly, RAN3 agreed to allow the NG-RAN node to report the following additional measurements to TCE: - Number of PDCP PDUs sent via MN or SN within a measurement period, when PDCP duplication is enabled. - Number of PDCP PDUs sent over MN within a measurement period, when the PDCP duplication is not enabled. - Number of PDCP PDUs sent over SN within a measurement period, when the PDCP duplication is not enabled.” In current version (V17.1.0) of 38.314, the additional measurements mentioned above are included. However, set of formulas covering the three mentioned scenarios are not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new sections to introduce formula to calculate total RAN delay per UE covering different scenarios. Impact analysis Impacted functionality: MDT Inter-operability analysis: If not implemented by the RAN nodes, the totall RAN delay may be measured in different ways, which lead to inconsistent total RAN delay measurement results in inter-vendor scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains for calculations for M6 measurements in split-bearer scenario with/without PDCP dupplication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.2.1.x Total RAN Delay in Split-DRB Scenario

### 4.2.1.x.1 Delay Calculation per Split-DRB per UE when PDCP duplication is enabled for entire measurement period

The objective of this measurement is to calculate total RAN delay per split-DRB per UE when PDCP duplication is enabled for the entire measurement period, for QoS verification of MDT.

Protocol layer: PDCP

Table 4.2.1.x.1-1: Definition for total delay for PDCP duplicated packets per Split-DRB per UE

| Definition | Total delay for PDCP duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total delay for a Split-DRB when PDCP duplication is enabled for the entire measurement period. Detailed Definition: M(T,drbid)=min(MMCG(T,drbid) , MSCG(T,drbid)) , where explanations can be found in the table 4.2.1.x.1-2 below. |
| --- | --- |

Table 4.2.1.x.1-2: Parameter description for total delay for PDCP duplicated packets per Split-DRB per UE

| M(T,drbid) | Total Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| MMCG(T,drbid) | Total Delay on the MCG per split DRB per UE during the time period T. |
| MSCG(T,drbid) | Total Delay on the SCG per split DRB per UE during the time period T. |
| min(MMCG(T,drbid) , MSCG(T,drbid)) | Minimum of the total delays between MCG and SCG legs per Split-DRB per UE during the time period T. |

### 4.2.1.x.2 Delay Calculation per Split-DRB per UE when PDCP duplication is not enabled for entire measurement period

The objective of this measurement is to calculate total RAN delay per split-DRB per UE when PDCP duplication is not enabled for the entire measurement period, for QoS verification of MDT.

Protocol layer: PDCP

Table 4.2.1.x.2-1: Definition for total delay for PDCP non-duplicated packets per Split-DRB per UE

| Definition | Total delay for PDCP non-duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total delay for a Split-DRB when PDCP duplication is not enabled for the entire measurement period. Detailed Definition: M(T,drbid)=IMCG(T,drbid)×MMCG(T,drbid)+ISCG(T,drbid)×MSCG(T,drbid)I(T,drbid) IT,drbid=IMCG(T,drbid)+ISCG(T,drbid), where explanations can be found in the table 4.2.1.x.2-2 below. |
| --- | --- |

Table 4.2.1.x.2-2: Parameter description for total delay for PDCP non-duplicated packets per Split-DRB per UE

| M(T,drbid) | Total Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| MMCG(T,drbid) | Total Delay on the MCG per split DRB per UE during the time period T. |
| MSCG(T,drbid) | Total Delay on the SCG per split DRB per UE during the time period T. |
| IT,drbid | Total number of PDCP non-duplicated packets sent per Split-DRB per UE during the time period T. |
| IMCG(T,drbid) | Total number of PDCP non-duplicated packets sent on the MCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |
| ISCG(T,drbid) | Total number of PDCP non-duplicated packets sent on the SCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |

### 4.2.1.x.3 Delay Calculation for per Split-DRB per UE when PDCP duplication is enabled per PDCP packet basis

The objective of this measurement is to calculate total RAN delay per split-DRB per UE when PDCP duplication is enabled and disabled per packet basis during the measurement period, for QoS verification of MDT.

Protocol layer: PDCP

Table 4.2.1.x.3-1: Definition for total delay per Split-DRB per UE when PDCP duplication is enabled/disabled per PDCP packet basis

| Definition | Total delay for PDCP per Split-DRB per UE when PDCP duplication is enabled/disabled per packet basis during a measurement period. This measurement is applicable for EN-DC and SA. This measurement provides the total delay for a Split-DRB when PDCP duplication status is switched between duplicated and non-duplicated during measurement period. Detailed Definition: M(T,drbid)=INon-DupT,drbid×MNon-Dup(T,drbid)+ IDup(T,drbid)×MDup(T,drbid)INon-Dup(T,drbid)+ IDup(T,drbid) , where explanations can be found in the table 4.2.1.x.3-2 below. |
| --- | --- |

Table 4.2.1.x.3-2: Parameter description for total delay per Split-DRB per UE when PDCP duplication is enabled/disabled per PDCP packet basis

| M(T,drbid) | Total Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| MNon-Dup(T,drbid) | Total Delay for the PDCP non-duplicated packets per Split-DRB per UE during the time period T, as defined in Table 4.2.1.x.2-1. |
| MDup(T,drbid) | Total Delay for the PDCP duplicated packets per Split-DRB per UE during the time period T, as defined in Table 4.2.1.x.1-1. |
| INon-Dup(T,drbid) | Total number of PDCP non-duplicated packets sent per Split-DRB per UE during the time period T, as defined in Table 4.2.1.x.2-2. |
| IDup(T,drbid) | Total number of PDCP duplicated packets sent per Split-DRB per UE during the time period T , as defined in Table 4.2.1.8.2-1. |