3GPP TSG-RAN WG2 Meeting #119e	R2-2208229

Online, 17th – 29th Aug, 2022

Agenda Item:	8.9.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on service continuity

Document for:	Discussion and Decision

# Introduction

In R17 NR sidelink Relay, the service continuity scenario was limited to intra-gNB direct-to-indirect and indirect-to-direct path switching due to the lack of time. In R18 NR sidelink Relay enhancement, we will be further discussing and specifying the enhance service continuity cases as described in the objective of R18 work item:

The objective of this work item is to specify solutions that are needed to enhance NR Sidelink Relay for the V2X, public safety and commercial use cases. … - Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]: - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”) - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”) - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”) - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”) Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

Figure 1 shows the enhance service continuity scenarios which is to be discussed during R18.

Fig. 1. Path switching scenarios in R18

In this paper, we will discuss the solutions for the above scenarios. Scenario A, B, C are prioritized, and their solutions can be reused in scenario D, as stated above in the WID objective.

# Discussion

In Rel-17 the intra-gNB path switching procedures have been specified for the cases switching from direct to indirect path and switching from indirect to direct path, and the stage 2 procedures are captured in TS 38.300 in clause 16.12.6. For Rel-18 inter-gNB path switching between direct and indirect path should take the above intra-gNB path switching procedure as baseline, instead of discussing the procedure from scratch.

Proposal 1: The Rel-17 intra-gNB path switching procedures for switching between direct and indirect path are taken as baseline for Rel-18 inter-gNB path switching between direct and indirect paths.

## 2.1 Inter-gNB indirect-to-direct path switching

Figure 2 shows the procedure for the inter-gNB indirect-to-direct path switching case which is extended from Rel-17 procedure for L2 U2N Remote UE switching to direct Uu cell as defined in clause 16.12.6.1 in TS 38.300.

Remote UE

Relay UE

S-gNB

9. sidelink release

T-gNB

3. HO request

4. HO ACK

6. RA to target gNB

8. RRC Reconfiguration

UL and DL data

5. RRC Reconfiguration message

1. Measurement configuration and reporting

2. HO decision

7. RRC Reconfiguration complete message

Fig. 2.  Inter-gNB indirect-to-direct path switching procedure

Step 1. Remote UE triggers the measurements report when the sidelink measurement quantity between Relay UE and Remote UE indicates deteriorated link quality. The triggering Event X1 and Event X2 can be reused in the measurement configuration. The Uu measurements of neighbour cells can be included in the measurements report.

Step 2. Source gNB decides to switch the Remote UE to a target cell in the target gNB based on the reporting results.

Step 3. Source gNB sends the HANDOVER REQUEST message to target gNB over the Xn interface/NG interface, which contains target cell ID, Remote UE’s UE context, and other necessary information. (Here we take Xn-based path switching as an example).

Step 4. Target gNB determines RRC configuration for the Remote UE at the target cell, and sends it via HANDOVER REQUEST ACKNOWLEDGE message to the source gNB.

Step 5. Source gNB sends the RRC configuration to Remote UE.

Step 6. The UE (Remote UE in previous steps) synchronizes with the target gNB and performs Random Access.

Step 7. The UE sends the RRC Reconfiguration complete message to target gNB via the direct path. From this step, the UE uses the RRC connection via the direct path to the gNB.

Step 8. Source gNB sends RRC Reconfiguration message to the Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB.

Step 9. Sidelink between Remote UE and Relay UE is released.

In the above procedures, we can see the HO preparation between S-gNB and T-gNB in step 3 and step 4 are added to enable inter-gNB path switching, other than that the whole procedure of intra-gNB path switching is applicable to inter-gNB path case. The information/configuration contained in the messages in step 3 and step 4 are the same as those used for a normal UE’s handover.

Proposal 2: To support inter-gNB indirect-to-direct path switching on top of Rel-17 intra-gNB path switching from indirect to direct path, the HO preparation between source gNB and target gNB is performed in the same way as legacy inter-gNB HO, without additional RAN2/RAN3 spec impact.

## 2.2 Inter-gNB direct-to-indirect path switching

Figure 3 shows the inter-gNB direct-to-indirect path switching procedure, which is extended from Rel-17 procedure for L2 U2N Remote UE switching to indirect path as defined in clause 16.12.6.2 in TS 38.300.

Remote UE

Target Relay UE

S-gNB

8. sidelink establishment if not exist

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

T-gNB

3. HO decision

2. Measurement configuration and reporting

4. HO request

5. HO ACK

9. RRC Reconfiguration complete message

6. RRC Reconfiguration message

UL and DL data

Figure 3.  Inter-gNB direct-to-indirect path switching procedure

Steps 1 and 2: Remote UE reports the candidate Relay UE(s) to the source gNB, which is discovered by discovery procedure in step 1. The triggering Event Y1 and Event Y2 can be reused in the measurement configuration. The reported results includes at least the Relay UE ID (source Layer-2 ID of Relay UE), Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the Relay UE).

Step 3: Based on measurement results, the source gNB is aware that HO/path switching is needed, and selects a target gNB (whether the source gNB also decides a target relay in the target gNB will be discussed later).

Step 4: To perform HO preparation, source gNB sends the HANDOVER REQUEST message to target gNB via Xn interface/NG interface. (Here we take Xn-based path switching as an example).

Step 5. Target gNB determines RRC configuration for the Remote UE at the target cell, and sends it via HANDOVER REQUEST ACK to the source gNB.

Step 6. Source gNB send the RRC configuration to Remote UE.

Step 7. Target gNB sends the relaying reconfigurations to the Relay UE.

Step 8. Remote UE establishes the sidelink RRC connection with the target Relay UE if it does not exist.

Step 9. Remote UE sends the RRC complete message to gNB via the Relay UE.

In the above procedures, we can see the Uu part can be the same with intra-gNB path switching case, but how to perform the HO preparation in step 3 and step 4 needs some discussion, in particular which node needs to make the decision whether legacy HO to a Uu cell or path switching to a relay UE is to be performed, and for path switching which Relay UE is suitable to be the target Relay UE. There are two options depending on whether the target Relay UE is selected by the source gNB or the target gNB.

- Option 1: Source gNB selects the target Relay UE. In this option, the source gNB decides to switch the Remote UE to a target Relay UE in the target gNB based on the measurement reporting. Then in step 4, the source gNB shall include the target Relay UE ID and the Relay UE’s serving Cell ID in the HANDOVER REQUEST message along with the necessary information as in the legacy procedure.

- Option 2: Target gNB selects the target Relay UE. In this option, the source gNB has to send the measurement results of the candidate Relay UE(s) in step 4. Source gNB can select and include those Relay UE(s) under the same target gNB.

Option1 is more aligned with the legacy HO logic wherein it is the source gNB to decide target gNB and the target cell which is indicated in HO Request message. Option 2 might allow target gNB to choose a Relay UE with better Uu link other than only considering sidelink measurement results reported by the remote UE, but it only applicable in case the Relay UE is in RRC_CONNECTED state. Comparing the two options, we slightly prefer to follow the legacy HO logic in option 1.

Proposal 3: For inter-gNB path switching from direct to indirect path, the target Relay UE is selected by the source gNB.

If option1 is agreed by RAN2, RAN3 needs to be informed, as option1 requires RAN3 to add relay UE ID in the HO Request message together with the target cell ID.

Proposal 4: To inform RAN3 with the above RAN2 agreement that for inter-gNB path switching from direct to indirect path, the source gNB decides the target Relay UE and indicate the target Relay UE ID in HO request message.

Regarding the measurement configuration, it would be beneficial if gNB can also indicate the Allowed-List and Block-List to restrict the Relay UE’s serving cell. Then Remote UE would not report the Relay UE that is camped on the Block-List cells or contained in the Block-List for Relay UE.

Proposal 5: Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell.

## 2.3 Intra-gNB indirect-to-indirect path switching

Combining the procedure of direct to indirect path switching with the one for indirect to direct path switching, figure 4 shows the intra-gNB indirect-to-indirect path switching procedure. Before the path switching, the Remote UE is connected with the same gNB via a Relay UE. After the path switching, the Remote UE established unicast link with a new Relay UE (i.e. target Relay UE), and the unicast between the Remote UE and the old Relay UE is be released (as in step 11).  Meanwhile, the gNB sends RRC message to the old Relay UE to release/reconfigure the relaying configuration associated the Remote UE (as in step 10). These procedures are similar to the indirect-to-direct path switching case in R17.

Remote UE

Relay UE

S-gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

9. sidelink release

8. RRC Reconfiguration

5. RRC Reconfiguration for Remote UE

2. Measurement configuration and reporting, contain the target Relay UE info

UL and DL data

4. RRC Reconfiguration message

6. sidelink establishment if not exist

7. RRC Reconfiguration complete message

Figure 4.  Intra-gNB indirect-to-indirect path switching procedure

Step 1 and Step 2. Remote UE triggers to report the candidate Relay UE(s), when the sidelink measurement quantity between Relay UE and Remote UE shows deteriorated link quality. The reporting results includes the at least the Relay UE ID (source Layer-2 ID of Relay UE), Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the Relay UE).

Step 3. The gNB decides to switch the Remote UE to a target Relay UE in the target cell of the same gNB based on the reporting results.

Step 4. The gNB sends an RRC configuration message to the Remote UE. The RRC message include at least L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s).

Step 5. The gNB sends an RRC configuration message to the target Relay UE, which includes at least the Remote UE's L2 ID and local ID, Uu and PC5 Relay RLC channel configuration for relay traffic and bearer mapping configuration.

Step 6. Remote UE establishes the sidelink RRC connection with the target Relay UE if not exist.

Step 7. Remote UE sends the RRC complete message to gNB via the target Relay UE.

Step 8. The gNB sends an RRC message to the source Relay UE to reconfigure the connection between the source Relay UE and the gNB.

Step 9. Sidelink between Remote UE and the source Relay UE is released.

Proposal 6: Agree the above procedure of intra-gNB indirect-to-indirect path switching.

For indirect-to-indirect path switch, the new measurement events are needed to compare the RSRP of candidate Relay UEs with the serving Relay UE as described below:

- Event1: Candidate Relay UE becomes offset better than the serving Relay UE

- Event2: Serving Relay UE becomes worse than threshold 1 and candidate Relay UE becomes better than threshold 2

Proposal 7: To agree the new measurement events for indirect-to-indirect path switching：

- Event1: Candidate Relay UE becomes offset better than the serving Relay UE

- Event2: Serving Relay UE becomes worse than threshold 1 and candidate Relay UE becomes better than threshold 2

## 2.4 Inter-gNB indirect-to-indirect path switching

Figure 5 shows the inter-gNB indirect-to-indirect path switching procedure, which reuses the procedures for intra-gNB indirect-to-indirect case and the procedures over the Xn interface is the same as the inter-gNB direct-to-indirect path switching case.

Remote UE

Relay UE

S-gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

T-gNB

4. HO request

5. HO ACK

11. sidelink release

10. RRC Reconfiguration

UL and DL data

2. Measurement configuration and reporting, contain the target Relay UE info

6. RRC Reconfiguration message

8. sidelink establishment if not exist

9. RRC Reconfiguration complete message

Fig. 5.  Inter-gNB indirect-to-indirect path switching procedure

Proposal 8: To support Rel-18 inter-gNB indirect-to-indirect path switching on top of Rel-18 intra-gNB path switching from indirect to indirect path, the HO preparation between source gNB and target-gNB is performed in the same way as Rel-18 inter-gNB path switching from direct to indirect path, without additional RAN2/RAN3 spec impact.

## 2.5 Data Forwarding for Inter-gNB cases

### 2.5.1 DL data forwarding

In the DL data forwarding for the inter-gNB cases, same as the handover, the S-gNB forwards the unacknowledged PDCP SDU to the T-gNB for AM DRB as follows.

As long as data forwarding of DL user data packets takes place, the source NG-RAN node shall forward user data in the same forwarding tunnel, i.e. - for any QoS flow accepted for data forwarding by the target NG-RAN node and for which a DRB DL forwarding tunnel was established for a DRB to which this QoS flow was mapped at the source NG-RAN node, any fresh packets of this QoS flow shall be forwarded as PDCP SDUs via the mapped DRB DL forwarding tunnel. - for DRBs for which preservation of SN status applies, the source NG-RAN node may forward in order to the target NG-RAN node via the DRB DL forwarding tunnel all downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have not been acknowledged by the UE. NOTE: The SN of forwarded PDCP SDUs is carried in the "PDCP PDU number" field of the GTP-U extension header. - for any QoS flow accepted for data forwarding by the target NG-RAN node for which a DL PDU session forwarding tunnel was established, the source NG-RAN node forwards SDAP SDUs as received on NG-U from the UPF.

For the cases where path switch from indirect path,  the S-gNB may successfully transmit the data packet to the relay UE and obtain the RLC ACK, while the relay UE has not obtained the PC5 RLC ACK from the remote UE. For example, S-gNB has transmitted packets to the relay UE with SN {1,2,3,4,5,6,7,8} , the next packet associated SN will be 9. S-gNB only get the ACK of packets with SN {1,3,5,6,8} from the relay UE. The relay UE also send the {1,3,5,6,8} to the remote UE, but none of them get ACK from the remote UE. Then the remote UE receives the path switch command and switch to the T-gNB for connection establishment.

According to the legacy data forwarding procedure, where only the unacknowledged data packets are forwarded, the remote UE will suffer from data loss, i.e., packets {1,3,5,6,8} are lost.

Proposal 9: Discuss how to avoid DL data loss during the inter-gNB path switch.

### 2.5.2 UL data forwarding

For UL data delivery during the path switch, some data packets is received by the relay UE but has not been transmitted to the gNB before the path switch. When the remote UE path switch to the direct link, the remote UE may be indicated to do PDCP reestablishment or PDCP data recovery to retransmit the data which has been confirmed by RLC. However, the confirmation from RLC only indicates the data PDU is received by the relay UE and the data may not be received by the gNB.  Therefore, there may also be data loss during UL data delivery.

According to the UL data forwarding procedure during HO, the transmission status of the UL transmission will be transmitted to the T-gNB via SN STATUS TRANSFER. Therefore, the T-gNB will know which data packets is not delivered. However, based on the legacy retransmission procedure of the UE, these data may not be retransmitted to the T-gNB due to these packets are acknowledged by the relay UE.

Proposal 10: Discuss how to avoid UL data loss during the inter-gNB path switch.

# Conclusion

Based on above discussion, we have the following proposal.

Proposal 1: The Rel-17 intra-gNB path switching procedures for switching between direct and indirect path are taken as baseline for Rel-18 inter-gNB path switching between direct and indirect paths.

Inter-gNB indirect-to-direct path switching

Proposal 2: To support inter-gNB indirect-to-direct path switching on top of Rel-17 intra-gNB path switching from indirect to direct path, the HO preparation between source gNB and target gNB is performed in the same way as legacy inter-gNB HO, without additional RAN2/RAN3 spec impact.

Inter-gNB direct-to-indirect path switching

Proposal 3: For inter-gNB path switching from direct to indirect path, the target Relay UE is selected by the source gNB.

Proposal 4: To inform RAN3 with the above RAN2 agreement that for inter-gNB path switching from direct to indirect path, the source gNB decides the target Relay UE and indicate the target Relay UE ID in HO request message.

Proposal 5: Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell.

Intra-gNB indirect-to-indirect path switching

Proposal 6: Agree the above procedure of intra-gNB indirect-to-indirect path switching.

Proposal 7: To agree the new measurement events for indirect-to-indirect path switching：

- Event1: Candidate Relay UE becomes offset better than the serving Relay UE

- Event2: Serving Relay UE becomes worse than threshold 1 and candidate Relay UE becomes better than threshold 2

Inter-gNB indirect-to-indirect path switching

Proposal 8: To support Rel-18 inter-gNB indirect-to-indirect path switching on top of Rel-18 intra-gNB path switching from indirect to indirect path, the HO preparation between source gNB and target gNB is performed in the same way as Rel-18 inter-gNB path switching from direct to indirect path, without additional RAN2/RAN3 spec impact.

Data Forwarding for Inter-gNB cases

Proposal 9: Discuss how to avoid DL data loss during the inter-gNB path switch.

Proposal 10: Discuss how to avoid UL data loss during the inter-gNB path switch.

# Reference

[1] RP-221262, Revised WID on NR sidelink relay enhancements, LG Electronics