3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2208248

Elbonia, 17 – 26 August 2022

Agenda item:	8.14.2

Source:	Nokia, Nokia Shanghai Bell

Title:	QMC enhancements for NR MBS

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In Rel-17, NR QoE Measurement Collection (QMC) has been extended for collecting and reporting of NR QoE measurement, according to study in TR 38.890.

Now with Rel-18, it is planned to enhance the NR QoE framework to support new types of 5G services, such as MBS, including the QoE Measurement Collection (QMC) in RRC Idle/Inactive mode, according to the agreed WID in RP-221803:

- Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2].

- Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting.

This implies the UE would be able to carry out and collect QoE measurements during Idle/Inactive RRC states. The UE needs to store the QoE measurements and to report them to the according collection entity in the network when the UE is returning back to RRC_CONNECTED state.

In this contribution, we want to start the discussion on new challenges which result from the fact that MBS services can be used by the UEs in RRC_IDLE/RRC_INACTIVE state while QMC is activated.

# 2	Discussion

## 2.1	QMC framework shortages for MBS support

The NR QoE Measurement Collection is facilitated by Application Layer Measurement Reporting in the RRC specification (section 5.7.16 of TS 38.331). The corresponding configuration and reporting procedures are defined in RRC_CONNECTED. From Rel-17, the NR QMC configuration, provided by the network when the UE is in RRC_CONNECTED state, survives also transitions to RRC_INACTIVE but not to RRC_IDLE mode.

Observation 1: Rel-17 NR QMC does not continue in RRC_IDLE.

For multicast/broadcast services (MBS), NR supports:

- Broadcast communication service - the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast service area are authorized to receive the data). A broadcast communication service is delivered to the UEs using a broadcast session. A UE can receive a broadcast communication service in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state.

- Multicast communication service - the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast service area are authorized to receive the data). A multicast communication service is delivered to the UEs using a multicast session. A UE can receive a multicast communication service in RRC_CONNECTED state.

With the extension of the NR QoE framework, towards MBS, it will be necessary to support QMC for MBS services, which can be used by the UEs in RRC_IDLE/RRC_INACTIVE state. In particular, the UE may receive broadcast communication service in all RRC states. While multicast communication service may only be received in RRC_CONNECTED in Rel-17, but it is foreseen that the UE support for receiving multicast communication service also in RRC_INACTIVE state as per Rel-18 RP-201038.

Observation 2: The UE may receive broadcast communication service in all RRC states.

Observation 3: For Rel-18, it is deemed to continue QMC for MBS services, for instance for broadcast communication service, which can be used by the UEs in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED state.

The configuration of the UEs for QoE measurement data that has been configured in RRC_CONNECTED mode will concern the service being further received even after the UE transition to RRC_IDLE or RRC_INACTIVE. The known problem is that the QoE configuration is released upon transition to RRC_IDLE, and that collected QoE metrics cannot be reported as long as the UE is in RRC_IDLE or RRC_INACTIVE state.

Observation 4: QoE reports cannot be reported immediately in RRC_IDLE/RRC_INACTIVE state.

The key task resulting from the WI objective for QMC support for MBS services, is to enable QoE measurements in RRC_IDLE, which implies required enhancements to QoE configuration, reporting and data storage. The missing mechanism should enable to survive QoE configuration through RRC_IDLE mode transitions, QoE measurement collection in RRC_IDLE and retrieval of the measurements.

Proposal 1: RAN2 specify QMC support for MBS focusing on the enhancements that are required due to continuous support of the MBS service through all RRC states.

## 2.2	QoE configuration and logging in RRC_IDLE

In principle, the QMC handling could be treated similar to the reporting of logged MDT data. For instance, the QoE configuration message received in RRC_CONNECTED could be extended for a new service type or measurements triggers, and maintained in all RRC states. However, its worth noting that a unified and continued RRC configuration for all RRC states may be not most appropriate, as the MBS communication mode can change or switch after the UE transitions to RRC_IDLE. In such a case the container-based configuration for all RRC states may not sufficient due to missing appropriate guideline to the UE for handling the measurements once being out of a dedicated RRC connection. Given the QMC configuration (originated from CN) for MBS service is RRC-state agnostic, we believe, that RAN2 work on the actual metrics definitions is out of this WID scope, but RAN should be able to control of some configuration parameters due to awareness of a UE’s state transitions.

Proposal 2: RAN2 investigate QoE configuration enhancements that enable control of the MBS specific QoE measurement collection through all RRC states (e.g., validity of the configuration, retrieval related parameters, destination address TCE/MCE, etc).

Such RAN-based control, necessary to enable QoE measurement collection through all RRC states transitions needs corresponding complementary feature on the UE side:

Proposal 3:  To support QMC for NR MBS, the UE  maintains QoE configuration with MBS specific information during its stay in RRC_IDLE  mode.

A comparable mechanism to Logged MDT in RRC_IDLE for the logging of Application Layer measurement in RRC_IDLE is currently not existing, and in consequence no storage space for interim logging during RRC_IDLE is foreseen. Following the QMC support in RRC_INACTIVE, any QMC data received from Application Layer may be directly mapped into the measReportAppLayerContainer on RRC layer for measurement reporting to the network side, after the UE returns back to RRC_CONNECTED.

If the RRC_IDLE or RRC_INACTIVE period is short and only limited amount of data has been transferred from Application Layer to Access Stratum, RRC measurement reporting can still provide all QMC information. But if the UE stays in RRC_IDLE/INACTIVE mode for a long period of time while QMC is activated, the content of measReportAppLayerContainer might be overwritten several times or discarded, which results in loss of needed QMC data.

Observation 5: QoE report is UE internally directly mapped into the reporting container on RRC layer, but those data might get overwritten and lost, in consequence, if RRC measurement reporting is not possible because of missing transition to RRC_CONNECTED state.

Proposal 4: To support QMC for NR MBS, RAN2 investigate enhancements of QMC handling, where the UE performs a temporary logging and a (temporary) reactivation of the RRC connection.

## 2.3	Alignment with the existing radio related measurement

The WI objective for MBS support, specifies additional requirement to enable the alignment of the existing radio related measurement and QoE reporting. Rel-17 already specifies that for the alignment of QoE measurements with radio metrics, MDT is used: The radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The MCE/TCE performs the correlation of the immediate MDT results and the QoE measurement results collected at the same UE (TS38.300).

Hence, if the UE is configured for MBS QMC (including RRC_IDLE), the natural choice by the network would be to use Logged MDT in order to align the radio related data in RRC_IDLE with the ongoing QoE session.

Observation 6: MBS QoE reports alignment with radio measurement reports can be easily achieved by simultaneous configuration of MBS QoE session and MDT session (Immediate and Logged MDT).

Proposal 5: Rel-17 mechanism for alignment of QoE sessions with MDT sessions is the baseline for the radio measurements alignment with QoE data collection for MBS .

In that context, the Rel-17 introduced the Start/Stop indication: The UE configured with QoE measurements sends an indication to inform the gNB about the Start or the Stop of a session of configured QoE measurements. However, the  becomes a main obstacle for data collection in RRC_IDLE, as the network cannot be informed about the QoE session Start and Stop to properly align the two sessions. This may lead to unnecessary logging of radio measurements. In our understanding this is the main issue to address and consider the UE involvement due to lack of the insight from the network side:

Proposal 6: RAN2 to investigate whether the UE can link the start and stop of logged MDT measurement with the start and stop of the QoE Session

# 3	Conclusion

This document has made the following proposals:

Proposal 1: RAN2 specify QMC support for MBS focusing on the enhancements that are required due to continuous support of the MBS service through all RRC states.

Proposal 2: RAN2 investigate QoE configuration enhancements that enable control of the MBS specific QoE measurement collection through all RRC states (e.g., validity of the configuration, retrieval related parameters, destination address TCE/MCE, etc).

Proposal 3:  To support QMC for NR MBS, the UE maintains QoE configuration with MBS specific information during its stay in RRC_IDLE mode.

Proposal 4: To support QMC for NR MBS, RAN2 investigate enhancements of QMC handling, where the UE performs a temporary logging and a (temporary) reactivation of the RRC connection.

Proposal 5: Rel-17 mechanism for alignment of QoE sessions with MDT sessions is the baseline for the radio measurements alignment with QoE data collection for MBS.

Proposal 6: RAN2 to investigate whether the UE can link the start and stop of logged MDT measurement with the start and stop of the QoE Session