3GPP RAN WG2 Meeting #119-e	R2-2208273

eMeeting August 17th – 26th, 2022

Agenda Item:	6.10.2.1

Source:	InterDigital

Title:	Blind Msg3 retransmission in Rel-17 NTN

Document for:	Discussion, Decision

# Introduction

In RAN2#118e, the following was agreed regarding blind Msg3 retransmission after initial transmission, however based on limited time and diverging opinion no conclusion was reached [1]:

Modification 4 to Contention Resolution Timer expiry in R2-2206207 is adopted as baseline and included in the TS 38.321 Rapporteur CR. Continue in the MAC CR discussion, trying to ensure that blind Msg3 retransmission is possible also for the first mgs3 transmission

This document continues discussion on blind Msg3 retransmission grant reception after initial transmission in Rel-17 NTN.

# Discussion

## PDCCH monitoring during a 4-step Random Access procedure

Upon Random Access Preamble transmission, the MAC entity will start the ra-ResponseWindow at a PDCCH occasion specified in [2]. While the ra-ResponseWindow is running, the UE will monitor PDCCH. Upon successful RAR reception containing Random Access Preamble identifiers that match the transmitted PREAMBLE_INDEX, the MAC entity may stop ra-ResponseWindow and hence monitoring for Random Access Response(s).

Upon Msg3 (re)transmission, the MAC entity will start the ra-ContentionResolutionTimer in the first symbol after the end of Msg3 (re)transmission. While the ra-ContentionResolutionTimer is running, the UE will monitor PDCCH. The MAC entity will stop the ra-ContentionResolutionTimer if notification of a reception of a PDCCH transmission is received from lower layers and the PDCCH transmission is addressed to C-RNTI or TEMPORARY_C-RNTI.

This procedure is modified for non-terrestrial networks by offsetting the start of ra-ResponseWindow and ra-ContentionResolutionTimer by the UE-gNB RTT. This ensures that the ra-ResponseWindow and ra-ContentionResolutionTimer do not prematurely expire due to the long propagation delays in non-terrestrial networks.

## Blind Msg3 retransmission in non-terrestrial networks

In terrestrial networks the gap between a MAC entity stopping the ra-ResponseWindow and starting the ra-ContentionResolutionTimer is usually quite brief, allowing near-continuous monitoring of PDCCH. By offsetting the start of the ra-ContentionResolutionTimer in NTN this gap is significantly increased, limiting the ability to quickly receive a blind Msg3 retransmission grant after the ra-ResponseWindow is stopped (Figure 1).

Figure 1: (A) Msg3 blind retransmission grant reception in a terrestrial network; (B) Msg3 blind retransmission grant reception in a non-terrestrial network;

Proposed solutions for fast (i.e. less than UE-gNB RTT) blind Msg3 retranmission grant reception after initial Msg3 transmission can be summarized as follows:

Solution 1) Do not stop the ra-ResponseWindow after initial RAR reception: In this case, the UE will continue to monitor PDCCH for a blind Msg3 retransmission grant after successfully receiving RAR.

Although this will have less impact to UE power consumption as compared to Solution 2, the differential delay due to very large cell sizes in NTN may mean a UE on cell edge (a key beneficiary of coverage enhancements) will only receive the RAR close to ra-ResponseWindow expiry anyways. In these scenarios, the additional benefit of continued monitoring may be limited unless the ra-ResponseWindow is extended (causing even more power consumption).

Solution 2) Starting the ra-ContentionResolutionTimer immediately after Initial Msg3 transmission: In this case, a UE operating in a non-terrestrial network will start the ra-ContentionResolutionTimer in the first symbol after the end of initial Msg3 transmission to monitor for a blind Msg3 retransmission grant.

Unless an additional configuration is introduced to disabled this feature, the UE would have to start the ra-ContentionResolutionTimer after every initial Msg3 transmission in a non-terrestrial network regardless of whether the network intends to send a blind Msg3 retransmission grant or not. Furthermore, the UE may have to start the ra-ContentionResolutionTimer a second time after the UE-gNB RTT due to the same initial Msg3 transmission, which is undesirable from a UE power consumption perspective.

Observation 1:	Previous solutions for fast (i.e. < UE-gNB RTT) blind Msg3 retransmission grant reception in NTN mandate additional monitoring for all UEs regardless of coverage conditions, causing unecessary power consumption.

A second issue is that under certain NTN deployment scenarios (e.g., GEO) the propagation delay is larger than the ra-ContentionResolutionTimer. In this case, if the UE does not receive a blind Msg3 retransmission grant during the first monitoring duration (started immediately after initial Msg3 transmission), the ra-ContentionResolutionTimer may expire before the second monitoring duration (started in the first symbol after initial Msg3 transmission plus the UE-gNB RTT). Under current specification the UE will then consider Contention Resolution unsuccessful.

Observation 2:	If the ra-ContentionResolutionTimer is started immediately after initial Msg3 transmission and a blind Msg3 retransmission grant is not received before timer expiry, the UE will declare premature Contention Resolution failure if the timer duration is less than UE-gNB RTT.

Considering the above observations and that the release is closed, it is proposed that this topic be postponed and revisited in Rel-18 NTN as part of the coverage enhancements work item objective. A companion paper is presented in [4] further discussing this topic.

Proposal 1:	No further optimizations are introduced in Rel-17 NTN to facilitate fast blind Msg3 retransmission grant reception after initial Msg3 transmission. RAN2 to revisit this issue as part of Rel-18 NTN work on coverage enhancements.

# Conclusion

In this contribution the following observations and proposal were made concerning blind Msg3 retransmission grant reception after initial transmission in Rel-17 NTN:

Observation 1:	Previous solutions for fast (i.e. < UE-gNB RTT) blind Msg3 retransmission grant reception in NTN mandate additional monitoring for all UEs regardless of coverage conditions, causing unecessary power consumption.

Observation 2:	If the ra-ContentionResolutionTimer is started immediately after initial Msg3 transmission and a blind Msg3 retransmission grant is not received before timer expiry, the UE will declare premature Contention Resolution failure if the timer duration is less than UE-gNB RTT.

Proposal 1:	No further optimizations are introduced in Rel-17 NTN to facilitate fast blind Msg3 retransmission grant reception after initial Msg3 transmission. RAN2 to revisit this issue as part of Rel-18 NTN work on coverage enhancements.

# References

Draft_R2-118e_Meeting_Report_v2

3GPP TS 38.213: Physical layer procedures for control v17.2.0

3GPP TS 38.321: Medium Access Control (MAC) protocol specification v17.1.0

R2-2208276 - Blind Msg3 retransmission in Rel-18 NTN - InterDigital