3GPP TSG-RAN WG2 Meeting #119-e	R2-2208280

Online, August 17 – August 26, 2022

Agenda Item		:	8.7.4

Source		:	LG Electronics Inc.

Title		:	Discussion on cell reselection enhancement for NTN

Document for		:	Discussion and Decision

1. Introduction

According to WID RP-220953, following objectives are specified.

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4]

  - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

In this document, we address the issues to consider cell reselection enhancement for earth moving cell.

2. Discussion

As the NTN cell is deployed to the satellite, there are two unique characteristics compared to the TN cell. Firstly, the radio quality of NTN cell measured by UE is not significantly different between when the UE is at the cell center and when the UE is at the cell edge. The constancy of radio quality makes harder for UE camping on a NTN cell to decide whether to conduct the measurement on intra-/inter-frequency having an equal or lower priority based on the radio quality of the serving NTN cell. Secondly, the cell coverage may not be fixed on the ground. The UE camping on the NTN cell may lose the radio link even though it is stationary. For example, a cell coverage of quasi-earth fixed cell disappears even though the radio quality measured by the UE is good. If the UE may relax the measurement due to good radio quality, the UE loses the radio link and has to find suitable cell. To solve these problems, the measurement rules for cell reselection were enhanced based on the distance between UE and the serving cell reference location and remaining service time (RST) of serving cell as follows:

- Distance-based measurement rule

  - Though the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, if the distance between UE and the serving cell reference location is longer than distanceThresh, the UE shall perform intra-frequency measurements.

  - Though the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, if the distance between UE and the serving cell reference location is longer than distanceThresh, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

- RST-based measurement rule

  - UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ.

Since such characteristics of NTN cell are the same regardless of the  type of the NTN cell, the measurement rules enhanced in Rel-17 should be available for UEs camping on the earth moving cell also.

Observation 1	The distance-based and remaining service time(RST)-based measurement rule introduced in Rel-17 should be available for UEs camping on an earth moving cell.

Unlike the quasi-earth fixed cell, the reference location of the earth moving cell would constantly change with time, and the UE would be required to update the reference location more frequently than the quasi-earth fixed cell. Such frequent SI re-acquisition may cause the excessive load to the UE as well as the network. To tackle the problem, the network could provide multiple reference locations simultaneously along with the valid time duration. Alternatively, the UE could calculate the reference location over time using an initial reference location and the assistance information provided by the network.

Option 1: NW-based reference location 
Opt-1a:
		- Network provides a single reference location.
		- Network triggers the SI modification procedure to update the reference location.
Opt-1b:
		- Network provides a set of reference locations, epoch time of the set and a valid duration.
		- UE selects the N-th element of the set as a reference location at the time (epoch time + (N-1)*valid duration).
		- UE initiates the SI acquisition procedure upon selecting the last element of the set.

Option 2: UE-based reference location 
		- Network provides epoch time, reference location at the epoch time, ephemeris and periodicity.
		- UE calculates current reference location with the assistance information.
		- UE acquires a new assistance information upon expiry of the validity duration of ephemeris.

Though the Opt-1a requires no spec change, considering the speed of the LEO satellite is 7.5 km/s when the altitude is 700 km, it seems undesirable because it would read to unacceptably frequent SI change notification for all UEs in RRC_IDLE/INACTIVE. Therefore, a new solution, such as option 1b or 2, needs to be specified for UE to acquire the reference location of the earth moving cell, and we propose to discuss how to provide the reference location of the earth moving cell to UE.

Proposal 1	RAN2 needs to discuss how UE can acquire the reference location of the earth moving cell.

During the discussion in Rel-17 NTN, companies had diverse opinions on introducing the location-based cell reselection and finally failed to reach a conclusion. As mentioned above, since the measurement result of the NTN cell could be good enough though the UE is located at the cell edge, the existing R-criteria is insufficient to sort out the candidate for cell reselection. If the distance to the neighbor cell is longer than the threshold, the cell should be excluded from the candidate for cell reselection though the cell satisfies the R-criteria.

Proposal 2	The UE should not consider a neighbor cell as candidate for cell reselection when the distance between the UE and the reference location of the neighbor cell is longer than the distance threshold.

Unlike the quasi-earth fixed cell, for which the coverage lasts in a certain area for a period of time and disappears in an instant, the coverage of the earth moving cell moves gradually along the movement route of the satellite. The service termination timing of the earth moving cell would vary from UE to UE depending on the UE’s location though they are camping on the same cell. Therefore, UE should find the UE-specific RST of the serving cell to apply the RST-based measurement rule to earth moving cell.

Figure 1 Illustration of the earth moving cell coverage.

For example, as depicted in Figure 1, L diminishes as the cell coverage moves. The UE could derive the remaining service time by dividing L by the speed of the reference location having identical direction with the moving direction of the reference location. We assume that the trajectory of the reference location is a straight line and the UE is stationary. To calculate L, UE needs to know cell coverage radius R, angles a and A. The UE may obtain R from the network. The UE could derive angles a and A from the UE location and the ephemeris information. The UE also could calculate the remaining service time of the earth moving neighbor cell as long as the UE can obtain the ephemeris information and cell coverage radius of the cell.

Proposal 3	RAN2 needs to discuss UE-specific RST of earth moving cell to apply the RST-based measurement rule introduced in Rel-17 to earth moving cell.

When the stationary UE camps on the TN cell, the UE hardly performs cell reselection. In contrast, the stationary UE camping on the NTN cell has to perform cell reselection due to the movement of the NTN cell. Furthermore, the service time of the NTN cell varies depending on the satellite's velocity and the beam steering scheme. Such property of the NTN cell may cause frequent cell reselections, i.e., the UE has to reselect again because it reselected the NTN cell that will stop to serving the area after a while. If UE considers the RST to perform cell reselection, it may reduce the number of cell reselection. We suggest considering the RST of the candidate cell as a criterion for cell reselection.

Proposal 4		RAN2 needs to discuss the enhanced cell reselection based on the RST of candidate cell.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1	The distance-based and RST(remaining service time)-based measurement rule introduced in Rel-17 should be available for UEs camping on an earth moving cell.

Proposal 1	RAN2 needs to discuss how UE can acquire the reference location of the earth moving cell.

Proposal 2	The UE should not consider a neighbor cell as candidate for cell reselection when the distance between the UE and the reference location of the neighbor cell is larger than the distance threshold.

Proposal 3	RAN2 needs to discuss UE-specific RST of earth moving cell to apply the RST-based measurement rule introduced in Rel-17 to earth moving cell.

Proposal 4		RAN2 needs to discuss the enhanced cell reselection based on the RST of candidate cell.

4. References

[1] TS 38.304 v17.1.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.

[2] R2-2206201, Report of [AT117-e][111][NTN] Idle mode, ZTE