3GPP TSG-RAN WG2 Meeting #119-e                                                 R2-2208297

Electronic, 17th – 26th August, 2022

Source:              Rakuten Mobile Inc

Title:	              Network Energy savings - UE grouping for efficient signaling

Agenda Item:	8.3.2

Document for:	Discussion and Decision

# Introduction

Based on study item New SI: Study on network energy savings for NR, RAN#94-e and agreements in RAN1#110[1] the paper will discuss RAN2 scope and related proposals.

Following Scope is defined for RAN2#119

Study and identify techniques on the gNB and UE side: Study feasibility from RAN2 perspective on candidate solutions for network energy saving , e.g. potential procedures, signalling design, support/feedback from UE, and potential UE assistance information for network energy saving;

# Discussion

2.1 RAN1 Progress

According to WID and Network energy savings plan RAN1 in RAN1#110 [1] have identified several energy savings techniques in time, frequency, spatial and power domain at the high level. Details of the solutions would be identified in future meetings.

RAN1 have also identified requirements related to signalling design, support/feedback from UE and potential UE assistance information for network energy savings.

Observation 1: RAN1 have identified high level energy saving techniques, however feasibility and energy saving gain analysis are to be completed in future meetings.

Proposal 1: RAN2 should focus on already agreed RAN1 Network energy savings techniques and investigate potential procedures, signalling design, required UE Support/feedback and assistance information to accomplish RAN1 agreed NES techniques.

2.2 UE Grouping or Classification method

A number of RAN1 agreed techniques include grouping of UE’s for efficient signalling or identification in spatial domain.

Agreement: Further study techniques and enhancements for increasing time domain energy saving opportunities by the gNB, including (but not limited to) the following aspects: […] e) dynamic adaptation of UE C-DRX configurations in a UE-group or cell-specific manner Note: For all techniques above, study of time domain techniques is applicable for single component carrier and multi-component carrier cases. Use of UE grouping and its interaction with proposed techniques can be considered. Agreement: - Further study techniques and enhancements for frequency resource usage adaptation by the gNB, including (but not limited to) the following aspects: […] - Enhancements to dynamic bandwidth adaptation including adjustments to RBs and/or BWP used by (Rel-18) UEs for transmission and reception, reducing BWP switch delay, UE-group BWP switching, and joint adaptation of transmission bandwidth and power spectral density - supporting UE group-common BWP or cell-specific BWP or dedicated BWP for network energy savings, and related BWP switching mechanism Agreement Further study techniques and enhancements for the adaptation of number of spatial elements of the gNB, including (but not limited to) the following aspects: […] - feedback/assistance information from the UE required for support dynamic spatial element adaptation - for example, CSI measurement and reports, SR, etc - signaling methods, including reduced signaling, for enabling dynamic spatial element adaptation […] Agreement - For operation with multi-carrier - enhancements on Scell activation and deactivation, enhancements on Scell dormancy and dynamic Pcell switching - including triggering conditions and methods for signaling activation/deactivation - including UE group common dynamic Pcell switching Agreement - signaling methods, including reduced signaling, for enabling dynamic spatial element adaptation - for example, group-common L1 signaling, broadcast signaling, MAC CE, etc. - grouping of UEs to reduce transmission and reception footprint at the gNB; including but not limited to the following - grouping of users in spatial domain

At least Following RAN1 agreed Network energy saving techniques require UE grouping or classification for the purpose of signalling reduction and optimisation of energy savings techniques.

Frequency Domain Techniques

- UE grouping for Dynamic Pcell Switching and S-Cell activation/deactivation.

- UE-Grouping for BWP Switching and joint adaptation of transmission bandwidth and power spectral density

Time Domain Techniques

- UE C-DRX configurations in a UE-Group or cell-specific manner

Power Domain Techniques

- UE group based Joint Transmit power indication (in UE groups)

Spatial Domain Techniques

- UE Group-Common L1 Signalling – Broadcast Signalling for enabling spatial element adaptation.

- Dynamic TRP dormancy implemented through UE-Group Specific Switching.

- Joint Adaptation of spatial-domain , frequency-domain and/or power domain configurations.

To achieve UE grouping for efficient signalling RAN2 can take two different directions.

Option 1: Define dedicated framework for each Network energy savings feature for group based dynamic signalling of Network Energy saving technique On and Off.

Like (DCP) DCI with CRC scrambled by PS-RNTI to group UE’s for UE Power saving or G-RNTI for multi-casting.

This will result in defining several new RNTI or Common MAC CE definition.

Option 2: RAN2 can defined a framework where UE’s can be grouped for the purpose of Energy savings and future ES techniques can be easily accommodated. In such framework single NES RNTI can be defined with multiple DCI Formats for each ES technique.

Observation 2: To perform efficient signalling for a particular group of UEs, efficient way of grouping and configuration update for each group is required.

Proposal #2: Define a common framework where UE’s can be easily grouped based on required Network energy savings by defining single NES-RNTI that can be scrambled with group RNTI.

New DCI format would define for different Energy savings techniques.

Table 2.2.1

|  | Network Energy Savings Technique | RNTI | Grouping Method | DCI Format | Possible Information Elements |
| --- | --- | --- | --- | --- | --- |
| 1 | UE grouping for Dynamic Pcell Switching and S-Cell activation/deactivation. | NES-RNTI | CRC Scrambling Mask For specific Group … | DCI2_7 | S-Cell Status indicated in Bitmap Or Pcell Switching indication with Cell ID or Candidate Cell ID. |
| 2 | UE C-DRX configurations in a UE-Group or cell-specific manner |  |  | … | Group Common C-DRX On indication. (Optional) C-DRX configuration |
| 3 | UE-Grouping for BWP Switching and joint adaptation of transmission bandwidth and power spectral density |  |  | … | BWP Switching ; BWP Candidate (Bitmask for selection) PDSCH Tx Power /EPRE |
| 4 | UE Group-Common L1 Signalling – Broadcast Signalling for enabling spatial element adaptation. |  |  | … | SS-PBCH-BlockPower (Bitmap) powerControlOffsetSS (Bitmap) |
| 5 | Joint Adaptation of spatial-domain , frequency-domain and/or power domain configurations. |  |  | … | Cell Sleep Pattern or Cell sleep state indication in Bitmask Along with relative spatial-domain information such as Beam ID and/or frequency domain like BWP or adaptive Frequency Bandwidth. |
| 6 | UE group based Joint Transmit power indication (in UE groups) |  |  | … | PDSCH Tx Power /EPRE |
| 7 | Dynamic TRP dormancy implemented through UE-Group Specific Switching. |  |  | DCI4_2 | Selection of s-TRP or m-TRP |
| … | … | … | … | … | … |

Table 2.2.1 shows one possible implementation of energy saving techniques using general NES-RNTI framework.

Observation 3: Few energy saving methods may require a lot of information for proper execution, in such case it might not be appropriate to carry all of it over PDCCH. Hence, information may be carried by RRC signalling and activation can be done through group-based signalling through MAC CE or PDCCH.

Proposal # 3: Another approach for efficient UE Group signalling can be a multi-stage process in which all possible configuration are provided to UE through RRC signalling in advance and activated through dedicated or group common MAC CE command.

2.3 Long Sleep for Cells with Legacy UE’s

RAN1 agreements indicate sleep states are to be defined.

Agreement - Mechanism to utilize potential energy saving states or sleep modes and the transition between states from leveraging cell on/off opportunities - including studies of waking up gNB due to user traffic, or user density, or gNB receiving wake up signal - including technique to allow discovery and measurement of cells in sleep or dormant states

Each sleep state may be associated with certain transmission and/or receive characteristics and duration.

A sleep mode may also refer to a discontinuous transmission (DTX). Multiple DTX cycles may be defined (e.g., long DTX cycle with 100 ms and short DTX cycle with 20 ms. duration.

Observation 4: Only the UE’s supporting Rel-18 Energy saving features would be able to access the cell in dormant mode (Without SSB) for over 20ms for standalone and 120ms for ENDC/MRDC configuration. RAN2 need to discuss possible solution for this problem.

Proposal 4: One of the possible solution to handle Legacy UE’s during long sleep period, is spatial Sleep where particular beams are switched to dormant (No SSB) only when no legacy UE is present in the coverage of that spatial steam.

SSB’s of the neighbouring spatial streams and neighbouring cells can be dynamically turned ON based on the mobility status and RF condition of the legacy UE.

Observation 5: In case of CA ,MR-DC or multi-cell coverage in a particular area, legacy UE’s can be connected to a particular cell and other cells will be able to go into dormant/sleep state for longer period of time.

If legacy UE is in idle mode and all cells are in dormant state then UE will be impacted for this reason RAN2 need to investigate how to handle such situation and co-ordination between cells.

Proposal 5: RAN2 to study efficient signalling to enable network-controlled idle mobility as well as differentiation between legacy and Rel-18 Network energy savings supported UEs.

# Conclusion

This contribution discussed techniques for grouping of UE’s for Network energy saving techniques as well as propose RAN2 to further investigate on handling of legacy UE’s during Cell Sleep/dormant state.

Observation 1: RAN1 have identified high level energy saving techniques, however feasibility and energy saving gain analysis are to be completed in future meetings.

Observation 2: To perform efficient signalling for a particular group of UEs, efficient way of grouping and configuration update for each group is required.

Observation 3: Few energy saving methods may require a lot of information for proper execution, in such case it might not be appropriate to carry all of it over PDCCH. Hence, information may be carried by RRC signalling and activation can be done through group-based signalling through MAC CE or PDCCH.

Observation 4: Only the UE’s supporting Rel-18 Energy saving features would be able to access the cell in dormant mode (Without SSB) for over 20ms for standalone and 120ms for ENDC/MRDC configuration. RAN2 need to discuss possible solution for this problem.

Observation 5: In case of CA ,MR-DC or multi-cell coverage in a particular area, legacy UE’s can be connected to a particular cell and other cells will be able to go into dormant/sleep state for longer period of time.

If legacy UE is in idle mode and all cells are in dormant state then UE will be impacted for this reason RAN2 need to investigate how to handle such situation and co-ordination between cells.

Proposal 1: RAN2 should focus on already agreed RAN1 Network energy savings techniques and investigate potential procedures, signalling design, required UE Support/feedback and assistance information to accomplish RAN1 agreed NES techniques.

Proposal #2: Define a common framework where UE’s can be easily grouped based on required Network energy savings by defining single NES-RNTI that can be scrambled with group RNTI.

New DCI format would define for different Energy savings techniques.

Proposal # 3: Another approach for efficient UE Group signalling can be a multi-stage process in which all possible configurations are provided to UE through RRC signalling in advance and activated through dedicated or group common MAC CE command.

Proposal 4:one of the possible solutions to handle Legacy UE’s during long sleep period, is spatial Sleep where particular beams are switched to dormant (No SSB) only when no legacy UE is present in the coverage of that spatial steam.

SSB’s of the neighbouring spatial streams and neighbouring cells can be dynamically turned ON based on the mobility status and RF condition of the legacy UE.

Proposal 5: RAN2 to study efficient signalling to enable network-controlled idle mobility as well as differentiation between legacy and Rel-18 Network energy savings supported UEs.

# References

R1-2205554, 3GPP TSG RAN WG1 Meeting #109-e, Electronic Meeting, May 09-20, 2022; Summary #3 for email discussion on energy saving techniques of NW energy saving SI; Moderator (Intel Corporation)

- RP-213544: New SI: Study on network energy savings for NR; TSG RAN Meeting #94, Electronic Meeting, Dec 6 - 17, 2021