3GPP TSG-RAN WG2 #119-e	Tdoc R2-2208310

Electronic meeting, August 17th - 26th 2022

Agenda Item:	6.12.2.2

Source:	Ericsson

Title:	Paging configuration for RedCap UEs in the initial DL BWP

Document for:	Discussion, Decision

# 1	Introduction

In Rel-17 RedCap WI [1] introduced support for UE complexity reduction techniques and UE power saving techniques suitable for IoT use cases such as industrial wireless sensors, video surveillance, and wearables. After a study phase [2], support for a reduced capability (RedCap) UE type and two UE power saving techniques: Extended DRX in RRC idle/inactive state, and RRM measurement relaxation for neighbor cells were specified.

The new reduced capability (RedCap) UE type enables reduced UE complexity through various UE complexity reduction techniques. A RedCap UE supports a maximum UE Rx/Tx bandwidth of 20 MHz in FR1 and 100 MHz in FR2 (whereas a normal NR UE supports at least 100 MHz in FR1 and 200 MHz in FR2). A RedCap UE cannot support larger Rx/Tx UE bandwidths than 20 MHz in FR1 and 100 MHz in FR2, and it cannot support features related to carrier aggregation (CA), dual connectivity (DC), more than 2 UE Rx/Tx antenna branches, or more than 2 DL/UL MIMO layers.

In this contribution we discuss the need to clarify the description for the field parameter pagingSearchSpace.

# 2	Discussion

When regular initial DL/UL BWP in a cell is configured with a bandwidth that is wider than the maximum RedCap UE bandwidth (i.e., wider than 20 MHz in FR1 or wider than 100 MHz in FR2), a separate RedCap-specific initial DL/UL BWP is configured for random access. Such separate initial DL BWP can, but does not need to, contain SSB/CORESET#0/SIB.

When separate initial DL BWP contains CD-SSB (and CORESET#0), a RedCap UE in idle/inactive mode monitors paging on this BWP. However, when separate initial DL BWP does not contain CD-SSB (and CORESET#0), it should be possible for a RedCap UE in idle/inactive mode to camp on a BWP associated with CD-SSB/CORESET#0 in order to monitor for paging. Note that the RedCap UE uses the separate initial DL BWP that does not contain CD-SSB (and CORESET#0) only for random access in this case.

In TS 38.331, paging is configured in: DownlinkConfigCommonSIB-> initialDownlinkBWP-RedCap-r17-> BWP-DownlinkCommon-> pdcch-ConfigCommon -> pagingSearchSpace. The field description for this parameter is as follows:

pagingSearchSpace ID of the Search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0..

In the field description above, it is stated that “if the field is absent, the UE does not receive paging in this BWP”. It is also captured that the field is absent for RedCap specific initial DL BWPs, if it does not include CD-SSB and the entire CORESET#0. This means that if the separate initial DL BWP, which is configured for random access for RedCap UEs in idle/inactive mode, is not associated with CD-SSB, it is not clear which BWP the RedCap UE should monitor for paging.

One can assume that the following text “This field is absent for the RedCap specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0” implies that a RedCap UE camps on the regular “initial DL BWP” if the RedCap specific initial DL BWP does not include CD-SSB and the entire CORESET#0. However this may imply that regular “initial DL BWP” cannot be wider than the maximum RedCap UE BW. This is not correct and hence it would be beneficial to capture explicitly that if pagingSearchSpace is absent for the RedCap UE specific initial DL BWP, RedCap UE shall monitor paging (at least in idle/inactive mode) in the initial DL BWP defined by MIB-configured CORESET#0.

If the regular “initial DL BWP” is wider than the maximum RedCap UE BW and the separate initial DL BWP, which is configured for random access for RedCap UEs in idle/inactive mode, is not associated with CD-SSB, it is not clear which BWP a RedCap UE should monitor for paging.

Hence, we propose the following:

In the field description for pagingSearchSpace, capture explicitly that if pagingSearchSpace is absent for RedCap UE specific initial DL BWP, a RedCap UE shall monitor paging in the initial DL BWP defined by MIB-configured CORESET#0.

# 3	Conclusion

In this contribution we discuss the need to clarify the description for the field parameter pagingSearchSpace.. The following observation was made:

Observation 1	If the regular “initial DL BWP” is wider than the maximum RedCap UE BW and the separate initial DL BWP, which is configured for random access for RedCap UEs in idle/inactive mode, is not associated with CD-SSB, it is not clear which BWP a RedCap UE should monitor for paging.

Based on the discussion in the previous section we have proposed the following:

Proposal 1	In the field description for pagingSearchSpace, capture explicitly that if pagingSearchSpace is absent for RedCap UE specific initial DL BWP, a RedCap UE shall monitor paging in the initial DL BWP defined by MIB-configured CORESET#0.

# References

RP-220966, “Revised WID on support of reduced capability NR devices”

TR 38.875 V17.0.0, “Study on support of reduced capability NR devices”