3GPP TSG RAN WG2 Meeting #119-e	R2-2208318

Electronic, 17th August – 26th August, 2022

Agenda Item:		8.2.3 (RAT-dependent Integrity)

Source:				Samsung

Title:					Discussion on integrity of RAT dependent positioning techniques

Document for:	Discussion

Introduction

  - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

    - Identify the error sources, [RAN1, RAN2].

    - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

    - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In the approved new SI for expanded and improved NR positioning [1], one important direction is to study solutions for Integrity for RAT dependent positioning techniques.

This contribution discusses aspects related to integrity of RAT dependent positioning techniques.

General modeling method and criteria for RAT-dependent integrity

Different with RAN-independent positioning integrity, RAT-dependent integrity has the error source and fault identified by 3GPP RAN WG, especially RAN1. The basic calculation mechanism for RAT-dependent integrity seems to have the same principle and system as RAT-independent mechanism such as:

Each selected error source should have a statistical error distribution. For integrity operation, the network will ensure that the probability of positioning error larger than bound for longer than TTA should less than or equal to the integrity risk probability (Residual Risk + IRallocation provided by LMF) under the condition that the all assistance data are within its validity period (NOT DNU). For example, it should be ensured that the probability of positioning error larger than 10 m for longer than 10 ms should less than or equal to 5% according to some valid positioning assistance data. According to the analyzation above, we can find that the bound is related to the distribution of the residual errors.

To obtain the bound, when the residual errors are independent and identically distributed, the paired over-bounding Gaussian formula in GNSS can be used to model the error sources for RAT dependent positioning techniques. Considering the law of large numbers and the central limit theorem, the paired over-bounding Gaussian formula is applicable to most error sources. However, the calculation method of the mean and standard deviation of each error source may need to be agreed in RAN1.

Observation 1. The paired over-bounding Gaussian formula can be used to model most of the error sources for RAT dependent positioning techniques.

Proposal 1: RAN2 agree that the general modeling of RAT-dependent integrity calculation is same as RAT-independent GNSS case.

However, still what is the errors and faults to be considered in this positioning integrity is not known, and RAN1 is working on this. Therefore, RAN2 needs to wait for their input on the error source identified, and how to calculate the mean and standard deviation of each error source.

Proposal 2: RAN2 wait for the input on the error source to be used in RAT-dependent integrity calculation and how to calculate the mean and the standard deviation of each error source from RAN1.

Supporting positioning integrity mode

In Rel-17, UE-based mode was discussed and specified, and the LMF-based mode was also discussed a bit, but it was not specified. The main reason was the time permitted was not enough, but at the same time the concern that it is hard to standardize over implementation details in LMF-based mode. This is again applied to RAT-dependent positioning integrity. And it is expected that the allowed time is not enough to facilitate all the possible variations. Starting from the already known one, i.e., UE-based positioning integrity mode would have the merit to secure the time for the remaining issues. Therefore, we are opt to prioritize the UE-based mode for the RAT-dependent positioning integrity mode.

Proposal 3. Regarding positioning integrity mode, RAN2 prioritize to specify the UE-based mode as in positioning integrity for GNSS in Rel-17.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1. The paired over-bounding Gaussian formula can be used to model most of the error sources for RAT dependent positioning techniques.

Proposal 1: RAN2 agree that the general modeling of RAT-dependent integrity calculation is same as RAT-independent GNSS case.

Proposal 2: RAN2 wait for the input on the error source to be used in RAT-dependent integrity calculation and how to calculate the mean and the standard deviation of each error source from RAN1.

Proposal 3. Regarding positioning integrity mode, RAN2 prioritize to specify the UE-based mode as in positioning integrity for GNSS in Rel-17.

References

- RP-213588, New SID for Expanded and improved NR positioning, Dec, 2021.